TOR 1: Update of the SEEMP guidelines (Resolution MEPC.282(70))

Comment sheet for Round 4

Country / Organization:_____________________________

Introduction

The updated document, using the base document containing the draft amendments to Resolution MEPC 282(70) on 2016 guidelines for the development of a ship energy efficiency management plan (SEEMP) and the work done by the Correspondence Group until MEPC 76 and having taken into account the comments of the CG members in the first round, is provided as “(TOR 1) SEEMP guidelines_MEPC.282(70)_R4.docx”. All the changes have been marked in red to facilitate the review.

The CG members are invited to consider the following issues:

.1	track changes in the updated document;

.2	for a ship that transfers to another company or flag, how this issue should be addressed in the SEEMP guidelines;

.3	in developing a plan of corrective actions, whether the plan should aim at achieving the required annual operational CII within a defined timeframe such as the following calendar year;

.4	appendices in the SEEMP guidelines, whether additional standardized sample formats are needed such as for the plan of corrective actions or for transferring data to a new company in case of transfer; and

.5	any other issues raised by the CG members with a concrete proposal.

Q1.	Track-changes in the updated document

The updated base document contains track-changes marked in red suggested by the Coordinators based on the feedback from round 1.

The CG members are invited to indicate their support to the suggested changes or propose alternative text together with a rationale and a clear reference to the paragraph concerned.

| Paragraph | Comment | Coordinators’ remark |
| --- | --- | --- |
|  | No comments were provided by Chile, CSC, Cyprus, France, Liberia, ICS, Intertanko, Pacific Environment Text is supported by Canada, EC, Greece, UK <Sweden> Sweden agrees with views expressed previously by many participants that the SEEMP should be kept as simple and short as possible. Therefore we suggest not to add provisions of general or policy nature that do not add any applied value to the SEEMP which is a practical tool intended to be used by individual ships. While we are revising the entire guidelines and if acceptable to the group, we suggest, therefore to remove this kind of paragraphs from SEEMP. We have identified two (1.2.4 and 3.1) but there might be more | Noted. |
| 1.2 | <Finland> Either "The aims of SEEMP are:" or "The aim of SEEMP is three-fold" | Noted. |
| 1.2.1 | <USA> US Comment: We don’t think this properly reflects one of the aims of the SEEMP. The proposed language reflects the aim of the guidelines, but not of the SEEMP. We suggest retaining the original language: “To encourage ship owners and operators to incorporate actions to improve the energy efficiency and carbon intensity of their ships and ship management practices.” ALTERNATIVELY: This section may no longer be needed and can be deleted. The SEEMP is required by MARPOL and the aims of the SEEMP cannot be changed through voluntary guidelines. | Noted. |
| 1.2.3 | <Italy> In line with other revised paragraphs (e.g. 1.3.3) , Italy suggests to insert at the end of the paragraph and after the term “Administration” the indication “or any organization duly authorized by it” as follows: 1.2.3 To specify the methodology the ship will is to use to calculate the attained annual operational carbon intensity indicator (CII) as required by Regulation 28.1 of MARPOL Annex VI and the processes that should be used to report the data to the ship’s Administration or any organization duly authorized by it. <USA> Delete “should” add “are to” | Duly reflected. |
| 1.2.4 | <Italy> Italy is of the view that this paragraphs could be included, as a general goal, in the paragraph 1.2 in way to read: 1.2 The aims of the SEEMP should assist the international shipping sector to achieve the goal of Chapter 4 of MARPOL Annex VI set out in Regulation 20 which is reducing the carbon intensity of international shipping, working towards achieving the levels of ambition in the Initial IMO Strategy on reduction of GHG emissions from ships, and are three-fold: ………. <Sweden> In light of the above comment Sweden suggest not to include this paragraph. In addition the IMO initial strategy will be revised, requiring consequential update of the guidelines, which we find unnecessary. | Duly reflected. Noted. |
| 1.3 | <Finland> Delete the second "is". Instead of "… in sections 3,4 and 5 of these guidelines is to provide…", "… in sections 3,4 and 5 of these guidelines to provide…" | Duly reflected. |
| 1.3.1 to 1.3.3 (comment on language) | <Sweden> First sentences in all three paragraphs need to be corrected and aligned. Instead of using “is addressed … is to provide” in first two paragraphs and “and addresses is to provide” in the third paragraph, we suggest the following: “Part X of the SEEMP, required by Regulation (in 1.3.4 will be “Regulations”) XX of MARPOL Annex VI is addressed in sections XXX is to provide …. (alternative: is addressed in sections XX and provides…) | Duly reflected. |
| 1.3.3. | <China> It is suggested to add “Part III applies to ships of 5,000 GT and above.” at the end of this paragraph to align with paragraph 1.3.1 and 1.3.2. <Sweden> Comment on language 1.3.3 Part III of the SEEMP, required by Regulations 26.3 and 28.8 of MARPOL Annex VI and,is addressed in sections 9, 10, 11, 12, 13 and 14 of these guidelines, is to provide (or “and provides”): the methodology that is to be used to calculate the ship’s attained annual operation CII required by Regulation 28 of MARPOL Annex VI; the processes that are to be used to report this value to the ship’s Administration or any organization duly authorized by it, (something is missing here, perhaps “the methodology to be used to calculate”?) the required annual operational CII for the next three years; a guidance (?) on drafting/creating an implementation plan documenting how the required annual operational CII will be achieved during the next three years; a procedure for self-evaluation and improvement for the next three years; and, for ships rated as D for three consecutive years or rated as E, a guidance (?) on drafting/creating a plan of corrective actions to achieve the required annual operational CII if necessary. <USA> For ships rated D for 3 consecutive years or rated E, a plan of corrective action is required, therefore the phrase “if necessary” must be deleted. | Duly reflected. Duly reflected. |
| 1.3.4 | <Italy> Italy would like to highlight only a typo error on the identification of the paragraph that now is indicated only with “.4”. See the text below: 1.3.4 A sSample forms of the various sections of the SEEMP is are presented in appendices 1, and 2, and 2bis for illustrative purposes. A standardized data-reporting format for the data collection system is presented in appendix 3. | Duly reflected. |
| 1.4 | <China> The explanation for Appendix 4 should also be included in this paragraph. It is suggested to add “A standardized data reporting format for the trial carbon intensity indicators on voluntary basis is presented in appendix 4.” in this paragraph. | Duly reflected. |
| 2.4 | <Italy> Italy has no strong views on the definition “Carbon Intensity Indicator”. <USA> This term is not defined in the regulation and does not need to be defined in the guidelines. don’t believe a definition is needed in the guidelines. It is understood that the term “carbon intensity indicator” mentioned throughout this guideline refers to the Regulation 27 of MARPOL Annex VI required annual operational carbon intensity indicator. The US does not agree with the proposed definition. Suggested to be deleted. | Noted. Noted. |
| PART I | <China> It appears that the revised title of Part I is duplicated with the title of these guidelines, so it is suggested to keep the title of PART I as it was. | Noted. |
| 3.1 | <Sweden> While we are revising the entire guidelines, we suggest to remove this paragraph (in line with our general comment above) <USA> Unclear as to where this change comes from. Part 1 of the SEEMP applies only to ships and emission reductions are to be delivered by ships and not “companies.” This change should be rejected and the text should refer to “ship operators.” ALTERNATIVELY: Given that this paragraph is simply re-stating Regulation 26.1 requirement, it can be deleted. | Noted. Duly reflected. |
| 3.2 (language) | <Sweden> Regulation 26.1 of MARPOL Annex VI requires each ship of 400 gross tonnage and above, subject to Chapter 4 (400 gross tonnage and above) to keep on board a ship specific Ship Energy Efficiency Management Plan (SEEMP). | Duly reflected. |
| 3.5 | <Italy> Italy prefers to keep the previous formulation of the text that it is much in line with the Annex VI/Regulation 22. <Norway> We do not agree with including “ , but is not required to”. SMS is only required for ships above 500 GT and for these ships the SEEMP is implicitly part of the SMS. The word “may” was in our understanding originally included to cover ships which had a SEEMP but did not require to have an SMS. <Sweden> Language In our understanding the word “may” means “optional”, there is no need to add “but it is not required to” to the sentence (unless the majority wants to keep it). <USA> This deletion of the word “ship’s” should be rejected as it changes the meaning of the paragraph. | Noted. Noted. Noted. Duly reflected. |
| 4.1.2. | For sake of clarity, Italy suggests to delete the text highlighted in yellow as follows: 4.1.2 Recognizing that there are a variety of options to improve efficiency and reduce carbon intensity (e.g. – speed optimization, weather routing and, hull maintenance, andretrofitting of energy efficiency devices, and use of alternative fuels), for example – and that the best package of measures for a ship to improve efficiency and reduce carbon intensity depends differs to a great extent depending upon ship type, cargoes, routes and other factors, the specific measures for the ship to improve energy efficiency and reduce carbon intensity that should be identified in the first place. These measures should be listed as a package of measures to be implemented, thus providing the overview of the actions to be taken for that ship. | No text marked in yellow was found on the document. For such reason no observation or modification was done. |
| 4.1.5 | <Sweden> The initial sentence is correct, and shall be kept as is, the addition of “the coordination” is ok. <USA> Unclear as to where this change comes from, it seems that including “ship owners, operators” would be preferred. “Rather, it may depend on many stakeholders including ship repair yards, companies, ship-owners, operators,)(…) 5.2.4 – requires good early communication among operators companies, ports | Noted. Duly reflected. |
| 4.1.7.2. | <Sweden> The last sentence of the paragraph says that the goal should be easy to understand, the same applies to the guidelines, they should also be easy to understand. We suggest the following redrafting of this paragraph: For ships or companies not subject to Regulation 28, there are no requirements to define a goal and to communicate it to the public or to be a subject to external inspection, surveys, or audits with respect to the SEEMP. Nevertheless, a meaningful goal should be defined to serve as a signal on a company’s commitment to improve the energy efficiency and carbon intensity of the ship. There are many ways to illustrate that, including the annual fuel consumption, Annual Efficiency Ratio (AER), cgDIST, Energy Efficiency Operational Indicator (EEOI) or other Carbon Intensity Indicators (CIIs). In all cases, the goal should be measurable and easy to understand. <USA> (…)There are many forms ways the goal can take(…) | Duly reflected. Duly reflected. |
| 4.2.2 | <Finland> Instead of "at least one responsible personnel", "at least one responsible person" <Greece> For ships subject to Regulation 28, it is recommended to designate at least one responsible personnel for oversight of the ship’s measures to achieve the carbon intensity target, to review regularly (at least yearly) the efficacy of the implemented measures and to decide on corrective actions, as applicable (see paragraph 14.3). Paragraph 14.3 has been deleted. Please remove. <Italy> For more clarity, Italy suggests the following modification in the text (see addition in blue and strikeout text highlighted in yellow): 4.2.2 For ships subject to Regulation 28, it is recommended to the company should designate at least one responsible personnel for oversight of the ship’s measures to achieve the carbon intensity target, to review regularly (at least yearly) the efficacy of the implemented measures and to decide on corrective actions, as applicable (see paragraph 14.3). It may also be helpful to define procedures for that person to follow if there are signs that the ship may not be on track to achieve its carbon intensity target. <Norway> We suggest to not include elements relevant for Regulation 28 into SEEMP Part I and propose to delete this paragraph. This is also covered in 4.3.4 <Sweden> In the first sentence word “person” shall be used: “… at least one responsible personnel for oversight of… | Duly reflected. Duly reflected. No text marked in blue or yellow was found on the document. For such reason no observation or modification was done. Duly reflected. |
| 4.3.1. | <China> There are three 4.3.1s in these guidelines. We suggest to keep the last one. <Italy> Italy supports the third option of the paragraph 4.3.1 as follows: 4.3.1 The energy efficiency of a ship should be monitored quantitatively. This should be done by an established method, preferably by an international standard. In many cases, the monitoring tool should target the goal indicator set out in paragraph 4.1.7 (e.g., AER, cgDIST, EEOI, or other CIIs as agreed by the Organization). The CII the EEOI, or another performance indicator developed by the Organization are some of the internationally established tools to obtain a quantitative indicator of energy efficiency of a ship and/or fleet in operation, and can be used for this purpose. Therefore, CII EEOI could be considered as the primary monitoring tool, although other quantitative approved measures also may be appropriate. <Sweden> Is our understanding correct that the 3rd 3.4.1 will be kept in the text. We can support combining two paragraphs in one. <USA> The United States would prefer Option 3 with the following edits: 4.3.1: The energy efficiency of a ship should be monitored quantitatively. This should be done by an established method, preferably by an international standard. In many cases, the monitoring tool should be the goal indicator set out in paragraph 4.1.7 (e.g., AER, EEOI, CII). If a quantitative goal is not defined for a ship, a quantitative performance indicator developed by the Organization (e.g., AER, EEOI, CII) or another internationally established tool should be selected. A ship subject to regulation 28 is likely to use the CII as its monitoring tool. | Noted. Noted. Noted. Noted. |
| 4.3.2. | <China> It is proposed to modify this paragraph as: “If used, it is recommended that the EEOI is these CIIs should be calculated in accordance with the Guidelines developed by the Organization, adjusted, as necessary, to a specific ship and trade.” <Sweden> The paragraph names only EEOI calculations, while the footnote refers to both EEOI and CII guidelines. Revision needed? | Noted. Noted. |
| 4.3.4 | <Brazil> Suggested Addition: “It is highly advised to conduct monitoring on a daily basis for checking consistency of data and verification assistance. The ship’s fuel oil consumption should be monitored using daily reporting, such as noon reports, or higher frequency data.” Even though it is recommended to use daily data, we should encourage and allow the use of higher frequency data (e.g. hourly, every minute etc.). <Greece> It is highly advised to conduct monitoring on a daily basis regular intervals for checking consistency of data and verification assistance. The ship’s fuel oil consumption should be monitored using daily reporting, such as noon reports. <Sweden> Is there a little contradiction between these two: 4.3.5 tries to minimize the burden, while 4.3.4. does the opposite by “highly advising to conduct monitoring on a daily basis…” using noon reports. Are these daily checks necessary? | Duly reflected. Duly reflected. Noted. |
| 4.3.5 | <USA> 4.3.5 It should be noted that, in order to avoid unnecessary administrative burdens on ships' staff, monitoring should be carried out as much as possible by shore staff, while the data can be automatically transferred or automatic data transfer systems, utilizing data obtained from existing required records such as the official and engineering log-books and oil record books, etc. Additional data could be obtained as appropriate | Duly reflected. |
| 4.3.7 | <China> It is proposed to modify this paragraph as: “When a ship diverts from its scheduled passage to engage in search and rescue operations, it is recommended that data obtained during such operations is not used in ship energy efficiency monitoring, and that such data may should be recorded separately.” | Duly reflected. |
| 4.4.3 | <China> The first sentence in this paragraph is suggested to be modified as: “For this process, procedures for self-evaluation of ship energy efficiency management should be developed.” | Duly reflected. |
| 5.1 | <USA> Unclear as to where this change comes from. Part 1 of the SEEMP applies only to ships and emission reductions are to be delivered by ships and not “companies.” This change should be rejected, and the text should refer to “ship operators.” | Duly reflected. |
| 5.2.3 | <USA> 5.2.3 Weather routeing guidance has a high potential for efficiency savings on specific routes. It is commercially available for all types of ship and for many trade areas. Significant savings can be achieved, but conversely weather routeing may also increase fuel consumption for a given voyage. | Duly reflected. |
| 5.2.4 | <USA> Language here lacks clarity we suggest deleting. 5.2.4 Good early communication with the next port should be an aim in order to give maximum notice of berth availability and facilitate the use of optimum speed where port operational procedures support this approach. Charter parties may have to be revised to allow on time arrival. | Duly reflected. |
| 5.2.9 | <USA> Unclear as to where this change comes from. Part 1 of the SEEMP applies only to ships and emission reductions are to be delivered by ships and not “companies.” This change should be rejected, and the text should refer to “ship operators.” It is recognized that under many charter parties the speed of the vessel is determined by the charterer and not the ship operators | Duly reflected. |
| 5.3.1. | <Italy> Italy has concerns on the added sentence “In general, trim optimization is effective and measurable on low block coefficient ships such as containerships where the generated wave resistance is increased” and prefers it to be deleted unless specific robust argumentation are provided. | Duly reflected. |
| 5.3.10 | <USA> Unclear as to where this change comes from. Part 1 of the SEEMP applies only to ships and emission reductions are to be delivered by ships and not “companies.” This change should be rejected, and the text should refer to “ship operators.” 5.3.10 Docking intervals should be integrated with the ship operator company's ongoing assessment of ship performance. | Duly reflected. |
| 5.6 | <Greece> We cannot support the suggested addition. Please remove. | Noted. |
| 5.10.2 | <Greece> The financial feasibility of a specific energy efficiency enhancement can be evaluated by various means. The easier way is to estimate the Return of Investment (ROI) time. Measures with lower ROI have the lowest cost but not necessarily the best results in performance improvement. <USA> We suggest that this paragraph clarifies that the measures it is referring to are those applicable to Part I of the SEEMP. 5.10.2 All measures identified in this document, as applied to Part I of the SEEMP, are potentially cost-effective in case of high oil prices. The financial feasibility of a specific energy efficiency enhancement can be evaluated by various means. One way would be The easier way is to estimate the Return of Investment (ROI) time. However, while Mmeasures with lower ROI may have the lowest cost, this does not guarantee but not necessarily the best results in performance improvement. Clearly, this equation is heavily influenced by the remaining service life of a ship and the cost of fuel. | Duly reflected. Noted. |
| 9.1 | <USA> This deletion creates confusion as to the applicability of this section. In addition, this is a requirement by the regulation, therefore the word “shall” need to remain. We suggest the following text edit: Regulation 26.3.1 of MARPOL Annex VI specifies that, for certain categories of ships of 5,000 GT and above, on or before January 2023, the SEEMP shall include: -deleted should -deleted as applicable | Duly reflected. |
| 9.2 | <China> It is proposed to add “9.2.6 to develop corrective actions, as applicable” in this paragraph. <USA> 9.2.2 - to describe the methodology that will be used to calculate the ship’s attained CII value and report this value to the ship’s Administration; 9.2.5 – delete as applicable | Duly reflected. Duly reflected. |
| 9.3 | <USA> US Comment: Test edit to make an accurate reference to the Regulation. The required annual operational CII is to should be calculated in accordance with regulation 28 and taking into account the Guidelines developed by the Organization | Duly reflected. |
| 9.4 | <China> It is suggested to replace “calculations” with “calculation methodologies”, since only the methodology of calculation needs to be included in the SEEMP Part III, rather than the details of the calculation. <IACS> It is suggested to delete“calculations”, due to the fact that CII rating is the responsibility of administration and/or RO | Duly reflected. Duly reflected. |
| 9.5 | <Italy> Italy supports, in general, the proposal made and suggests the following modification (in blue) that appears to be in line with paragraph 14.1 indication of the Resolution A.911(22): 9.5 The ship’s operational carbon intensity rating is to should be calculated in accordanceconsistent taking into account with the Guidelines developed by the Organization. <Norway> We propose to add “and” as both the CII and rating should be calculated: The ship’s operational carbon intensity and rating <Russian Federations> We propose to delete the words “calculations and descriptions of the ship’s operational carbon intensity rating and” because the carbon intensity rating should be calculated by Administration or RO and mentioned in SoC, and there is no requirement to include calculations and descriptions of rating into SEEMP: 9.4 In addition, pursuant to regulation 28 of MARPOL Annex VI, Part III of the SEEMP shouldis further to include calculations and descriptions of the ship’s operational carbon intensity rating and consequential plan of corrective actions for ships that are rated D for 3 consecutive years or rated as E. | Duly reflected. Noted. Duly reflected. |
| 9.6 | <USA> We do not support this edit. The text should revert to “strongly encouraged” 9.6 Ships of 5,000 gross tonnage and above that are subject to regulations 26.3 and 28 of MARPOL Annex VI are strongly encouraged to should review Part I of their SEEMP to revise it as needed to reflect the actions taken to achieve the ship’s CII requirements. | Duly reflected. |
| 9.7.1 | <Finland> Delete "estimated", because Regulation 26.3.1.2 requires the required annual operational CII as specified in regulation 28 for the next three years. (The formula is given in regulation 28.4.) <USA> Delete “estimated”. | Duly reflected. Duly reflected. |
| 9.7.2 | <China> According to Appendix 2bis, the ship's attained annual operational CII within the last three years should be prepared. Therefore, the text “for the preceding calendar year” in this paragraph is proposed to be deleted. | Duly reflected. |
| 9.7.3 | <USA> Delete “if applicable” | Duly reflected. |
| 9.8 | <USA> This seems to be a typo. It should read: “rely” instead of “relay” | Duly reflected. |
| 9.9 | <USA> Add “voluntarily” | Duly reflected. |
| 9.10 | <China> It is suggested to delete this paragraph, since the requirements of this paragraph have gone beyond the scope MARPOL Annex VI. New 9.10 It is suggested to add a new paragraph 9.10 “9.10 Part III of the ship’s SEEMP need not be updated unless voluntarily modifications or necessary corrective actions are involved.” to clarify that there is no need to revise Part III of the ship’s SEEMP annually. In other words, "Required CII over the next 3 years, attained CII and rating over three consecutive years" in FORM OF SHIP MANAGEMENT PLAN TO IMPROVE ENERGY EFFICIENCY (PART III OF THE SEEMP) does not need to be updated unless any part of Part III of the ship’s SEEMP has been revised. <Italy> Italy supports the changes made, however is of the opinion that a place where a records need to be retained should be clearly indicated to avoid confusion. In this regard and having considered that the records are relevant also under SMS (ISM Code), Italy suggests the follows,: 9.10 Relevant records should be retained on board and by company for a minimum of three years. | Noted. Noted. |
| 10.1 | <USA> These are regulatory requirements; therefore the change should be rejected and revert it back to “shall” | Duly reflected. |
| 10.2 | <Denmark> The Data collection plan should include data quality control measures, which should be incorporated into the existing shipboard Safety Management System. – Suggest to change “should” with “could” with reference to 2021 revised Marpol Annex VI regulation 26.1 <Greece> The suggested addition has been already provided in paragraph 7.6. Please remove. <Norway> The method on how to obtain data should be kept in SEEMP Part II and we propose to delete this point from 10.2: 10.2 In describing the calculation methodology, Part III of the SEEMP should include a detailed description of the data required for the calculation of the attained annual operational CII and how that data will be obtained. The data collection should follow the relevant methodology and relevant requirements on the Fuel Oil Data Collection System pursuant to regulation 27 of MARPOL Annex VI (see Part II of these guidelines) should be updated to include all data required. <IPTA> Change the words “should be incorporated ” to “could be incorporated ” as the Data Collection Plan is not required to be part of the shipboard management system. <Sweden> Sweden does have a questions to the additional wording in this paragraph starting with “The Data Collection Plan…” We support the addition itself, however we question the placement of these provisions. Since this additional wording refers to the “Data collection Plan” which is included in Part II of the SEEMP isn’t it more logical to place these provisions under Part II of SEEMP? For instance under p.7.6 data quality? <USA> Insert: 10.2 In describing the calculation methodology, Part III of the SEEMP should include a detailed description of the data required for the calculation of the attained annual operational CII and how that data will be obtained. The data collection should follow the relevant methodology and relevant requirements on the Fuel Oil Data Collection System pursuant to regulation 27 of MARPOL Annex VI (see Part II of these guidelines). In describing the methodology, Part III of the SEEMP should include a detailed description of the data required for the attained annual CII calculation and how that data will be obtained. <WSC> We are concerned with the text underlined below, since the plan is not required to be part of the SMS (see paragraph 3.5). This text could be deleted, and so capture cases where it forms, and does not form part of the SMS. The Data Collection Plan should include data quality control measures which should be incorporated into the existing shipboard Safety Management System. Additional measures to be considered could include a procedure 10.2.2 Delete “to” insert “for” | Duly reflected. Duly reflected. Duly reflected. Duly reflected. Noted. Noted. Duly reflected. Noted. |
| 10.3 | <USA> We restate that this paragraph should be deleted as it is not consistent with regulation 27, which details in regulation 27.4-7 and 27.10bis the requirements in case of a transfer of a ship. Consistent with our comment above, we suggest this new 10.3bis paragraph be deleted. These suggestions risks making the non-binding guidelines more complicated and to require more than what has been agreed through Regulation. If parties would like to add additional requirements these should be adopted through amendments to MARPOL Annex VI. | Noted. |
| 11.1 | <Sweden> The sentence is very long, complex and describes several possible situations, which makes it difficult to read and understand. <USA> This is a regulatory requirement. We suggest deleting this addition. This is a regulatory requirement regardless of whether reduction factors are known. Part III of the SEEMP should shall describe the required annual operational CII values for the ship for each of the next three years, or for all the years for which the reduction factors (Z factors) are known in the case the Z factors have not been determined for the entire period of the next three years, calculated in accordance with regulation 28 of MARPOL Annex VI and taking into account the Guidelines developed by the Organization , as the basis for those calculations. | Noted. Duly reflected. |
| 12.1 | <Norway> “Ship specific” is already stated in 12.2 and is not needed here. Alternatively, 12.2 could be deleted. <Sweden> Perhaps, there is no need for addition of wording in red: “The 3-year implementation plan, which is ship specific,..” since the next point, 12.2 says the same: “The 3-year implementation plan should be ship specific.” Even the point 12.3 contains description “spcific”, and if it implies “ship specific” then there is a triple repetition. Perhaps the easiest is to delete 12.2? (the same could apply to points 15-2-2 and 15.2.5. but in this case we repeat only twice). <USA> SEEMPs are ship specific, this text is redundant and unnecessary. 12.1 The 3-year implementation plan, which is ship specific, should describes the measures the ship plans to take to continue to achieve the required annual operational CII over the next | Duly reflected. Noted. Duly reflected. |
| 12.2 | <USA> Delete “should be” add “is” | Duly reflected. |
| 13.1 | <USA> The U.S. disagrees with these deletions. The purpose of self-evaluation is to evaluate the effectiveness of the planned measures and their implementation 3-year implementation plan, to deepen the understanding on the overall characteristics of the ship’s operation such as what types of measures can/cannot function effectively, and how and/or why, | Duly reflected. |
| 13.2 | <USA> 13.2 For this process, procedures for self-evaluation of the ship’s energy usage and carbon intensity, along with its planned measures and their 3-year implementation, plan should be developed and included in this section of the SEEMP. Furthermore, self-evaluation should be implemented periodically by using data collected through monitoring, and the evaluation methodology should be developed based on such data. | Duly reflected. |
| 13.3.1 | <USA> The U.S. disagrees with this edit. We prefer the term “regular” | Noted. |
| 13.5.7 | <USA> “of” instead of “if” | Duly reflected. |
| 13.5.8 | <USA> this sentence seems unnecessary. .8 Where relevant, iIdentifictation of critical factors that contributed to missing the CII target. The critical factors should allow to further prioritize the measures to be implemented for reaching the goal or maintaining the ship operational rating. | Duly reflected. |
| 13.3.3. | <Denmark> Propose the phrase “periodical review” should be more precise. Periodical review is not clear. | Noted. |
| 14 | <IACS> Para 14 and sub-paragraphs describe the process of the review and update of Part III of the SEEMP. The given format in Appendix 2bis is not facilitated to include a log for when it has been reviewed and updated and identify which parts have been changed (para 14.6). | Noted. |
| 14.2 | <China> It is suggested to modify this paragraph as follows, to avoid duplication with the requirements of paragraph 14.5 and to be consistent with the proposed modification above (New 9.10): “The company should ensure that the SEEMP is reviewed and updated at regular intervals, when necessary as per paragraph 9.10 preferably every year, to include as a minimum, the required annual operational CII.” | Duly reflected. |
| 14.5 | <USA> US Comment: The U.S. disagrees with these edits as they change the meaning for this paragraph. The intent was to provide a list of what needs to be updated. We suggest reverting the text to its previous version (including the suggestions provided by the U.S. in the previous round, as follows: 14.x The SEEMP should be updated, taking into account, but not limited to, the following: .1 The required annual operational CII for the next three years; .2 The timeline for the review process; .3 An update of the 3-year implementation plan, where applicable, and; .4 A plan of corrective actions, where applicable. | Noted. |
| 15.1.2 | <Norway> We suggest replacing “on” with “including” <USA> For a ship that is required to develop a plan of corrective actions in accordance with regulation 28.7 of MARPOL Annex VI, a revised SEEMP on including the corrective actions for CII reduction shall be submitted to the Administration or any organization duly authorized by it for verification. preferably The revised SEEMP should be submitted together with, but in no case later than 1 month after reporting the attained annual operational CII in accordance with paragraph 2 of regulation 28 | Duly reflected. Duly reflected. |
| 15.2.2 | <USA> Delete “should be” insert “is” | Noted. |
| 15.2.4 | <Norway> The added text seems to be repeated partly in 15.2.1 and in 15.2.5. Some editing should be done after the final content is in place | Noted. |
| 15.2.5/ 12.3 | <China> The explanations of SMART in paragraph 15.2.5 and paragraph 12.3 should be consistent 12.3 The 3-year implementation plan should be SMART (Specific, Measurable, Achievable, Realistic, and Timely). It should could include: 15.2.5 The plan of corrective actions should be SMART (Specific, Measurable, Achievable, Realistic, and Time bound). It could include: | Noted. |
| 15.2.5.3 | <USA> The U.S. disagrees with this deletion. .3 Calculation showing how the combination of measures will achieve the required CII and at what point in time; | Noted. |
| 15.2.5.4 | <Greece> Designation of qualified responsible a company personnel to be responsible for the implementation of corrective actions, monitoring and recording performance throughout the year for the reviewing of the effectiveness of the corrective actions; and | Duly reflected. |
| 15.2.9 | <USA> The U.S. disagrees with this deletion. 15.2.9 Shipowners and operators are encouraged to make technical improvements to the ship and/or its operations that it assesses would be sufficient to achieve an “A” or “B” rating, to help prevent the ship from returning to a “D” or “E” rating in the near-term. | Noted. |
| Appendix 2bis/3&5 | <Greece> Designation of qualifiedresponsible a company personnel | Duly reflected. |
| Appendix 2bis nº5 | <China> The part III in “Description” should be deleted to be consistent with the requirements in paragraph 15.2.5. | Duly reflected. |

Q2.	For a ship that transfers to another company or flag, how this issue should be addressed in the SEEMP guidelines

Paragraph 10.3 of the base document contained a proposal addressing the situation of a transfer of the ship to another flag or company. Numerous comments were received from the CG during round 1. Given the complexity of the matter, the coordinators wish to propose an alternative paragraph 10.3bis for the CG members consideration.

The CG members are invited to indicate to comment on this specific issue with a view to facilitating a way forward.

| Paragraph | Comment | Coordinators’ remark |
| --- | --- | --- |
| 10.3 bis | No comments were provided by Brazil, Chile, China, CSC, Pacific Environment Prefers existing text EC, Norway, UK Paragraph 10.3bis supported by Canada, Denmark, Finland, France, Greece, Liberia, ICS, Intertanko, IPTA, Republic of Korea, Sweden |  |
|  | <Cyprus> No comments on the procedures par-10.3bis related to the provisions that should be included into the SEEMP PART II in case of re-flagging or change of company. <EC> Even though we value the explanations provided, we find the coordinators proposal unnecessarily descriptive and would therefore prefer the existing text. <Greece> It is generally supported. There is a good value in providing the details of what Part III of SEEMP should include. It would also be of great assistance if a template was provided including the detailed procedures/ descriptions, ensuring uniformity of the same while leaving the particularities of the procedures’ description at the discretion of the editor of the SEEMP. <IACS> Suggested to replace” If there is a possibility that” with “If applicable” Suggested to replace “the losing Administration or the Administration of the losing company” with “the current Administration and/or the gaining Administration” <Italy> Italy is of the opinion that the paragraph 10.3 is clear enough and it is preferred taking into account that the similar wording is today used for Continuous Synopsis Record. In addition, Italy suggests clearing identify what elements and data need to be transferred between flag/company. Such information should be included in the Annex of the guidelines <Norway> Norway does not support the inclusion of 10.3bis and would prefer to retain 10.3. There is no need for detailed data from a previous period in order to calculate a CII for the full 12-month period. The verified data required to be verified and submitted to the IMO database is sufficient. We prefer the simpler basis of paragraph 10.3. However, the text should distinguish between change of company and change of flag and needs to reflect that data is never verified by the company. Proposals to this effect in Round 1 from several CG members were unfortunately not taken into account, so we repeat our proposal here as we think this is the only viable approach clearly spelling out the responsibilities, but without being overly prescriptive. 10.3 In case of a transfer of a ship from one Company to another according to Regulation 27.5 or 27.6 of MARPOL Annex VI to another flag or company, all relevant data necessary for the calculation of the attained CII should be submitted by the former flag or company to the receiving flag or company within 12 months after the date of transfer. These data should be verified according to Regulation 6.7 of MARPOL Annex VI. by the former flag or company. The format of the transfer should be agreed between former and receiving flag or company and should be such that the receiving flag or company can use it in the calculations of the attained CII for the whole year, in which the transfer takes place. 10.3bis In case the former Company does not transfer the required data, the Administration may make relevant data submitted to the IMO Fuel Oil Consumption Database available to the receiving Company. In case of a transfer of both company and Administration concurrently, the incoming administration may request the Organization for access to the data according to Regulation 27.10bis. If no such data is available, the Attained annual operational CII can be calculated and verified using the available data covering a period of the preceding calendar year as long as practically possible 10.3ter In case of transfer of a ship from one Administration to another according to Regulation 27.4 of MARPOL Annex VI the data needed for calculating the annual attained CII is already in possession by the relevant Company and no further exchange of data is needed. <Russian Federation> We propose to replace the words “If there is a possibility that…” with “Where applicable…” <Sweden> Sweden can accept the proposed text. However, the text in the paragraph might be simplified. <UK> The UK questions if this could be a repetition of the requirements for data to be recorded. If the ship complied with the requirements for data recording, there is not a need to specify in the SEEMP the list of data that will be transferred to the new owners. Similarly, there is not then a need to give details on how the aggregation should be made, and so on. The original paragraph 10.3 is preferred for its simplicity and lack of prescription. <USA> The United States suggest deleting this paragraph. Consistent with our comment to delete paragraph 10.3, we suggest this new 10.3bis paragraph be deleted. These suggestions risks making the non-binding guidelines more complicated and to require more than what has been agreed through Regulation. If parties would like to add additional requirements these should be adopted through amendments to MARPOL Annex VI. <WSC> We consider that the below should be made explicit, otherwise, there may be an incentive to change company/flag just for the sake of postponing corrective actions: In cases of transfer, while the content of Corrective Action Plans may change the time period for the plan should remain unchanged and not pro-longed. | Noted. Noted. Noted. Duly reflected. Noted. Noted. Duly reflected. Noted. Noted. Noted. Noted. |

Q3.	In developing a plan of corrective actions, whether the plan should aim at achieving the required annual operational CII within a defined timeframe such as the following calendar year

Paragraph 15.2.1 of the updated base document specifies that the purpose of the plan of corrective actions is to set out what actions a ship that was rated “D” for 3 consecutive years or “E” for one-year plans should take to achieve the required annual operational CII for the following calendar year.  Although MARPOL Annex VI regulation 28.7 addresses this matter, the timeframe for the ship to achieve the required annual operational CII is however not specified. It is the coordinators’ view that the SEEMP guidelines could include an indicative timeframe for this which could be the following year relative to the year of the reporting of the attained annual operational CII.

The CG members are invited to indicate to comment on this specific issue with a view to facilitating a way forward.

| Paragraph | Comment | Coordinators’ remark |
| --- | --- | --- |
| 15.2.1. | No comments were provided by Brazil, Chile, IACS, Russian Federation Paragraph 15.2.1. supported by Canada, Denmark, EC, Finland, France, Liberia, Norway, UK, USA Accepted by CSC and Pacific Environment, Sweden |  |
|  | <Canada> Support the timeframe indicating one year sufficient to implement corrective action. <China> There is no need to set a timeframe for the ship to achieve the required annual operational CII, otherwise it would go beyond the requirements of the Convention. <Cyprus> Text is supported in general since the CII should be within the allowable limit constantly, however instructions / Provisions in the cases that the corrective actions that need to the taken by the company in order to improve the CII from category “D” or “E” to category “A” or “B” or “C” require more time (e.g innovative technologies) may be taken into the consideration. Taking into the account that the SoC shall be issued by latest end of May each year (Reg 6.6) remaining period may be considered as insufficient in some cases for completion of such corrective actions. <EC> We agree with the coordinators proposed way forward. Yet, we consider it relevant to also reflect in the Verification guidelines for Administrations further guidance related to the progress made in the implementation of the plan of corrective actions and appropriate follow-up measures, particularly in case where those corrections have not been made or insufficient planned improvements achieved. <Greece> We support the inclusion ‘following year’. We don’t support the wording ‘the required annual operational CII’. We have a preference on the wording suggested by Norway: The purpose of the plan of corrective actions is to set out what actions a ship that was rated “D” for 3 consecutive years or “E” for one-year plans to take to achieve a “C” rating or better by the following calendar year. <ICS> We agree with the proposed timeframe. However, for clarity we suggest this should not be indicative, but a firm requirement for the annual operational CII to be achieved within the following year relative to the year of the reporting of the attained annual operational CII. <Intertanko> We agree the result of application of the plan of corrective action should correspond to one calendar year as the attained annual CII value must be calculated with the one calendar year reported data. <IPTA> IPTA would like to see the timeframe to be “as agreed” leaving some flexibility for the shipowner to include options they may require scheduled dockyard work to comply. <Italy> NOTE: When considering the above proposed text, Italy would remind that the rating for the previous year may be confirmed only by April-May. This means that there will only be 7 months remaining in that years reporting period. Italy is of the view that the sentence should be modified as follows to accept the entire C rating band instead of the Required CII which is the middle point of C rating: “.2.1 The purpose of the plan of corrective actions is to set out what actions a ship that was rated “D” for 3 consecutive years or “E” for one-year plans should take to achieve the required annual operational CII or better at least a “C” rating for the following calendar year.” <Norway> We agree with the current proposal in 15.2.1. The intention is to ensure that the rating is brought back to a C or better rating as soon as possible – i.e. the following calendar year. The systematics allows for a continuous monitoring and self-evaluation on the effectiveness of the measures which should enable companies to be aware of the need for implementing corrective action by the end of the reporting period, if not earlier. <Republic of Korea> We propose to update paragraph 15.2.1 as follws; “The purpose of the plan of corrective actions is to set out what actions a ship that was rated D for 3 consecutive years or E for one-year should take to continuously achieve the required annual operational CII including or better for the following calendar year.” <UK> The UK supports. We also support specifying that correction plans achieve a rating improvement the following calendar year. <WSC> It will be very difficult in practice if only one year is allowed for the plan of corrective actions – ships will be rated at the start of the 12 month period then shipowners need to consider what to do, change network, agree work with shipyards etc. We suggest a timeframe of two years as a reasonable compromise. At the same time, we support that it should not be indicative, there should either be a fixed time frame or no time frame. Otherwise, we risk an unlevel playing field for those who feel/are obliged to follow guidelines. We propose the following text: 15.2.1 The purpose of the plan of corrective actions is to set out what actions a ship that was rated “D” for 3 consecutive years or “E” for one-year should take to achieve the required annual operational CII or better. The corrective plan should detail how the ship will achieve the required CII within a period of two years. When considering the above text we would remind members of the group that the rating for the previous year may only be confirmed only by April-May. This means that there will only be 7 months remaining in that years reporting period. | Noted. Noted. Noted. Noted and reflected. Noted. Noted. Noted. Duly reflected. Duly reflected. Noted. Noted. Noted. Noted. |
| 13.4.4/Additional note | <Greece> Upon confirmation of the Administration, compliance period may be longer depending on applied corrective measure. | Noted. |

Q4.	Appendices in the SEEMP guidelines, whether additional standardized sample formats are needed such as for the plan of corrective actions or for transferring data to a new company in case of transfer

The base document already contained updated appendices and two new additional ones. The coordinators would like to seek views on the current appendices, and on whether additional appendices are needed such as standardized sample formats for the plan of corrective actions or for transferring data to a new company in case of transfer.

The CG members are invited to indicate to comment on this specific issue with a view to facilitating a way forward.

| Appendix | Comment | Coordinators’ remark |
| --- | --- | --- |
|  | No comments were provided by Brazil, Chile, Cyprus, Liberia, Russian Federation, USA <China> Recommend the following sample formats to be provided: - transferring data to a new company in case of transfer. - certain correction factors and/or voyage adjustments <Denmark> Fine the way it is drafted now. We support that it is not too prescriptive. <Finland> Finland supports the proposed standardized sample formats. <Greece> We fully support the idea of providing additional appendices in sample format for the corrective actions and for transferring data to a new company in case of transfer. Please also note that the base document has no reference to appendices 1, 2 and 2 bis. This would need to be amended. <IACS> Recommended the following sample formats to be provided: - transferring data to a new company in case of transfer. - certain correction factors and/or voyage adjustments <ICS> We suggest the extent of appendices is sufficient However, including a worked example for each would help to ensure consistency of responses from shipowners. <Intertanko> We suggest the current templates need to identify measurement and reporting of amounts of fuel consumptions which may qualify for corrections under the final G5 guidelines. For guidance and consistency in reporting, it will be helpful to fill in examples for each table. <IPTA> 4.1.2 Add “Port and Berth management” after weather routing. <UK> The UK supports adding a standardized sample for the plan of corrective actions. We do not have a preference on a standardized sample for transferring to a new company. <WSC> The appendices are supported. | Noted. Noted. Noted. Noted. Noted. Noted. Noted. Duly reflected. Noted. Noted. |
| Appendix 2, Para 10 and 11 | <Republic of Korea> There are no relevant contents regarding data storage and monitoring strategy in guidelines. Therefore, the paragraph 10(Data storage and securing) and 11(Monitoring strategy) should be deleted | Duly reflected. |
| Appendix 2bis | <IACS> A sample format for risk analysis will be very helpful as a guidance for company and may be added as an Appendix to the Guidelines. | Noted. |
| Appendix 2bis, paragraph 1 | <CSC and Pacific Environment> We propose to add a section to the table to record the ship’s calculated operational CII for the previous three consecutive years before any correction factors or exclusions have been applied, as follows: Calculated CII before any correction factors or exclusions; <year -1>. <year -2>. <year -3>. | Duly reflected. |
| Appendix 2bis, nº3 | <Norway> Due to the complex nature of describing measures to reduce energy use and carbon intensity, and how to verify the effectiveness of the implementation plan, we recommend to provide a more detailed template to ensure a consistent level of description and application across the fleet Proposed template for item 3 in the appendix, including three examples: Calculation showing the combined effect of the measures and that the required operational CII will be achieved Year; Estimated attained CII; Required CII. <year>:. <year + 1>. <year + 2>. | Duly reflected. |
|  | <Norway> In line with the above template for the implementation plan, we suggest three template tables: - Evaluation of the effectiveness of the implemented measures - Analysis of other causes for inferior rating - Additional measures or revised measures to be added to the implementation plan Proposal: - Plan of corrective actions (if applicable) Analysis of causes for inferior CII rating Cause; Analysis of effect; Actions. Port congestion. …. Analysis of measures in the implementation plan Measure; Analysis of effect; Actions. Weather routing. Speed optimization. Optimum propeller and propeller inflow considerations. …. List of additional measures and revised measures to be added to the implementation plan Measure; Impact on CII; Time and method of implementation and responsible personnel; Impediments and contingency measures. …; Milestone; Due; Responsible.; Impediments; Contingencies.. | Duly reflected. |
| Appendix 3 | <CSC and Pacific Environment> We propose to add a row to the table to record “Calculated CII before any correction factors or exclusions” above the “Attained annual CII” row <EC> We would propose adding two rows instead of the current “Attained annual CII” to reflect the latest MEPC 77 outcomes (endorsing the ISWG-GHG 10 Report recommendations) and therefore reporting both the uncorrected and corrected Attained annual CII values. As regards to the existing appendices, we consider that current ones adequate. However, we would favor including additional ones built on specific proposals from the group, in particular any templates that could facilitate harmonized reporting e.g. 3-year implementation plan, plan of corrective actions, self-evaluation and for the data transfer between administrations and/or companies. More generally, we agree with the recommended way forward to the entire guidance, together with our above proposed amendments; however, same as of the relevance of the content, we are of the view that technical rules setting data exchange/reporting formats - including electronic templates - should be developed within a 1 (one) fully automated system with a view to ensure their uniform implementation. This will greatly facilitate the communication while minimizing the administrative burden between all actors involved (companies, administrations & ROs) and later with the technical Secretariat at the IMO hosting and managing the DCS. In simple words: no exchange of files from/to different systems; instead, a single ‘stop-shop’ for everyone (reference is made to the EU MRV Regulatory Framework system: THETIS-MRV). Something to be considered in close connection with the IMO DCS Review debate; therefore, we would appreciate if these comments could be later reflected in the final report of the CI Reduction CG. | Duly reflected. Duly reflected. Noted. Noted. |

Q5.	Any other issues

The CG members are invited to raise any other issues which are not currently addressed, where applicable with a concrete proposal.

| Item | Proposal | Coordinators’ remark |
| --- | --- | --- |
| 4.1.2 | <WSC> A key part of optimizing operational efficiency is to minimize time spent at anchor or slow steaming because of port congestion and non-availability of berths. We therefore propose the following insertion highlighted in yellow: 4.1.2 Recognizing that there are a variety of options to improve efficiency and reduce carbon intensity (e.g. – speed optimization, confirming berth availability and arrival time with port of destination, weather routing and, hull maintenance, andretrofitting of energy efficiency devices, and use of alternative fuels), for example – and that the best package of measures for a ship to improve efficiency and reduce carbon intensity depends differs to a great extent depending upon ship type, cargoes, routes and other factors, the specific measures for the ship to improve energy efficiency and reduce carbon intensity that should be identified in the first place. These measures should be listed as a package of measures to be implemented, thus providing the overview of the actions to be taken for that ship. | Duly reflected. |
| 4.2.2 | <WSC> We are concerned that one of the key determinants of ship efficiency is timely port arrival and departure, along with minimizing delays to port departure. We therefore recommend the following addition. Implementation and record-keeping 4.2.2 The planned measures should be carried out in accordance with the predetermined implementation system. Record-keeping for the implementation of each measure is beneficial for self-evaluation at a later stage and should be encouraged. If any identified measure cannot be implemented for any reason(s), the reason(s) should be recorded for internal use. It is recommended that events and operational conditions outside the control of the ships crew (for example, waiting for berths, extended port dwell times, operation in severe adverse weather) which may affect the ships rating be documented. | Noted. |
| 5.2 | <WSC> If an arrival time is agreed which forms the basis of passage planning only for the ship to be directed to wait at anchor for an extended period because there is no berth available, this wastes fuel during the passage and results in ships running auxiliary engines at anchor which degrades AER values. We therefore recommend the following addition highlighted in yellow to section 5.2. We also suggest amending the sentence on charter parties to better reflect the nature of such agreements, as underlined below. Just in time 5.2.4 Good early communication with the next port should be an aim in order to give maximum notice of berth availability and facilitate the use of optimum speed where port operational procedures support this approach. Charterparties are encouraged to cooperate to facilitate on time arrival of vessels. 5.2.5 Optimized port operation could involve a change in procedures involving different handling arrangements in ports. Port authorities should be encouraged to maximize efficiency and minimize delay. - Port authorities are encouraged to communicate with ships and advise berth availability to facilitate efficient passage planning and avoid ships anchoring on arrival because of berth non-availability. When optimizing shaft power, overall efficiency of machinery systems should be considered, it is not always beneficial to reduce shaft power if this results in having to operate oil fired boilers and auxiliary engines because load/shaft speed drops below the minimum power necessary to operate energy recovery systems and shaft generators. This may increase overall emissions. We therefore suggest the additional paragraph below. Optimized shaft power 5.2.10 Operation at constant shaft RPM can be more efficient than continuously adjusting speed through engine power (see paragraph 5.7). The use of automated engine management systems to control speed rather than relying on human intervention may be beneficial. 5.2.11 When optimizing shaft power, due attention should be given to overall power system efficiency. For example, in some cases reducing load or shaft speed below the minimum necessary to operate energy recovery systems and shaft generators may increase overall emissions. | Noted. Duly reflected. |
| 5.4.1 | <WSC> We recommend deleting the first sentence of 5.4.1 which is obsolete. 5.4 Waste heat recovery 5.4.1 Waste heat recovery is now a commercially available technology for some ships. Waste heat recovery systems use thermal heat losses from the exhaust gas for either electricity generation or additional propulsion with a shaft motor. | Duly reflected. |
| 5.9.4 | <ICS> Although Wind Assisted Propulsion Systems can provide beneficial fuel savings, the wording of this paragraph under the category “other measures” suggests it is an outlier in terms of efficiency measures. To raise awareness of the potential of WAPS we suggest including a separate section on this technology under section 5, i.e.: 5.X Wind Assisted Propulsion Systems (WAPS) Various systems are available for retrofit, including Flettner Rotors, wing sails and aerofoil kites. <WSC> Suggest deletion of 5.9.4 which is phrased in a rather negative way. Wind power is already addressed in paragraph 5.9.2. | Duly reflected. Noted. |
| 5.10.3 | <ICS> The following sentence suggests that all WAPS may be infeasible in certain circumstances: “For example, wind-enhanced power sources might not be feasible for short sea shipping as these ships generally sail in areas with high traffic densities or in restricted waterways.” At least for certain WAPS this is not correct. For example, soft sails may be deployed when conditions allow, and therefore pose no problem in the described scenarios. Hence we suggest modifying this sentence to read: “For example, certain types of wind-enhanced power sources might not be feasible for short sea shipping as these ships generally sail in areas with high traffic densities or in restricted waterways. Air draft limitations may also affect the feasibility of certain types.” | Duly reflected. |
| 5.10.5 – New Paragraph | <WSC> Suggest additional paragraph in section 5.10, below: Trade and sailing area 5.10.5 Environmental conditions and the nature of cargo carried varies between regions. For example, some routes may carry greater volumes of goods requiring careful temperature conditioning, or transit regions subject to frequent severe adverse weather conditions. This will impact energy efficiency of ships serving some routes and regions and should be recognized when considering their performance. | Duly reflected. |
| 10.3 | <Finland> Instead of "the attained CII", "the attained annual operational CII" | Duly reflected. |
| 10.4 – New Paragraph | <WSC> 10.4 Part III of the SEEMP should include a procedure to record: .1 Periods of 8 hours of greater operating in conditions where the mean significant wave height is 4.0m or greater and mean wind speed is 28 knots or greater; .2 Periods of [72] hours or greater at anchor or slow steaming waiting for a berth; and .3 Delays of [72] hours or greater to port departure time. | Noted. |
| 13.5 | <WSC> Suggest new bullet to Part III paragraph 13.9 as follows: .8 Where relevant, identifictation of critical factors that contributed to missing the CII target. The critical factors should allow to further prioritize the measures to be implemented for reaching the goal or maintaining the ship operational rating; and .9 Where factors contributing to missing a CII target are the result of external factors, such as port congestion, port delays and extended operation in severe adverse weather this should be documented along with supporting information. | Noted. |
| new Paragraph 16 | <CSC and Pacific Environment> As stated in document MEPC 77/7/29, which we co-sponsored, a ship’s attained CII and associated rating (A-E) must be publicly accessible if the market is to reward first-movers and punish laggards. Since that submission, Pacific Environment and CSC have considered how the SEEMP Guidelines might be amended in this respect. We propose to add a new paragraph 16 to the 2022 SEEMP guidelines as follows: 16 REPORTING 16 Ships subject to regulation of 28 of MARPOL Annex VI should report the following information to a public database each year: .1 the ship’s required annual operational CII for the preceding calendar year; .2 the ship’s calculated operational CII for the preceding calendar year before any correction factors or exclusions have been applied; .3 the ship’s attained operational CII for the preceding calendar year; .4 the ship’s operational carbon intensity rating for the preceding calendar year (A, B, C, D, or E); .5 the ship’s required annual operational CII for and the next three years; .6 whether any plan for corrective action is required for the ship (Y/N) .7 whether any plan for corrective action has been approved by ship’s Administration (Y/N); and .8 whether any plan for corrective action has been implemented by the ship (Y/N). | Noted. |
|  | <EC> We would like to recommend including a reference on consistency between the SEEMP, and in particular the 3 year implementation plan and/or plan of corrective actions when required, and the general energy management system that the company might have in place such one based on ISO standard 50004 which provides practical guidance for implementing an energy management system. We would also propose to perform a general consistency check during the next consultation round. | Noted. |
| Part III | <Intertanko> In Part III, there are several footnotes referring to the CII Guidelines. We would suggest that G5 are also referred to in footnote number 7 (see paragraph 9.3 of the SEEMP), footnote number 9 (see paragraph 10.1 of SEEMP), footnote number 11 (see paragraph 11.1 of SEEMP) | Noted. |
| New Para 5.2.11 | <Italy> Optimized shaft power Add the paragraph 5.2.11 reported below: 5.2.11 When optimizing shaft power, due attention should be given to overall power system efficiency. For example, reducing load or shaft speed below the minimum necessary to operate energy recovery systems and shaft generators in some cases may increase overall emissions. | Duly reflected. |
| 12.3 | We would propose changes to paragraph 12.3: - we prefer should instead of could; - we don’t understand what is meant by possible “additional measure”; - we suggest the monitoring intervals is made part of the self-evaluation and improvement plan, rather than for each measure as this may not be feasible - a new sentence in .2 in order to be more precise on the description. - Editorial changes to .4 This is also aligned with the proposed changes to Appendix 2bis. 12.3 The 3-year implementation plan should be SMART (Specific, Measurable, Achievable, Realistic, and Timely). It could should include: .1 List of measures that improve the energy efficiency and reduce the carbon intensity of the ship, including possible additional measures with time and method of implementation, necessary for achieving the required operational CII, including monitoring intervals if necessary; .2 Description of how, when the listed measures are implemented, the required operational CII will be achieved, taking into consideration the combined effect of the measures on operational carbon intensity. Explanation showing how the required operational CII will be achieved; .3 The company personnel to be responsible for the 3-year implementation plan, monitoring and recording performance throughout; and .4 Identification of possible impediments to the effectiveness of the measures for improving the energy efficiency and reducing the carbon intensity of the ship, including to possible additional contingency measures put in place to overcome and how these impediments will be overcome. | Noted. |
| 13.1 and 13.2 | <Norway> 13.1 [keep text until the last sentence]… This phase evaluation should produce meaningful feedback based on experience in the previous period, to inform enhance performance in the next period. 13.2 For this process, procedures for self-evaluation of the ship’s energy usage and carbon intensity, along with its planned measures and their implementation, plan should be developed and included in this section of the SEEMP. Furthermore, Self-evaluation should be carried out implemented periodically based on by using data collected through monitoring, and the evaluation methodology should be developed based on such data. In addition, It is recommended to invest time in identifying the cause-and-effect of the ship’s performance duringin the evaluated period to identify measures for improving performance during the next period. | Noted and reflected. |
| 13.3 | <Norway> Suggested improvements: .1 Periodical internal shipboard and company audits to verify implementation and the effectiveness of the system .2 Evaluate such audit reports and should implement corrective actions including preventive or modifying measures to prevent recurrence; | Duly reflected. |
| 13.5.6 | <Norway> 6 Where relevant, actions that will be taken to bring the ship back into better CII ratings including estimated quantification of the additional expected reduction in carbon intensity; | Duly reflected. |
| 13.5.7 | <Norway> .7 Where applicable, identification if a plan of corrective actions is required, if so, the plan should include items listed under 14.2.5 to ensure actions taken will bring the ship out of inferior performance; and | Duly reflected. |
| 14.5 | <Norway> We do not see the need for this paragraph and propose to delete it. | Duly reflected. |
| 15.2.5 | <Norway> We propose several changes to the text, including to bring it in line with the text for the implementation plan: 15.2.5 The plan of corrective actions should be SMART (Specific, Measurable, Achievable, Realistic, and Time bound). It should could include: .1 Analysis of the cause for the inferior CII rating ; .1bis Analysis of the performance of implemented measures .2 List of additional measures and revised measures to be added to the implementation plan with time and method of implementation, necessary for achieving the required operational CII,designed to ensure that the ship attains the required annual operational CII with timing of implementation, including monitoring intervals, and how to implement such measures; .4 The company personnel to be responsible for the added and revised measures in the implementation plan, monitoring and recording performance throughout; Designation of responsible company personnel to be responsible for the implementation of corrective actions, monitoring and recording performance throughout the year for the and reviewing of the effectiveness of the corrective actions; and .5 Identification of possible impediments to the effectiveness of the measures for improving the energy efficiency and reducing the carbon intensity of the ship, including to possible additional contingency measures put in place to overcome and how these impediments put in place and how these will be overcome. | Duly reflected. |
| 15.2.7 | <Norway> We suggest to delete this as it is covered by 15.2.5.5 | Noted. |
| Appendix 2 | <Norway> We can not see why items 10 and 11 are included. They are not mentioned in the main part of the guidelines | Duly reflected. |
| Appendiz 2 nº10 | <IACS> “Data storage and securing” suggest to be revised as “Data storage and security” or “Data protection and storage” | Duly reflected. |
| Appendix 2 nº11 | <IACS> From the guidelines, text is not clear. How this section would be different from monitoring requirements required as given in Appendix 2bis 3/V and 5/VI? | Noted. |