2017 2022 GUIDELINES FOR ADMINISTRATION VERIFICATION

OF SHIP FUEL OIL CONSUMPTION DATA AND OPERATIONAL CARBON INTENSITY

1	INTRODUCTION

1.1	Regulation 22A27 of MARPOL Annex VI establishes the IMO Ship Fuel Oil Consumption Database, to be administered by the Organization, to which each Administration will submit relevant data for their registered ships of 5,000 gross tonnage (GT) and above. Regulation 22A27.7 specifies that "the data shall be verified according to procedures established by the Administration, taking into account guidelines to be developed by the Organization". This document contains the Guidelines referred to in that regulation and is intended to assist Administrations in developing their own verification programme.

1.2	Regulation 28 of MARPOL Annex VI establishes the operational carbon intensity rating mechanism. Regulation 28.6 specifies that “the attained annual operational CII of a ship shall be documented and verified against the required annual operational CII to determine operational carbon intensity rating A, B, C, D or E, either by the Administration or by any organization duly authorized by it, taking into account the guidelines developed by the Organization”.

1.3	This document contains the Guidelines referred to in Regulation 27.7 and Regulation 28.6 and is intended to assist Administrations in developing their own verification programme.

1.24	A data verification procedure should ensure the reliability of the collected data and the correctness of the attained annual operational CII, while minimizing the costs and associated burdens to the ship and the Administration.

2	DEFINITIONS

For the purpose of these Guidelines, the definitions in MARPOL Annex VI apply.

3	RESPONSIBILITIES

3.1	The responsibilities of Administrations and ships are set out in MARPOL Annex VI. These Guidelines do not change those or create any new obligations.

3.2	Under the data collection system for fuel oil consumption and the operational carbon intensity rating of ships, as specified in MARPOL Annex VI, an Administration may authorize an organization to receive the data from a ship, verify the data for compliance with the requirements, verify the attained annual operational CII against the required annual operational CII, determine the operational carbon intensity rating, issue the Statement of Compliance, and submit the data to the Organization and perform other actions authorized by the Administration with respect to the IMO Ship Fuel Oil Consumption Database. In every case, the Administration assumes full responsibility for all tasks conducted by the Administration or any organization duly authorized by it (hereinafter referred to as "the Administration").

4	VERIFICATION OF THE REPORTED DATA

4.1	To facilitate data verification, the Administration should indicate what additional documentation a ship should submit along with its annual data report. Specification of this documentation can be done on a ship basis, as part of the assessment of the Data Collection Plan, or it may be done as a general policy statement or through such other policy instruments as the Administration deems appropriate. Additional documentation to facilitate data verification may include the following, as well as other documentation that the Administration deems relevant:

.1	a copy of the verified ship's Data Collection Plan (SEEMP Part II);

.2	summaries of bunker delivery notes (BDNs), in sufficient detail to show that all fuel oil consumed by the ship is accounted for (see sample form of BDN summary set out in appendix 1);

.3	summaries of disaggregated data of fuel oil consumption, distance travelled and hours underway, in a format specified by the Administration (see sample form of data summary set out in appendix 2);

.4	information to demonstrate that the ship followed the Data Collection Plan set out in its SEEMP, including information on data gaps and how they were filled as well as how the event that caused the data gap was resolved; and

.5	copies of documents containing information on the amount of fuel oil consumption, distance travelled and hours underway for the ship's voyages during the reporting period (e.g. the ship's official logbook, oil record book, BDNs, arrival/noon/departure reports, and from auto-log data files, etc.).

.6	supported by documentary evidence, copies of the fuel oil mass to CO2 mass conversion factor provided by fuel supplier in case the type of fuel is not covered by the guidelines developed by the Organization.

4.2	In addition to the documentation described in paragraph 4.1, the Administration may request a ship to submit such documentation needed to perform a comprehensive review of a ship's annual fuel oil consumption, distance travelled, and hours underway. The Administration may request this documentation be submitted by all ships or a subset of the ships subject to its jurisdiction. This documentation may be used by the Administration to verify whether the ship followed the methodology specified in its Data Collection Plan, with a view to confirming:

.1	consistency of reported data and calculated values, including with previous reporting periods (if applicable), through recalculating the annual reported values using the underlying data, etc.;

.2	completeness of data (e.g. perform substantive testing based on reconciliation, recalculations, and document cross-check, for example with official logbook and/or arrival/noon/departure reports, Auto-log report files; recalculate hours underway and total quantities of fuel oil used, and distance travelled and hours underway); and

.3	reliability and accuracy of the data (e.g. test that the data quality procedures as described in the Data Collection Plan (see section 9 of sample form of  Data Collection Plan, as set out in appendix 2 of the 2016 Guidelines for the development of a ship energy efficiency management plan (SEEMP)) have been properly implemented, carry out site visits (typically to the Company's offices rather to the ship) to test the systems, processes and the control activities) through corroborating fuel oil consumption data with distance travelled and hours underway, comparing reported fuel oil consumption with that which is expected for the ship size, operational profile, and technical characteristics, and/or comparing reported fuel oil consumption total fuel bunkered, analysing the relevance of the fuel quantity measurement instrumentation including measurement accuracy, tolerances, calibration data and frequency,  etc.

4.3	The parameters to calculate the trial CIIs (e.g. EEPI, cbDIST, clDIST or EEOI), if voluntarily collected and reported, should be verified by the Administration following the same procedure as for the mandatory data.[Moved to 5.8]

4.3	Should any discrepancy be identified by the Administration in the reported data, it should be communicated to the Company on a timely basis for correction. On receipt of corrected data from the Company and satisfactory completion of the verification, the Statement of Compliance will be issued by the Administration.

5	VERIFICATION OF THE ATTAINED ANNUAL OPERATIONAL CII AND DETERMINATION OF THE CII RATING

5.1	To facilitate the verification of the attained annual operational CII, the Administration should indicate what additional documentation a ship should submit along with its annual data report. Additional documentation to facilitate the verification may include the following, as well as other documentation that the Administration deems relevant:

.1	a copy of the verified ship's Operational Carbon Intensity Plan (SEEMP Part III);

.2 	documents (Stability Booklet or International Tonnage Certificate) evidencing the capacity parameter of the ship in the metric relevant for the calculation of its operational carbon intensity (deadweight or gross tonnage).

.3	aggregated data of fuel oil consumption and distance travelled covering the entire calendar year to calculate the attained annual operational CII (AER or cgDIST) (see sample form of data summary set out in appendix 2);

.4	the aggregated values of the parameters and associated calculation methods to determine the annual metric value of the trial CIIs on voluntary basis, if any (see sample form of data summary set out in appendix 2- add1);

.5	supported by documentary evidence, copies of documents, including necessary evidence, containing information on the correction factors and voyage adjustments applied in the attained annual operational CII calculation, if any, during the reporting period (see sample form of data summary set out in appendix 2); and

.6	statements of compliance for previous two calendar years where applicable.

5.2	The attained annual operational CII should be given with three decimal places significant digits and should be verified using the data over a 12-month period from 1 January to 31 December for the preceding calendar year, by the Administration. In cases that the calculation of the attained annual operational CII is not possible due to the unavailability of some data, such as where a new ship is delivered after 1 January in the preceding year, the attained annual operational CII should be verified using the available data covering the corresponding period of the preceding calendar year.

5.3	In case of a ship with multiple loadline certificates or with a loadline certificate containing multiple loadlines, the highest deadweight value should be used to calculate the required and attained annual operational CII.

5.4	For a ship which permanently changes its deadweight (DWT) and/or its gross tonnage (GT) during the year, for which the conversion is in nature a method or the result of a method to improve the ship’s operational carbon intensity performance as explicitly specified in its operational carbon intensity implementation plan or corrective action plan, the required annual operational CII should always be calculated and verified on the original DWT or GT value before conversion. For the year when the conversion is made, the attained annual operational CII should be calculated and verified for the entire calendar year on the average DWT or GT value weighted on distance travelled before and after conversion. For the following years, the attained annual operational CII should be calculated and verified on the new DWT or GT value after conversion.

5.5	Except for those specified in 5.4, for a ship which is regarded by the Administration as a newly constructed ship as per Regulation 5.4.3 of MARPOL Annex VI due to major conversion, including extensive changes of carrying capacity and/or ship type during the year, the required and attained annual operational CII should be calculated and verified as per a newly constructed ship for the period after conversion. The data before conversion should still be reported for verification but will not be included in the calculation and verification of the attained annual operational CII.

5.6	For a ship which has undergone a transfer addressed in regulations 27.4, 27.5 or 27.6 of MARPOL Annex VI, the losing Administration neither needs to verify the attained annual operational CII nor to determine the annual CII rating of the ship for partial year. The attained annual operational CII should be verified by the receiving Administration using the data over an entire calendar year. In such cases, the aggregated data necessary to calculate the attained annual operational CII before transfer, which should have already been verified by the losing Administration, can be directly used by the receiving Administration without further verification (see sample form set out in appendix 3 and appendix 3 - add1).

5.7	The administration should determine the operational carbon intensity rating for the ship, taking into account the guidelines developed by the Organization. The required annual operational CII and the rating boundaries should be applied with three decimal places. If the attained annual operational CII happens to land on a rating boundary, the following rules should be applied:

    - if the attained annual operational CII lands on the upper or the lower boundary, it should be rated as C；

    - if it lands on the superior boundary as B; and

    - if it lands on the inferior boundary as D.

5.8	The trial CIIs (e.g. EEPI, cbDIST, clDIST or EEOI), if voluntarily calculated and reported, should be verified by the Administration following the same procedure as for the attained annual operational CII (AER or cgDIST). The Administration needs not to assign a rating to a ship based on the trial CIIs.

6	ISSUE OF A STATEMENT OF COMPLIANCE

6.1	In accordance with regulation 6.6 of MARPOL Annex VI, upon receipt of reported data pursuant to regulation 27 of MARPOL Annex VI and attained annual operational CII pursuant to regulation 28 of MARPOL Annex VI and satisfactory completion of the verification, the Statement of Compliance should be issued by the Administration.

6.2	Notwithstanding paragraph 6.1, the administration should consider whether a corrective action plan is required according to Regulation 6.8 of MARPOL Annex VI. In the case of a corrective actions pan being required but not submitted together with the attained annual operational CII, the administration should inform the Company in a timely manner that a revised SEEMP including a plan of corrective actions, has to must be submitted for verification no later than one month after reporting the attained CII. The Statement of Compliance should not be issued in such a case unless a corrective action plan is duly developed and reflected in the SEEMP and verified by the Administration taking into account the guidelines developed by the Organization.

6.3	Should any material discrepancy be identified by the Administration in the reported data and/or the calculation of required/attained annual operational CII, it should be communicated to the Company on a timely basis for clarification or correction. A discrepancy is considered material if the discrepancy or aggregation of discrepancies could influence the reported total by more than ±5%. The Statement of Compliance should not be issued in such a case unless the material discrepancy is clarified or corrected. On receipt of clarification or corrected data, required/attained annual operational CII and, if needed, a corrective action plan from the Company and satisfactory completion of the verification, the Statement of Compliance can be issued by the Administration.

***

APPENDIX 1

SAMPLE OF THE BDN SUMMARIES

[Note: the fuel oil types “methanol” and “Ethanol” will be inserted before the column “Others CF)” in the clean version, in line with 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships (resolution MEPC.308(73))]

APPENDIX 2

SAMPLE OF THE COLLECTED DATA SUMMARIES

APPENDIX 2

SAMPLE OF THE COLLECTED DATA SUMMARIES

| Date from (dd/mm/ yyyy) | *Date to (dd/mm/ yyyy) | Distance travelled (n.m) | Hours under way (hh:mm) | **exceptional conditions specified in Regulation 3.1 of MARPOL Annex VI (Y/N) | **Sailing in ice condition (Y/N) | **STS Operation (Y/N) | Fuel consumption (metric tonns) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | in total |  | **attributed to in-use refrigerated containers |  | **attributed to gas cargo cooling |  | **attributed to cargo discharge |  | **used by boiler for cargo heating |  |  | **used by self-powered discharge pump |  |
|  |  |  |  |  |  |  | ***DO/GO | … | DO/GO | … | DO/GO | … | DO/GO | … | DO/GO | … | DO/GO |  | … |
| 01/01/2023 |  | 210 | 24:00 | N | N | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 02/01/2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 03/01/2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31/12/2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX 2 - ADD 1
SAMPLE OF THE COLLECTED DATA SUMMARIES FOR THE PARAMETERS TO CALCULATE TRIAL CII METRICS ON VOLUNTARY BASIS

The following aggregated data should be additionally included in the table in Appendix 2, if one or more trial CII metrics have been applied on voluntary basis:

| Date from (dd/mm/yyyy) | *Date to (dd/mm/yyyy) | Laden distance travelled (n.m) | ****Transport work (metric of transport work ) |
| --- | --- | --- | --- |
| 01/01/2023 |  |  |  |
| 02/01/2023 |  |  |  |
| 03/01/2023 |  |  |  |
| 31/12/2023 |  |  |  |
| Annual total |  |  |  |

* In the case of daily underlying data, this column would be left blank.

** Refer to [Guidelines on correction factors and voyage adjustments for CII calculations (G5)]. Supporting documentation may be additionally submitted to facilitate the verification when necessary, such as Baplie files where the number of in-use reefer containers onboard are recorded.

*** Refer to ship types specified in 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships (resolution MEPC.308(73))

****As defined in Section 3 of Guidelines for voluntary use of the ship energy efficiency operational indicator (EEOI) circulated by MEPC.1/Circ.684.

Explanatory remarks: If bunker supply/correction data have been recorded in a Company’s electronic reporting system, the data is acceptable to be submitted in the existing format instead of submitting the data by this format.

APPENDIX 3 - SAMPLE OF THE AGGREGATED DATA BEFORE A TRANSFER OF FLAG/COMPANY ADDRESSED IN REGULATIONS 27.4, 27.5 OR 27.6 OF MARPOL ANNEX VI

| Date of transfer (dd/mm/yyyy) | Type of transfer (flag/ company/ both) | Reporting period |  | Distance Travelled (n.m) |  | Hours Under way (hh:mm) | Fuel Consumption (metric tons) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Date from (dd/mm/yyyy) | Date to* (dd/mm/yyyy) | Total distance travelled | distance to be deducted from CII calculation |  | in total |  |  | to be deducted from CII calculation |  |  |
|  |  |  |  |  |  |  | *DO/GO | LFO | … | *DO/GO | LFO | … |
| 12/05/2023 | Flag | 01/01/2023 | 11/05/2023 |  |  |  |  |  |  |  |  |  |
| 15/06/2023 | Company | 12/05/2023 | 14/06/2023 |  |  |  |  |  |  |  |  |  |
| 02/11/2023 | Both | 15/06/2023 | 01/11/2023 |  |  |  |  |  |  |  |  |  |
| …… |  |  |  |  |  |  |  |  |  |  |  |  |

APPENDIX 3 – ADD1 
SAMPLE OF THE AGGREGATED DATA BEFORE A TRANSFER OF FLAG/COMPANY ADDRESSED IN REGULATIONS 27.4, 27.5 OR 27.6 OF MARPOL ANNEX VI FOR THE PARAMETERS TO CALCULATE TRIAL CII METRICS ON VOLUNTARY BASIS

The following aggregated data may be additionally included in the table in Appendix 3, if one or more trial CII metrics have been applied on voluntary basis:

| Date of transfer (dd/mm/yyyy) | Type of transfer (flag/ company/ both) | Reporting period |  | Laden Distance Travelled (n.m) | **Transport work(metric of transport work) |
| --- | --- | --- | --- | --- | --- |
|  |  | Date from (dd/mm/yyyy) | Date to* (dd/mm/yyyy) |  |  |
| 12/05/2023 | Flag | 01/01/2023 | 11/05/2023 |  |  |
| 15/06/2023 | Company | 12/05/2023 | 14/06/2023 |  |  |
| 02/11/2023 | Both | 15/06/2023 | 01/11/2023 |  |  |
| …… |  |  |  |  |  |

* Refer to ship types specified in 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships (resolution MEPC.308(73))

** As defined in Section 3 of Guidelines for voluntary use of the ship energy efficiency operational indicator (EEOI) circulated by MEPC.1/Circ.684.

***

Refer to the Guidelines for the Authorization of organizations acting on behalf of the Administration, adopted by the Organization by resolution A.739(18), as amended by resolution MSC.208(81), and the Specifications on the Survey and Certification Functions of Recognized Organizations Acting on Behalf of the Administration, adopted by the Organization by resolution A.789(19), as may be amended by the Organization.

Refer to [2022 Guidelines for the development of a Ship Energy Efficiency Management Plan (SEEMP), adopted by resolution MEPC.XXX(XX)].

Refer to 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships (resolution MEPC.308(73)), as may be amended.

Refer to 2021 guidelines on operational carbon intensity indicators and the calculation methods (CII Guidelines, G1) adopted by resolution MEPC.336(76).

Refer to [Guidelines on correction factors and voyage adjustments for CII calculations (G5)]

Refer to 2021 guidelines on the operational carbon intensity rating of ships (CII Rating Guidelines, G4) adopted by resolution MEPC.339(76).

Refer to 2021 guidelines on operational carbon intensity indicators and the calculation methods (CII Guidelines, G1) adopted by resolution MEPC.336(76).

Refer to Guidelines for the verification and company audits by the administration of the Ship Energy Efficiency Management Plan (SEEMP) adopted by resolution MEPC. XXX(XX).

Also refer to Guidelines for the verification and company audits by the administration of the Ship Energy Efficiency Management Plan (SEEMP) adopted by resolution MEPC. XXX(XX).