TOR 2.4: the PSC procedures

Compiled comments at Round 4 and the Coordinators’ remark for Round 5

Q1.	Track-changes suggested by the Coordinators in appendixes 12 and 18 of the base document

| Appendix | Comment | Coordinators’ remark |
| --- | --- | --- |
|  | Support to the suggested documents by the Coordinator for Round 4 Canada, Chile, Finland, Norway, UK, IPTA |  |
| 12 | - Part B 33bis <Denmark> Is this requirement stated in the revised MARPOL Annex VI? <Italy> Statement of Compliance related to fuel oil consumption reporting and operational carbon intensity rating covering at least the last five years or, for new ships, the major numbers of years available (MARPOL Annex VI regulation 6)] <Russia> We do not agree that the proper way to reflect the requirement of reg.9.12 (having SoC for at least five previous years) is simply to indicate that the “SoC – fuel oil consumption and operational carbon intensity rating” should cover the previous 5 years. The first SoC with that new title will be issued in 2024 covering year 2023. Consequently in 2024 a ship to which CII is applicable should have two sets of SoCs: “SoC - fuel oil consumption and operational carbon intensity rating” for 2023 as well as “SoC – fuel oil consumption reporting” for the period before 2022 inclusive. The suggested amendment does not take it into account. In order to reflect in more appropriate way the situation to be faced at least during first years of the application of the CII framework, it is suggested to have throughout the PSC Procedures two separate entries – one for “SoC fuel oil consumption reporting” with a note that those documents should be for the period up to 2022 inclusive, another entry for “SoC - fuel oil consumption and operational carbon intensity rating”, stating that it is from 2023 onwards and that all together both documents should be for at least five previous years. <WSC> it is suggested to add ‘as applicable’ in order to consider that until 2028 vessels won’t have five years of history, similarly for newbuildings or vessels having had conversions. | Please see Regulation 9.12. WSC commented from the same point of view, and for the sake of simplicity, we will adopt WSC's proposal. Same as above. Reflected. |
| 18 | - 2.4bis.3, 2.4bis.4 and 2.7.2.9 <China> We cannot agree to include 2.7.2.9 “the implementation plan required in regulation VI/26.3.1.3 and/or the plan of corrective actions for a ships rated as D for 3 consecutive years or rated as E are not implemented by the ship as planned within at the time of the inspection.” Not implementing the plan should not be a detainable deficiency. The reasons are as follows: 1. The implementation of plan involves a long list of items. Some items may be required at a time beyond the time of the inspection, or some items may go beyond the responsibility of PSCO inspections. 2. “Not implemented by the ship” is a vague expression, does it mean that a single item in the plan not implemented will also entail a detention? If it is so, this is not a fair requirement. Therefore we suggest to delete 2.7.2.9. <Greece> Coordinators’ reply to Greece 3rd round comments is understood. However, we wish to reiterate Greece’s concern by using some examples. Implementation of SEEMP and its corrective actions is not a static, nor a checklist-type, procedure. It is a dynamic process which may allow laxer or stricter actions than envisaged, depending on the circumstances. Thus, inspector’s impression “at the time of inspection” may not always be relevant. Example 1: In the beginning of a year, a ship may have the leeway to proceed at higher speeds with the intention to substantially limit its operational speed in the subsequent voyages to comply with required CII or its corrective actions. A PSCO inspection in the first months of a year cannot possibly evaluate properly the “current” state of SEEMP implementation (whether ship action in the first few months of the year constitute a “proper” SEEMP implementation). Example 2: Despite best efforts, due to ship’s trading area (involving e.g., short ballast voyages, winter zone bad weather, etc.), a ship may not be on target to achieve its corrective action. The managing company, to assist in improving ship’s CII, may decide to send the ship to a different trading area for the remainder of the year. In both examples, the ship risks being detained when PSCO “in his/her view” determines that a vessel is not implementing properly SEEMP “at the time of inspection”. Thus if 2.4bis.3 and 2.4bis.4 are to be retained, they should at least be qualified with the note “Note: proper implementation of SEEMP and its possible corrective actions is a dynamic process, which may require continuous adjustment, the results of which may not be apparent in the first year of implementation” For the same reasons, we feel that 2.7.2.9 should be preferably deleted. If not, same qualification ‘Note’ should be applied. <Japan> As regulation 10 of the MARPOL Annex VI has already been and amended to include PSC on operational requirements to SEEMP, therefore it is not appropriate to delete this sentence as the coordinator commented. However, the wording of "duly implemented by the ship" is vague and interpretation may varies among PSCOs as Greece pointed out. Therefore, it is necessary to clarify what PSCO should/can check during the initial inspections. We may suggest the wording of "In case where the ship is rated as D for 3 consecutive years or rated as E, PSCO may verify that the SEEMP includes a plan for corrective action and the plan is duly approved by the Administration." as possible alternative to this sentence. - 2.1.2.17 <Russia> It is understood that the proposal in 2.1.2.17 refers to the text in Reg.26, however we question how and on what grounds a PSC inspector could examine the methodologies in SEEMP? Apart from the issue of competence it goes beyond the authority of inspector as SEEMP and the methodologies mentioned therein are subject to Administration’s approval. Furthermore, it could be simply impossible to examine the methodology of CII calculation because officers onboard may not be involved in that, it would be done by the Company on shore. The only element that inspector could examine in that respect is documentation, if any, required in accordance with methodologies. Also the use of Future Tense raises questions. In Reg.26 it is used correctly as it reflects what shall be included in the SEEMP. However when it comes to the PSC inspection on board, the SEEMP has already been produced and the methodology implemented. So the use of Future Tense is not appropriate. It is proposed to revise as follows: 2.1.2.17 the Ship Energy Efficiency Management Plan (SEEMP) including, where applicable, the documentation required in accordance with the methodology that will be used to collect the data required by regulation 27 of the Annex and the methodology that will be used to calculate the ship’s attained annual operational CII required by regulation 28 of the Annex (with a plan of corrective actions included - for a ship rated as D for 3 consecutive years or rated as E) according to regulation 28 of the Annex and the associated Confirmations of Compliance - 2.1.2.18 and 2.7.2.1ter <Denmark> Is this requirement stated in the revised MARPOL Annex VI? <Italy> for the year 2023 and onwards that the ship has, no later than 1 June of each following year, the required Statement(s) of Compliance – Fuel Oil Consumption Reporting and operational carbon intensity rating covering at least the last five years or, for new ships, the major numbers of years available. <WSC> As for paragraph 33bis in appendix 12, ‘as applicable’ should be added. | Noted. As in coordinator’s remarks after Round1 and Round 3, the element of 2.4bis.3 is provided by regulation 10.6 of MARPOL VI. And 2.4bis4 is a direct consequence thereof as it highlights the requirements of regulation 28 related to the plan of corrective actions. Therefore, it is difficult to justify the deletion of 2.4bis.3. and 2.4bis.4. Noted. Implementing a plan and having a plan approved by the Administration do not necessarily mean the same thing. Coordinator would like to respect the outcome of III7 and to keep the draft amendment text as circulated in Round 4. Please see Regulation 9.12. WSC commented from the same point of view, and for the sake of simplicity, we will adopt WSC's proposal. Reflected. |