TOR 3: Development of Draft Guidelines on correction factors for certain ship types, operational profiles and/or voyages for the CII Calculations (G5)

Consolidated Answers for Round 4

Contents

A.	cgDIST for Ro-Ro Cargo Ships	4

B.	Reference Line Split for Ro-Ro Cargo Ships	8

C.	Split of reference line for Vehicle Carriers (e.g. PCTC)	13

D.	HSC	19

E.	Change of reference line for Combination Carriers	23

F.	CFnorm Normalisation of Rating boundaries for Bulk Carriers and Tankers	26

G.	FCvoyage Voyage Adjustment for Adverse Weather	33

H.	FCvoyage Voyage Adjustment for Sailing in Ice	43

I.	CFshort Standalone Correction Factor for Short Voyages of 72 hours or less	51

J.	FCelectrical Correction Factor for Reefer Electrical Consumption	63

K.	FCelectrical Correction Factor for Cargo Handling Electrical Consumption	74

L.	FCelectrical Correction Factor for Cargo Maintenance Electrical Consumption	82

M.	FCBoiler Correction Factor for Boiler Consumption	89

N.	FCBOG Correction Factor for Boil Off Gas	97

O.	FCBOG,Nitrogen Correction Factor for Boil Off Gas	106

P.	CFPT Port Time Correction Factor for Cruise Passenger Ships	110

Q.	CFSTS Correction Factor for Tankers with STS voyages	120

R.	CFShuttle Tankers Correction Factor for Shuttle Tankers	127

S.	fc EEDI Capacity Correction Factor for Chemical Tankers	133

T.	fiVSE EEDI Capacity Correction Factor for Voluntary Structural Enhancement	138

U.	FCextra weight Additional Fuel Consumption for increased displacement	142

V.	CFsludge Correction Factor for Sludge	146

W.	fladen Correction factor for sailing in laden conditions	151

Introduction

In Round 4 we will evaluate the proposals and information which have been submitted in Round 3.

Note that 3 further correction factors have been submitted to MEPC 77 – that is MEPC 77/7/9, 77/7/13 and 77/7/14. Subject to the agreement of MEPC 77, the submitters of these proposals will be invited to reformat their submissions according to the Round 3 ToR 3 question and answer template and submit to the CG by 29 November 2021. We will leave space in the Round 4 question and answer sheet for the CG to be able to additionally evaluate these 3 proposals.

As such, refinements and additional supporting information for correction factors and voyage adjustments included in the Round 4 question and answer sheet below may also be provided during Round 4 if submitted by 29 November 2021.

In the 3rd round, a number of correction factors and voyage exclusions were either not included in CG responses or the CG responses did not provide sufficient evidence. These have been removed from further consideration.

These are:

Voyage adjustments

- Ships which remain stationary for a period exceeding [30 days] such as but not limited to warm layup, drydock, operation as fuel storage unit, stop for technical repairs

- For ships where waiting time at port or anchorage exceeds [24] hours,

- For cruise passenger ships which spend greater than [20%] of a calendar year in port, at anchor or drifting (now replaced by CFPT)

Correction factors

- EEDI correction factors fcLNG, fcRoPax, fiCSR and cubic capacity correction factor for bulk carriers designed to carry light cargo

Note that the questions in this questionnaire are primarily designed to evaluate correction factors and voyage adjustments, i.e. the development of G5 guidelines as per the mandate of the CG. For proposals that target changes to G2 and G4, please answer to the extent possible.

Note that in Round 5, the discussion on verification of correction factors to be incorporated into Resolution MEPC.292(71) 2017 Guidelines for administration verification of ship fuel oil consumption data will be addressed under ToR 2.1.

As per the outcome of ISWG-GHG 10 (see paragraph 15 of ISWG-GHG 10 WP.1), note that subject to the agreement of the Committee on correction factors and voyage adjustments, such corrected CII values will also be the values used to derive ratings.

It should be noted that while corrected CII values will be used for compliance, calculation of the operational carbon intensity of international shipping and the achieved carbon intensity reduction in international shipping is governed by uncorrected values of fuel consumption, distance and capacity, as explained in paragraph 3 of the G3 Guidelines.

In the following pages is a document table listing all the relevant proposals for correction factors, voyage adjustments and changes to G2 and G4. The Coordinators have aggregated the proposals and supporting documents to help the CG carry out the evaluation, however the table below may be used as an overview, and also if the original CG submissions are to be consulted.

# cgDIST for Ro-Ro Cargo Ships

Ro-Ro Cargo ships shall calculate their annual attained CII based on GT (cgDIST) instead of AER.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Canada, Chile, China, Cyprus, Denmark, Finland, France, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERFERRY, WSC No | ☐ Yes ☐ No |
| Comments <Denmark> Denmark supports this proposal is further considered, with a view to change the CII indicator for RoRo Cargo vessels. <Finland> This matter is described in the document ISWG-GHG 8/3 and in the replies of Denmark and Interferry in Round 3 of the CG <Greece> It is more appropriate to use GT instead of DWT for this type of ships. <Italy> This solution would be in line with the CII assessment proposed for other type of ships and in line with EEDI and EEXI formulations. <Spain> We support the proposal to use cgDIST as a metric for all ro-ro cargo ships. <Sweden> Sweden supports the correlation to GT, gross tonnage seems better than deadweight. Why not use net tonnage (NT) since this better reflects the cargo capacity? <EC> Issue already established at previous CG Better statistical fit, distributing the rate of improvement more evenly across the size range <ICS> ICS fully supports the position of Denmark and Interferry with respect to RoRo cargo ships, i.e. like other categories of RoRo vessel, they are volume carriers and should receive similar treatment under CII. Hence we agree it is appropriate to utilize the cgDIST metric instead Of AER. <WSC> We support the proposal to use cgDIST as a metric for all ro-ro cargo ships. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Cyprus, Denmark, Finland, France, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERFERRY, WSC 2026 - Canada | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Cyprus, Denmark, Finland, France, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERFERRY, WSC No | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> It appears that cgDIST has better statistical fit, distributing the rate of improvement more evenly across the size range |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Cyprus, Denmark, France, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERFERRY, WSC No | ☐ Yes ☐ No |
| Comments <Denmark> Changing from AER to cgDIST for RORO vessel will provide a more fair and evenly distributed effort across the global fleet range. <Spain> Applying the same metric to all ro-ro ship types will provide consistency and avoid potential distorted perceptions if different ro-ro segments are assessed using different, non-comparable, metrics. <INTERFERRY> Under AER, the required CII increases relatively with ship size, i.e. the opposite of economy of scale. Under AER, the ranking of the ro-ro cargo ships deviate significantly from the intended 15/20/30/20/15 rating bands <WSC> Applying the same metric to all ro-ro ship types will provide consistency and avoid potential distorted perceptions if different ro-ro segments are assessed using different, non-comparable, metrics. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Cyprus, Denmark, France, Greece, Italy, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK US, EC, ICS, INTERFERRY, WSC No | ☐ Yes ☐ No |
| If not, please explain your answer <Liberia> The arguments provided seems acceptable, however using GT could give incentives for Ro-Ro Cargo ships to have the upper cargo deck space open rather than partly or fully enclosed to reduce GT. This will provide less protection to the cargo and thus reduce the safety level at the cost of improved environmental performance. This may need further consideration. <Spain> We cannot identify any perverse incentives associated with the change <WSC> We cannot identify any perverse incentives associated with the change |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - China Accurately Correct – Chile, Greece, Liberia, Norway, Russian Federation, Sweden, UK, ICS Over Correct | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <US> This proposal is not for correction factors or voyage adjustments, but rather to use a more appropriate metric for ro-ro cargo ships |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Greece, Liberia, Russian Federation, UK, ICS, INTERFERRY Further development needed No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <US> This proposal is not for correction factors or voyage adjustments, but rather to use a more appropriate metric for ro-ro cargo ships |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Greece, Liberia, Russian Federation, Sweden, UK, ICS, INTERFERRY No | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# Reference Line Split for Ro-Ro Cargo Ships

Split of cgDIST reference line at 25,000 dwt for Ro-Ro Cargo Ships.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Canada, Chile, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, UK, EC, ICS, INTERFERRY, WSC No – Japan, Norway, Sweden, US | ☐ Yes ☐ No |
| Comments <Denmark> Denmark supports that this proposal could be further considered <Finland> This matter is described in the document ISWG-GHG 8/3 <Greece> It is important to split the reference lines for Ro-Ro cargo ships <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Norway> Not enough information to evaluate the proposal. In general, we support the proposal given that the reference line is continuous for all size segments. We would like to see the proposed reference lines using the 2019 DCS data before deciding. <Sweden> Sweden does not support a split in the reference line. Have net tonnage (NT), representing useful capacity with regard to transport capacity of cargo, been evaluated as possible/ better match to avoid the split? <EC> Further discussion on how to solve the issue is needed. <ICS> Although section B of this questionnaire refers to “Split of cgDIST reference line at 25,000 dwt for Ro-Ro Cargo Ships” we can find no corresponding proposal within the round 4 documentation. Hence, we assume this will be submitted by Interferry at a later stage. Nevertheless, based on Interferry’s earlier round 3 submission, we agree there are two distinct vessel types falling under the Ro Ro cargo ship reference line, i.e. the smaller classic RoRo cargo ships and the larger container RoRo ships. As these types vary in design, and in the market segments they serve, it is understandable that application of the existing single reference line to these two groups is inadequate. Hence in principle we agree that the existing reference line needs improvement, and will await Interferry’s confirmation of their proposed solution. <INTERFERRY> INTERFERRY was the organization originally cautioning that there was an inherent issue with the ro-ro cargo dataset and proposed a split at 25,000 DWT to distinguish large, deep-sea con-ro ships from conventional short-sea ro-ro ships. Equipped with new information, we believe there is a better way to re-define the reference line, avoiding any over-laps. INTERFERRY will present this solution to MEPC78 and kindly ask the CG to keep the ro-ro vs. con-ro issue open for now. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Finland, Greece, Liberia, The Netherlands, Russian Federation, Sweden, UK, ICS, INTERFERRY, WSC 2026 – Canada, Cyprus, Denmark, Italy, Spain, EC | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Finland, Greece, Liberia, The Netherlands, Russian Federation, Spain, UK, INTERFERRY, WSC No – Cyprus, Denmark, Italy, Sweden, US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Denmark> The majority of RoRo vessels operate as part of short-sea shipping, while a smaller amount of larger RoRo vessels operate as ocean going with lower port calls. Denmark agree that the reference line for larger Con-Ro and smaller RoRo should be split, in order to better determine the efficiency of same. <Sweden> Sweden does not support a split in the reference line. Have net tonnage (NT), representing useful capacity with regard to transport capacity of cargo, been evaluated as possible/ better match to avoid the split? <US> We do not support a non-continuous reference line. <EC> While the situation is improved, the split with significantly different regression lines may artificially constrain ro-ros to be below 25,000 dwt and adversely affect existing ro-ros above 25,000 dwt <ICS> In principle we agree that the existing reference line needs improvement, and will await Interferry’s confirmation of their proposed solution. <INTERFERRY> The ro-ro cargo dataset needs to be reviewed so that a fair and robust reference line can be implemented for all ships affected by it, but not necessarily through the 25,000 DWT split proposed by ourselves (INTERFERRY). |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERFERRY, WSC No – Chile, US | ☐ Yes ☐ No |
| Comments <INTERFERRY> It is evident that conventional ro-ro cargo ships and small con-ro ships would be penalized by being mixed up with large con-ro ships. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Greece, Liberia, The Netherlands, Russian Federation, Spain, Sweden, UK, WSC No – Cyprus, Denmark, Italy, US, EC | ☐ Yes ☐ No |
| If not, please explain your answer <Italy> While the situation is improved, the split with significantly different regression lines may artificially constrain roros to be below 25,000 dwt and adversely affect existing roros above 25,000 dwt <US> The use of a non-continuous reference line introduces a step change in the emission standard based on DWT, which could have perverse incentives (e.g., encouraging designs that keeps ships to the “beneficial” side of the reference line). The purpose of the correlating parameter (x axis) is to ensure that larger and smaller ships are treated fairly. <ICS> In principle we agree that the existing reference line needs improvement, and will await Interferry’s confirmation of their proposed solution. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Greece, The Netherlands, Russian Federation, UK Over Correct | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Sweden> Sweden does not support a split in the reference line. Have net tonnage (NT), representing useful capacity with regard to transport capacity of cargo, been evaluated as possible/ better match to avoid the split? It is difficult to evaluate the deviation split/ no split to other possible solutions. <ICS> In principle we agree that the existing reference line needs improvement, and will await Interferry’s confirmation of their proposed solution. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Greece, Russian Federation, UK Further Development Needed - INTERFERRY No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <ICS> In principle we agree that the existing reference line needs improvement, and will await Interferry’s confirmation of their proposed solution. <INTERFERRY> We believe that additional analysis is necessary |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Greece, Russian Federation, UK No - US | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <US> The data presented in this paper do not support the need for a non-continuous reference line for roro cargo ships. <ICS> In principle we agree that the existing reference line needs improvement, and will await Interferry’s confirmation of their proposed solution. <INTERFERRY> It would be possible to make the verification based on the 25,000 DWT split, but INTERFERRY suggests to also consider an alternative proposal, which will be submitted to MEPC78. |  |

# Split of reference line for Vehicle Carriers (e.g. PCTC)

Split of the Vehicle Carrier reference line into 3 sections.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Singapore, Spain, Sweden, UK, US, EC, ICS, INTERFERRY, WSC No - Japan | ☐ Yes ☐ No |
| Comments <Brazil> We noticed that there are some vessels that are not considered on the regressions presented on figure 5 taking figure 6 as a reference. It seems that the vessels highlighted in green above indicate a trend steeper than the proposed green curve on figure 5 (below for reference). We suggest to use the DCS database without excluding any vessels that will use the reference line. <Denmark> This could be further considered <Finland> This matter is described in the document ISWG-GHG 8/3/2 and in the replies of WSC in Round 3 <Greece> This is important for the different size car carriers as small car carriers are adversely penalized. <Italy> WSC has provided detailed information and analysis which provides all necessary information to support the adjustment. We would further note that other ship types have split reference lines. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Spain> WSC has provided detailed information and analysis which provides all necessary information to support the adjustment. We would further note that other ship types have split reference lines. <Sweden> Sweden supports development of an appropriate reference line for vehicle carriers to avoid gap in the baseline and avoid the development of “rule” ships that would fall around the gap. A proposal to develop a reference line that better reflects characteristics of the fleet is therefore supported. <EC> Clear split demonstrated by data <WSC> WSC has provided detailed information and analysis which provides all necessary information to support the adjustment. We would further note that other ship types have split reference lines. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Singapore, Spain, Sweden, UK, US, EC, ICS, INTERFERRY, WSC 2026 - Canada | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Singapore, Spain, Sweden, UK, EC, ICS, INTERFERRY, WSC No – Japan, Norway, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Japan> The apparent CII rating may change, but the relative rating of the same ship type remains the same. <Norway> An adjustment to the reference line seems to be a good solution, but the proposed reference line appears to over-correct. <US> It makes sense for the 2019 Reference line to fit the data, including at the extremes. In cases where the default equation cannot achieve this, it seems reasonable to use multiple curves, as long as they are continuous (no step changes). If this were done, it would need to be based on DCS data as recommended by this paper. The United States believes that more thought needs to be put into the use of a nearly horizontal line for the larger vessels. Ships with larger GT have lower CII scores, by definition, and this should be reflected in the curve shape with a declining CII for increasing ship size. Additional thought needs to be put into where 2c vs 2b vessels fit into this equation |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Singapore, Spain, Sweden, UK, EC, ICS, INTERFERRY, WSC No – Japan, US | ☐ Yes ☐ No |
| Comments <Greece> Small car carriers engaged in short voyages are severely penalized. <Japan> The apparent CII rating may change, but the relative rating of the same ship type remains the same. <Sweden> Sweden sees the risk that some vehicle carriers will have inappropriate and unjustifiably poor CII values and ratings since the current reference line does not take into account the characteristics of the fleet. This could result in very efficient ships receiving low ratings. Sweden supports the proposal to avoid gap in the baseline and avoid “rule” ships around the gap. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Singapore, Spain, Sweden, UK, US, EC, ICS, INTERFERRY, WSC No | ☐ Yes ☐ No |
| If not, please explain your answer <US> The continuous reference line would prevent ships near the change in the reference line equation from being treated differently. <EC> Demonstrated perverse incentive for most efficient vessels, result needs to be confirmed by IMO secretariat with DCS data |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Greece, Liberia, Singapore, Spain, Sweden, UK, ICS, WSC Over Correct – Norway | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Greece> To improve accuracy, the IMO DCS data could be used <Norway> The proposed adjusted reference line seems to result in an over-correction, with a lower proportion of ships being rated E and D and more being rated A. A: 12 % before adjustment and 19% after adjustment D: 21% before adjustment and 17% after E: 14 % before adjustment and only 10% after adjustment According to the rating guidelines, the ratings are designed so that 30% of ships in 2019 achieve C, 20% achieve B or D and 15% achieve A or E. Some additional adjustment of the reference lines might be needed to get a more accurate correction and distribution of ratings. The distribution of ratings before adjustment are more aligned with the rating guidelines, but we agree that some adjustment is needed as the current reference line does seem to penalize the smaller and larger ships in this category. We also do not understand why ships below 3000 GT as adjusted as included in the below table as only ships above 5000 GT are subject to the CII. A proposal was also submitted in round 4 to apply the correction factor for short voyages (CFshort ) to the smaller ships in this ship category. The two proposals need be considered simultaneously to avoid correcting twice. <Sweden> The proposal does not suggest to introduce correction factors, only an adjustment to the reference line. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Greece, Liberia, UK, ICS Further Development Needed No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Greece, Liberia, UK, US, ICS No | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# HSC

New reference line for HSC Ro-ro passenger ships

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Australia, Canada, Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Spain, Sweden, UK, US, EC, ICS, INTERFERRY No - Japan | ☐ Yes ☐ No |
| Comments <Australia> Categorising HSC and conventional RoPax together with a single reference will result in an unfair advantage for certain RoPax types and sizes. Specifically, HSC would be responsible for RoPax reference line increases at lower gross tonnages, which gives lower gross tonnage conventional RoPax an unfair advantage, as they will require less stringent CII reductions to achieve a stronger rating (A-C). The opposite will occur for the higher gross tonnage conventional ships. <Denmark> Denmark supports that new reference lines for HSC’s are developed <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Spain> WE CONCUR THAT THIS SHIP TYPE NEEDS TO BE CONSIDERED SEPARATELY TO ROPAX AND HSC ROROS. HSC SHOULD BE REMOVED FROM THE CURRENT REFERENCE LINES SO THAT A PROPER CALCULATION CAN BE MADE OF ROROS AND ROPAX <Sweden> HSC probably contribute to greenhouse gas emissions to a not neglible part. <EC> Offset reference line as proposed by Australia. We believe that it is a more proportionate solution than to create a separate ship category. <ICS> ICS fully supports Interferry’s position, i.e. high speed craft are very different to conventional RoPax vessels and should be treated differently under CII. It is clear from the Australia/Interferry submission that grouping these disparate groups together may unfairly penalize high speed craft and we fear that in that scenario there is a possibility that modal shift may occur to air and road transportation, with consequential increases in overall emissions. <INTERFERRY> HSC must be separated from conventional ro-pax in order to have a fair CII application. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Australia, Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Spain, UK, US, EC, ICS, INTERFERRY 2026 – Canada, Sweden | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Australia, Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Spain, UK, US, EC, ICS, INTERFERRY No - Sweden | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Australia> Australia considers this issue is adequately solved by implementing an offset reference line for HSC, as suggested in the additional supporting document Australia submitted to Round 4. This solution is able to achieve an equitable CII rating assignment across both HSC and conventional types without needing to wait for the collection of sufficient data to develop an accurate reference line for HSC. The benefit of this approach is that it would provide certainty to industry and ensure that HSC and conventional RoPax ships have consistent reference lines and fair required annual operational CII’s to meet from the date of effect of the new measure. <Norway> We support the proposal in the document by Australia to apply a separate HSC line/offset line. This enables solving the issue now instead of waiting for changes to MARPOL. We would propose to include the reference line parameters in the main table in G2, and not as a footnote, and with a definition of HSC in section 2. <Sweden> Correct baseline must be developed and HSC as a separate ship category since HSC probably contribute to greenhouse gas emissions to a not neglible part. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Australia, Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Spain, Sweden, UK, US, EC, ICS, INTERFERRY No – | ☐ Yes ☐ No |
| Comments <Australia> As per Australia’s response to Round 3, all 392 RoPax identified in the IMO DCS data are affected by categorising HSC and conventional RoPax together with a single reference line. Without mitigations, there will be significant negative impacts on both HSC and conventional RoPax types. For example, all HSC will be assigned a CII rating of “E”, with one exception, which would be assigned a rating of “D”, purely as a result of their distinct operation and differences in design and construction. |  |
| 5. Are the perverse incentives addressed? Yes – Australia, Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Spain, Sweden, UK, US, EC, ICS, INTERFERRY No | ☐ Yes ☐ No |
| If not, please explain your answer |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Australia, Chile, Greece, Liberia, Norway, Spain, UK, ICS Over correct | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Australia> The adequacy and accuracy of an offset reference line for HSC would be reviewed as part of the 2026 review. In the interim, HSC ships would continue to collect the data required to establish a new category of their own in case future reviews determine this would be the better solution. <Sweden> Sweden agrees that baseline must be developed for HSC as a separate ship category since HSC probably contribute to greenhouse gas emissions to a not neglible part. <INTERFERRY> By breaking out HSC, the reference lines for both conventional ro-pax and for HSC would by default be more accurate than what is currently the case. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Greece, Liberia, The Netherlands, Sweden, UK, ICS Further development needed No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Greece, Liberia, The Netherlands, UK, US, ICS, INTERFERRY No - Sweden | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <Sweden> The baseline must be devolped for HSC as a separate ship category since HSC probably contribute to greenhouse gas emissions to a not neglible part. |  |

# Change of reference line for Combination Carriers

Change of reference line parameters for Combination Carriers

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Canada, Chile, Denmark, Italy, Japan, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO No | ☐ Yes ☐ No |
| Comments <Denmark> Denmark can support to look further into this issue <Japan> Regarding the change of the reference line for Combination Carriers, Japan does not see any rationale for making it the same as that for bulk carriers. If there is rationality in the change after further data analysis, Japan can accept this change. <The Netherlands> Depends on the information that will be provided <EC> Adjust the slope to that of bulk carriers (20-30 ships affected) <ICS> Combination carriers are flexible in their operation, enabling the carriage of both solid and liquid cargoes. Due to this flexibility they can achieve a lower proportion of ballast voyages and have a relatively high efficiency in terms of emissions per cargo tonne-mile. Hence we agree with Norway that this type of vessel should not be penalized due to the identified anomaly. <INTERTANKO> We agree with Norway that combination carriers have to be carefully addressed. They are built to swap between wet and dry cargoes. This results in a higher degree of laden voyages which means higher transport efficiency. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO 2026 - Canada | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO No | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO No | ☐ Yes ☐ No |
| Comments <US> We note that this is not a correction factor, but a proposal to make a correction to the reference line. The problem seems to be driven by the small number of data points for combination carriers and the solution seems reasonable, based on the information available. <EC> Demonstrated perverse incentive for most efficient vessels |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO No | ☐ Yes ☐ No |
| If not, please explain your answer |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - Japan Accurately Correct – Chile, Liberia, Norway, Russian Federation, Sweden, UK, US, ICS, INTERTANKO Over Correct | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Japan> Regarding the change of the reference line for Combination Carriers, Japan does not see any rationale for making it the same as that for bulk carriers. <Sweden> Sweden supports presented baseline |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Liberia, Russian Federation, Sweden, UK, ICS Further Development Needed, INTERTANKO No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Liberia, Russian Federation, Sweden, UK, ICS, INTERTANKO No | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# CFnorm Normalisation of Rating boundaries for Bulk Carriers and Tankers

Proposal for normalisation of rating boundaries for bulk carriers and tankers.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Greece, Liberia, The Netherlands, Norway, Spain, Sweden, UK, ICS, INTERCARGO, INTERTANKO No – Cyprus, Denmark, France, Italy, Japan, US, EC, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Denmark does not support this proposal <France> The proposal is clear <Greece> Normalization (correction) factors need to be applied to achieve a fair distribution on a size segment basis per ship type. The actual width of the band for different ship sizes is wider than others in some cases, thus confirming the theory of size-dependency and the need for correction factors. The proposed methodology could be followed to normalize the rating boundaries of other ship types. <Italy> Broad measure (other ship types may be affected), subject to re-negotiation of boundaries and risk of creating new perverse incentive. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. - It is natural that energy-saving performance improves for ships with larger tonnage, and it is meaningless to compare the distribution of ships by rating at one tonnage with that at another tonnage. <Norway> We see some merits in this, but we think more analysis is needed to understand the causes. For example, some size segments could consist of new and more efficient tonnage that result in the different distribution of ratings. We think this could be addressed during the review in 2026. <Spain> IT MERITS CONSIDERATION BUT THIS IS A POLICY DECISION THAT SHOULD HAVE BEEN CONSIDERED BEFORE. NOTE OUR COMMENTS AT THE END OF THIS DOCUMENT <Sweden> Sweden supports the proposal, since we agree that it is not logical that rating A-E is dependent on DWT for tankers and bulk carriers. Instead of using split reference lines for different sizes, which we do not support, correction factors could be developed. <US> Information provided is insufficient to justify any correction factor <EC> Broad measure (other ship types may be affected), subject to re-negotiation of boundaries and risk of creating new perverse incentive. <ICS> Our understanding is that the proposed correction methodology is an alternative to the split-reference line methodology proposed by other members of the correspondence group for other ship types. Whilst we acknowledge the need to correct the anomalies identified by Greece we would prefer if possible to maintain a consistent approach across ship types, i.e. our preference is to either adopt Greece’s approach, or the split refence line approach but not both. We also note and agree Greece’s response to question 1c, i.e., “The analysis has been focused on bulk carriers and tankers of all size segments. Other ship types may also be affected.” Hence we recommend that consideration is also given to extending this proposal to other types of vessel. <INTERCARGO> Without the correction factor applied there is an unfair distribution across the ratings, adversely affecting vessels of certain size ranges. <INTERTANKO> We suggest the proposed arrangement needs further checks. Two issues: - proposed correction factors for re-alignment for the data collected in one year would not give same results for another year of data collection. - in the analysis presented, large tankers will have to correct upwards their calculated CII; those in cat. B with 5.5% and those in cat. C with 9%; for sure these tankers will become C, D or and even E with such a change One needs to better understand this proposal comparing data over several years. <IPTA> IPTA has no comments on the methodology, but we support that a correction is needed. <Pacific Environment> As a general comment, we remain opposed to all correction factors or exemptions. It is important to note that each additional correction factor, exemption, or adjustment inhibits the ability of the CII to accurately score a ship's performance in a given year and to reduce its carbon intensity. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Greece, Liberia, Spain, Sweden, UK, ICS, INTERCARGO, INTERTANKO 2026 – Brazil, Canada, China, The Netherlands, Norway <The Netherlands> We should not reopen now the debate on rating boundaries. | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Greece, Liberia, UK, ICS, INTERCARGO No – Brazil, China, Cyprus, Denmark, France, Italy, The Netherlands, Spain, Sweden, US, EC, INTERTANKO | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Brazil> As stated in document MEPC 76/7/55, a detailed evaluation should be conducted to assess the correct correction factor. We also believe that developing specific rating boundaries for each size baskets could create distortions. For example, a larger ship with lower CII could have a lower rating than a smaller vessel with higher CII. <China> - It is noted that the sample used for this correction factor is different from the sample used for CII guidelines development, according to document MEPC 76-INF.10. - This correction factor will severely penalize tankers with more than 125,000 DWT. However, large tankers have a better efficiency than small and medium tankers. This will incentivise operators to replace tankers of better propulsion performance with inefficient ships, which leads to more GHG emissions and undermines IMO's contribution to GHG emissions reduction. We suggest that correction factor may be 1 while applying to tankers with more than 125,000 DWT. <France> We understand the concerns, but we do not want to re-open now the discussion on the determination of the rating boundaries. This global measure cannot take into account all specific situations, otherwise it would risk becoming meaningless. Every ship will have to adapt and make efforts to comply whater its initial situation and its own technical and operational characteristics. <Sweden> Correction factors and calculations for the attained index per ship need to be developed. <US> There is insufficient justification/explanation provided of why a correction factor is needed other than to help the data meet a line. <EC> Subject to the assessment of impact on the integrity of the CII framework (in particular with reference to the ‘size bins’) <INTERTANKO> Segment; A; B; C; D; E; average. 5-10k; 1.02; 1.03; 1; 0.99; 0.9; 0.988. 10-25k; 1; 0.99; 0.99; 0.98; 0.94; 0.98. 25-40k; 1; 0.99; 0.99; 0.98; 0.94; 0.98. 40-55k; 1.03; 1.03; 1; 1; 0.99; 1.01. 55-85k; 1; 0.99; 0.94; 0.92; 0.91; 0.952. 85-125k; 1; 0.95; 0.95; 0.92; 0.9; 0.944. 125-200k; 1.04; 1.03; 1.02; 1.02; 1; 1.022. 200-300k; 1.04; 1.04; 1.06; 1.09; 1; 1.046. >320k; 1.02; 1.055; 1.09; 1.1; 1; 1.053. average; 1.02; 1.01; 1.00; 1.00; 0.95. We see that the average changes per category are insignificant except for category E. However, we also note significant average proposed changes for each segment. This begs two questions: - would results be different if the size bins are changed - is the population in each size bin equally represented between the min. and max. size borders in number and relevant voyages? It is without any doubt that larger tankers ARE more efficient in transportation (fuel/t-m), and they do have LONGER voyages than other tanker size categories. So, the proposed re-alignments may penalize large tankers’ CII value by an increase of 5.5%, 9% and 10%. We question it. |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Brazil, Chile, China, Greece, Liberia, Sweden, UK, ICS, INTERCARGO No – Cyprus, Denmark, France, Italy, The Netherlands, Spain, US, EC, INTERTANKO | ☐ Yes ☐ No |
| Comments <Greece> The analysis indicated that some size segments are penalized while others are favoured. In this respect, correction factors have been produced which once applied on the ship’s attained CII, a fair distribution is derived on fleet level and per size segmentation. <INTERTANKO> INTERTANKO has separate proposals to consider DP Shuttle Tankers and tankers engaged in STS operations. Due to their operations, such tankers do have significant higher CII values that need to be corrected. We are concerned that these proposed corrections with a pure mathematical rational may impact on the other correction factors based on a rational linked to the operation of the ship. Therefore, this proposal needs to be carefully addressed. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Greece, Liberia, Sweden, UK, ICS, INTERCARGO No – Brazil, China, Cyprus, Denmark, Italy, The Netherlands, Spain, US, EC, INTERTANKO | ☐ Yes ☐ No |
| If not, please explain your answer <China> This correction factor will severely penalize tankers with more than 125,000 DWT. However, large tankers have a better efficiency than small and medium tankers. This will incentivise operators to replace tankers of better propulsion performance with inefficient ships, which leads to more GHG emissions and undermines IMO's contribution to GHG emissions reduction. We suggest that correction factor may be 1 while applying tankers with more than 125,000 DWT. <France> In our view, the absence of this correction factor would not generate any perverse incentive. In contrary, the adjustment proposed would generate a risk to create new perverse incentives hard to predict. <Greece> If the measure is not implemented, penalized ships will need to reduce further their speed to compensate for the bad rating. Even new efficient ships falling in the penalized categories may need to reduce their speed. <Italy> Subject to the assessment of impact on the integrity of the CII framework (in particular with reference to the size bins). <US> Adding the correction factors proposed in this paper would overly complicate the standards and create a discontinuous line, leading to equity issues. Why should the standard for a 64,999 DTW ship be vastly different than that for at 65,001 DWT tanker? <INTERTANKO> Please consider difference in values of correction factors if a tanker is 54,999 DWT or 55,001 DWT. Same as being 124,999 DWT or 125,001 DWT. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Greece, Liberia, Sweden, UK, ICS, INTERCARGO Over Correct – Brazil, China, INTERTANKO | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <China> For tankers with more than 125,000 DWT, this correction factor is over correct. <Greece> The proposed normalisation factor is based on analysis carried out using the 2019 IMO DCS data from IMO DCS database for: • 10586 bulk carriers • 7324 tankers A large number of data have been used to reach a high level of accuracy. <Sweden> Correction factors and calculations for attained index per ship need to be developed. The proposal is supported in principle. <INTERTANKO> Could do all together. See correction factors values around some size bin borders. The large tankers would be seriously penalised for sure. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Greece, Liberia, Sweden, UK, ICS, INTERCARGO Further Development Needed - INTERTANKO No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <INTERTANKO> Apply the same analysis to 2020, 2021 and 2022 data to check consistency and accuracy. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Greece, Liberia, Sweden, UK, ICS, INTERCARGO No - INTERTANKO | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <Greece> The application of correction factors for normalizing CII rating boundaries per size segments is an easy straight forward process. No additional data or information is needed. Once the attained CII has been calculated, the specific normalisation factors (CFnorm) shall be applied on the attained CII based on ship’s rating and size segmentation for tankers and bulkers. The same methodology could be followed to normalize the rating boundaries of other ship types. |  |

# FCvoyage Voyage Adjustment for Adverse Weather

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Belgium, Brazil, Canada, Chile, China, Greece, Liberia, Malaysia, The Netherlands, Norway, Russian Federation, UK, US, ICS, INTERCARGO INTERTANKO, IPTA, WSC No – Denmark, France, Italy, Japan, Spain, Sweden, EC, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Belgium> Broad measure which is hard to verify <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Denmark does not support this proposal for FCvoyage <France> This “adjustment” would lead to large parts of fuel oil consumption to be excluded, which would inevitably undermine the effect of the CII as a means to reduce CO2 emissions from ships. If we try a mere estimation, taking area 34 as an example (middle Atlantic Ocean),: Figure 2 IACS standard wave data The scatter diagram above shows occurrence of Hs>5.5 m = 10 %. (link between Bf and Hs: https://folk.ntnu.no/oivarn/fld37083/beaufat303.pdf) Based on estimates for the link between sea state and extra power needed (https://ittc.info/media/1595/75-02-03-015.pdf), at first approximation, excluding emissions due to sailing in above Bf(7) leads to excluding from the CII calculations around 25% extra CO2 emissions (assuming constant specific fuel oil consumption), which would not be acceptable for us. <Greece> The proposal should adequately define these elements related to the definition of the weather/sea state (i.e., wave height, wind speed, etc.) in order to minimise the administrative burden for the shipping companies to identify the various sea states. <Italy> Broad measure (Beaufort scale 7 in principle), significant overlap with other, more targeted proposals <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Malaysia> For LNG Tanker and Membrane Type tanks, it is important to consider adverse weather. LNG Tankers & Membrane tanks type will generate more boil of gas due to rolling effect when operate more than BF7. This rolling effect will generate more boil off due to sloshing effect due to our LNG membrane or Moss tank design with for steam turbine LNG Tankers. <The Netherlands> Adverse Weather adjustment is difficult to execute. The effect of adverse weather on the fuel consumption depends on many different circumstances such as direction of the wind, wave height, temperature, speed of the ship, route planning based on weather prediction etc. All these elements make it difficult to monitor and verify when and how such adjustment should be applied. Furthermore the Cii is calculated over a full year and a year of sailing exists of different types of weather. A high consumption during adverse weather can be compensated during a better weather period. Ships operating in the same trade and area will in most cases experience on average the same type of weather. Therefor the adverse weather itself does not create a disadvantage towards the competition. Whether or not a compensation for certain type of weather is necessary should be considered during the review in 2026. <Norway> We recognize that adverse weather can be a challenge for some ships. However, we believe a correction factor applied to the period travelling in adverse weather is more appropriate than deleting that part of the journey from the calculation of CII entirely. Verification might also be a challenge. <Spain> Ships will suffer adverse weather and therefore this is a fundamental issue that should have been dealt with during the definition of reference lines. It needs to be <Sweden> Bad weather is part of normal operation of ships and is therefore included in the baseline (NB: “bad weather” in our view is not “adverse weather”) <UK> Many ships have no option other than to travel in adverse weather conditions, while others experience these conditions very infrequently. That being said, it is very difficult to define the line where adverse weather conditions start. We would support a correction factor for these situations as long as those conditions are very extreme, using an appropriate level of the Beaufort scale. <EC> Broad measure (Beaufort scale 7 in principle), significant overlap with other, more targeted proposals <INTERCARGO> The information provided allows a solution for the increased fuel consumption due to adverse weather affects. Without the proposed solution there will be negative effects on a vessel’s AER that are beyond the owner/operator’s control. <IPTA> IPTA fully supports this proposal in order to avoid seasonal market distortion for ports in bad weather areas. <Pacific Environment> Exempting voyages in Bf 7 and above is inappropriate for a number of reasons. First, fuel consumption in adverse weather conditions still has a climate impact. Second, adverse weather conditions are expected to become more common as the climate warms. They also explain that the North Atlantic trade has Bf of 7 or greater 40% of the time between November and March and 10% from May to August. At what point do we start excluding an unreasonable share of international shipping fuel consumption from CII calculations? This proposal is risky not only for the environment but also for seafarers. <WSC> This is an urgent matter which needs to be addressed to avoid potential penalizing of efficient ships because of the effects of operation in adverse weather. The variability of adverse weather and the fact that it affects different regions differently makes this a matter of maintain equity in global trade and avoiding undue penalizing of trade to/from regions subject to a heightened prevalence of adverse weather. If not agreed the consequences for member States exposed to a heightened prevalence of adverse weather could be significant, this cannot be addressed by improving ship technical efficiency or operation since there will always be an efficiency penalty when operating in adverse weather. We consider that the rationale in support of an adverse weather correction is equivalent to that for ice navigation. As such, both corrections should be taken forward. We see no logical argument why one of these two corrections could be supported at the same time as not supporting the other, such an approach would be contradictory and risk serious concerns with the process used to agree corrections. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Greece, Liberia, Malaysia, Russian Federation, Sweden, UK, ICS, INTERCARGO, INTERTANKO, IPTA, WSC 2026 – Brazil, Canada, The Netherlands, Norway, US <Norway> Verification might be a challenge. We would prefer to wait until the 2026 review to consider this issue. | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Liberia, Malaysia, Russian Federation, UK, ICS, INTERCARGO, INTERTANKO, IPTA, WSC No – Belgium, Denmark, France, Italy, Japan, The Netherlands, Spain, Sweden, US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Belgium> Not enough evidence provided <France> First we would like to recall that what is proposed is an exclusion, not a “correction” or “adjustment”. We are clearly opposed in principle to such exclusion, for the following reasons: - It is a very broad exclusion, with significant overlap with other, more targeted proposals. Corrections or adjustments should be possible only for very specific issues, affecting a limited number of ships or situations easy to identify. - The attained CII will be calculated over the yearly-averaged fuel consumption values, which will enable shipowner to adapt to variability in carbon intensity performances of ships because of adverse weather conditions have by definition already been taken into account when designing the measure, with reference lines based on actual 2019 data – including average operating conditions of this year (for which there is no evidence to have been less adverse weather than the average); The rating mechanism has been designed to allow the needed flexibility to shipowners. <Japan> Practical problems such as DCS data collection are likely to occur <Malaysia> LNG tanker on the impact of adverse weather on boil off gas due to rolling effect when operate more than BF7 <The Netherlands> The submissions indicate that further work is required with respect to defining how the meteorological conditions will be recorded. <Spain> The solution is not a factor correction but including this in the reference lines. We feel that this is already there somehow <Sweden> Bad weather is included in normal ship operation. <US> the DCS data used to establish the baseline for the standards includes fuel consumed during all weather conditions. In fact, one of the main purposes of using annual values is to allow for variation in weather and other conditions. However, at the 2026 review point, MEPC should review the impacts of this proposal to evaluate whether CIIs can be normalized for periods of adverse weather, especially in the context of more stringent standards. <EC> No evidence of severe impacts on member States or perverse incentives provided |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Liberia, Malaysia, Russian Federation, UK, ICS, INTERCARGO, INTERTANKO, IPTA, WSC No – Belgium, Denmark, France, Italy, Japan, The Netherlands, Spain, US, EC | ☐ Yes ☐ No |
| Comments <France> See comments above: the CII will be a yearly-average value. <Japan> Ships operating on the same route are expected to encounter adverse weather with a certain probability. When the ship encounters adverse weather, the load on the main engine may be reduced due to the inability to obtain ship speed, and the CII value may improve naturally. <Malaysia> About 8% of total FOC is accounted voyages are greater than BF6. LNG Tankers will generate more boil of gas due to rolling effect when operate more than BF7. With voyage in heavy weather of BF7, AER value increase is about 8%. <US> The purpose of using annual values to allow for variation in weather and other conditions. It would be expected that the DCS data includes fuel consumed during bad weather <WSC> Ships will be penalized because of conditions outside the control of shipowners and crews. As well as negative impacts on ship rating there will be a negative impact for member States located in areas subject to a heightened prevalence of adverse weather. Crucially, a correction factor is the only appropriate way forward because of the variability of weather conditions. This variability means that the affects of adverse weather cannot be built into the reference line in a sensible way since some ships will always suffer a penalty whilst other ships will receive a superior rating as a result of operating in benign conditions. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Liberia, Malaysia, Russian Federation, Sweden, UK, ICS, INTERCARGO, INTERTANKO, IPTA, WSC No – Belgium, Denmark, France, Italy, Japan, The Netherlands, Spain, US, EC, CSC, Pacific Environment | ☐ Yes ☐ No |
| If not, please explain your answer <Belgium> No information provided <France> This exclusion would create a loophole in the regulation that could be seen as a perverse incentive (cf answer to question 7 below). <Italy> No evidence of severe impacts on member States or perverse incentives provided. <Japan> The BF number is subject to the judgment of the crew and may be recorded arbitrarily high. There is a perverse incentive to target Rough Sea instead of Adverse on safety, as it is contrary to the objective of reducing CO2 emissions at sea. <Malaysia> Penalizing and possible complete removal of steam turbine LNG Tankers We do not see any perverse incentives by the application of the correction factors suggested for LNG tankers. The correction factors do replicate the differences between the actual operational profile of these ships and the operational profile of LNG tankers engaged in voyages. <The Netherlands> The weather adjustment factor is open for many interpretation. Especially the monitoring and verification is open for over estimation of the weather circumstances. Secondly, when applying the weather adjustment factor, the incentive to optimize the route or avoid bad weather is reduced. <Spain> The issue of increasing the CII to deal with adverse weather should have been included before hand in the reference lines. <US> Such an exclusion would weaken the standards and remove incentives for use of carbon reduction approaches that can be applied regardless of weather. It would also remove any incentive, from the CII, for improved weather routing. <INTERCARGO> It is our expectation that perverse incentives will be addressed, however up to 2026 this should be closely monitored to ensure the desired effect. <Pacific Environment> Third, it creates a perverse incentive whereby ships may be unwilling to route around adverse weather because there is a CII reward to be had for risking sailing in poor weather. The co-sponsors of the proposal acknowledge that adverse weather is becoming more common. <WSC> Perverse incentives can only arise if the correction factor is not agreed since routes serving certain member States would be at a disadvantage. There are also safety implications if ships face regulatory demands to reduce CII values and potential reputational damage if poorly rated, if operating in conditions which demand higher engine power (increasing CII values) to maintain safety. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct – China, Malaysia, Russian Federation, WSC Accurately Correct – Chile, Liberia, UK, ICS, INTERCARGO, INTERTANKO, IPTA Over Correct – The Netherlands, Sweden, US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Belgium> Not enough evidence <France> This question cannot be answered as what is proposed is not a correction but a pure exclusion (even with the 8-hour buffer proposed). <Malaysia> The proposed measurement for FCvoyage,j is the total mass (in grams) of fuel of type j will be under correcting as weather will increase the Boil of Gas due operation profile of the membrane tank. Unable to predict the right amount of Boil of Gas due to rolling effect when operate more than BF6. <The Netherlands> It is difficult to monitor the weather circumstances in relation to the ship operation in a fair and transparent manner. The factor opens the door for over estimating of the weather situation and incorrectly application of the adjustment. <Sweden> Bad weather is included in normal ship operation and to introduce correction factors in our view would be “over correct”. <US> It is not clear how the correction factors are calculated. Only annual effects are discussed in the relevant comments. An overcorrection could actually result ships operating more in adverse weather having a lower CII than similar ships that avoid adverse weather. <INTERCARGO> INTERCARGO currently believes that the proposed adjustment is accurate however further review is recommended following EIF of the measure to ensure accuracy. <EC> Insufficient evidence to assess <WSC> The proposed boundary conditions have been set at a level which excludes most adverse weather from the correction for most ships, so it will under correct. Additionally, a buffer has been proposed which will further lead to under correction. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Liberia, Malaysia, Russian Federation, UK, ICS, INTERCARGO, INTERTANKO, IPTA, WSC Further Development Needed – The Netherlands No – Belgium, Denmark, France, Italy, Japan, Sweden, US, EC | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> In practice, it will be impossible for any Administration, recognized organisation or PSC officer to check the correspondence between the weather undergone by the ship during an entire year and the fuel oil excluded. Therefore this exclusion could constitute a loophole that could be used by shipowner to adjust the attained CII. This would put in danger the level playing field. This exclusion would generate a lot of complexity and increase the administrative burden for the shipowners, Administrations and ROs. <Japan> The following items are problematic during verification; - The date and time declared as Adverse Weather Condition will be checked based on AIS information and Hindcast data, etc. However, it is necessary to clarify how to handle the case where there is a discrepancy between the ship's record and AIS/Hindcast. In particular, Satellite AIS data may be missing due to unseasonable weather or heavy marine traffic, and it may not be possible to obtain cross-check data that can withstand verification. In this case, inappropriate corrections may be made. - The current Noon Report is too coarse in order to properly separate the data in Adverse Weather as defined in the draft guidelines. A finer-grained report will be required. - It is generally impossible to obtain accurate data on CO2 emissions during Adverse Weather. In official records such as ship's Log books, there is ROB for Departure/Arrival, but there is no ROB for any given date and time, and it is highly likely that the said FOC will be adopted. In this case, there is no way to disprove if the amount used in Adverse Weather is overreported out of the total FOC, and there is concern about loopholes. In order to avoid the concerns in 1)-3) above, it is necessary to provide the verifier with the data obtained from VDR and Data Logger by Real Time Monitoring etc. in a form that cannot be tampered with. <Sweden> Bad weather is included in normal ship operation and baseline per shipcategory <US> As France notes in 76/7/23, it would be almost impossible to check matching of fuel consumption and distance with the periods where 7 Bf is reached. <EC> Impossible to assess in the absence of numerous guidance provisions (listed in the proposal) that would be needed to put such measure in place |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Liberia, Malaysia, Russian Federation, UK, ICS, INTERCARGO, INTERTANKO, IPTA, WSC No – Belgium, Denmark, France, Italy, The Netherlands, Sweden, US, EC | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <Belgium> Need for extra guidance provisions that are not yet available <Denmark> How can the weather be verified? <France> The proponent propose items to be considered for a good reporting by the ship, which are relevant (how to determine wave height, data logging requirements, weather records, etc.), but they do not propose any tool or procedure to verify these records. We believe it is absolutely impossible to verify independently and robustly such reportings, and therefore that this voyage exclusion is not applicable. <Italy> Impossible to assess in the absence of numerous guidance provisions (listed in the proposal) that would be needed to put such measure in place. <WSC> WSC and others have already proposed the means of verification, however this needs to be further defined in the guidelines, e.g. log book entries are required but not sufficient to confirm weather conditions experienced, thence weather records or wind records should be at all times provided to ascertain the adverse weather conditions. Therefore, more work is needed to draft guidance, however we are confident this can be satisfactorily completed in a further round of correspondence. |  |

# FCvoyage Voyage Adjustment for Sailing in Ice

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Belgium, Brazil, Canada, Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA, WSC No – Japan, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Belgium> Enough evidence in proposal <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Finland> The proposal for FCvoyage, "Voyage adjustment when sailing in ice conditions, which means sailing of an ice-classed ship in a sea area within the ice edge" was made by Estonia, Finland, Russian Federation and Sweden in document MEPC 76/7/21. In our reply to Q2 in Round 3 of the CG Finland provided further information on this issue. The information provided is based on sound scientific research and data from ships sailing in ice conditions in the Baltic Sea area. In our view, the level playing field between ships sailing in ice conditions and ships sailing only in open water can be achieved by adopting the proposed voyage adjustment when sailing in ice conditions. The idea of the proposal is that the additional operational CO2 emissions generated by ice-classed ships sailing in ice conditions are adjusted by a transparent and accurate method in the calculation of the attained CII of an ice-classed ship. <Italy> Proposal substantiated in depth <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Norway> Only the portion of the voyage when the ship has sailed in ice conditions should be addressed, not the entire voyage. We would prefer to apply a correction factor to the relevant portion of the voyage to ensure all emissions are included in the CII calculation, instead of excluding it altogether. <Spain> The issue of sailing in ice deserves a correction factor by itself a heavier ship in need of more power to break the ice, however the ice conditions will differ somehow so determining an accurate factor is not easy <Sweden> "Sea ice": Any form of ice found at sea which has originated from the freezing of sea water (paragraph 1.1 in the WMO Sea-Ice Nomenclature, March 2014). Ice conditions are not taken into account in the current baseline. Ice-classed ships consume much more fuel when sailing in ice-covered waters compared to sailing in the same area in open water conditions. <US> However, as proposed, at the 2026 review point, MEPC should review the impacts of this proposal to evaluate whether CIIs can be normalized for periods of operation in ice conditions, or whether to consider voyage exclusions. <EC> Proposal substantiated in depth <ICS> Please note that the Finnish Shipowners Association (FSA) have shared with the ICS Secretariat their completed round 4 TOR 3 questionnaire. I confirm we are fully aligned with their responses and comments to section H. <IPTA> IPTA fully supports this proposal on the basis of the same comments we made for the adverse weather correction. <Pacific Environment> As we have stated previously, we oppose any voyage exclusions or correction factors for voyages in ice. The purpose of the CII is to obtain an accurate estimate of a ship’s actual operational carbon intensity throughout the year, which must include emissions associated with all fuel consumption. Exclusions for operations in ice conditions also create a perverse incentive to operate in ice conditions, which risks the safety of the ship and its crew as well as increases the risks of oil spills in polar regions like the Arctic, and would reward fuel consumption and emissions, including Black Carbon, in the Arctic. <WSC> We consider that the rationale in support of an ice navigation correction are equivalent to that for operation in adverse weather. As such, both corrections should be taken forward. We see no logical argument why one of these two corrections could be supported at the same time as not supporting the other, such an approach would be contradictory and risk serious concerns with the process used to agree corrections. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Belgium, Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERTANKO, IPTA, WSC 2026 – Brazil, Canada, US | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Belgium, Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, Sweden, EC, ICS, INTERTANKO, IPTA, WSC No – Norway, UK, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Norway> We would prefer to apply a correction factor to the relevant portion of the voyage to ensure all emissions are included in the CII calculation, instead of excluding it altogether. <Sweden> Sweden supports proposal for voyage adjustment for ships when sailing in ice conditions. The voyage adjustments for ice-classed ships when sailing in ice conditions are needed, because ice-classed ships consume much more fuel when sailing in ice-covered waters compared to sailing in the same area in open water conditions. Sweden also supports excluding operations in severe adverse weather conditions from a shipʹs carbon intensity indicator (CII) rating calculation as proposed. <UK> While we do support the actual concept of introducing a correction factor for ice conditions, the definition of near ice conditions and removing ice trajectories completely from the total could lead to actually planning more trips near the ice than necessary to avoid counting this fuel consumption. Moreover, this correction factor could be added/harmonized with the adverse weather conditions one. <US> More work needs to be done on the consideration of correction factors for sailing in ice conditions. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Belgium, Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA, WSC No | ☐ Yes ☐ No |
| Comments <Belgium> Certainly for short voyages <Finland> Yes, there will be a significant negative impact on ice classed ships if voyage adjustment of sailing in ice conditions is not accepted. Failing to appropriately consider the operational impact of ships sailing in ice conditions causes a risk for disproportionate negative impact on several member states. The proposed voyage adjustment is important to the industry and communities located for instance in the northern Baltic Sea area, which utilizes the transport capacity of ice-classed ships having the ability to sail safely in ice conditions, in order to ensure all-year-round maritime transport in this area, even during the winter months. Shipping to the ice-bound ports might meet serious difficulties without the proposed mitigations for ice-classed ships. We would also like to point out that even if the winters would become milder due to global warming, from the shipping point of view, a mild winter with a small ice coverage can be even more challenging than a harsh winter with a large ice coverage. For instance, breaking ice-edge may create a glue like ice-slush, which can be extremely difficult to pass - even with the help of an icebreaker. Our reply to Q3 in Round 3 of the Correspondence Group gives further information on the negative impact on ships. Especially our reply to question Q3.d highlights the negative impacts of sailing in ice conditions on the attained CII of ice-classed ships, if the proposed voyage adjustment would not be applied to these ships. The assessment of the effect of sailing in ice conditions on the attained CII of ice-classed ships was based on the information provided in document MEPC 76/INF.67, Information and analysis of the effect of sailing in ice conditions on the attained CII of ice-classed ships, submitted by Finland, as well as a more recent study conducted by the Finnish Meteorological Institute: Majamäki Elisa, Jalkanen Jukka-Pekka and Johansson Lasse (2021), Effect of sea ice on fuel consumption and carbon intensity of shipping in the Baltic Sea area in 2009 - 2019, where the impact of sailing in ice conditions on fuel consumption and on the attained carbon intensity of ice-classed ships has been estimated. <EC> Major overlap identified with the proposal on short voyages (see below) <ICS> ICS are fully aligned with FSA’s responses and comments to section H of the round 4 TOR 3 questionnaire. |  |
| 5. Are the perverse incentives addressed? Yes – Belgium, Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, US, EC, INTERTANKO, IPTA, WSC No – UK, ICS | ☐ Yes ☐ No |
| If not, please explain your answer <Russian Federation> One may suggest that as a perverse incentive a ship may intentionally sail in ice for the only reason to adjust its fuel consumption and improve CII. It is extremely unlikely that a ship would do that under any circumstances as there is number of obvious factors that make sailing in ice more risky, less economically feasible, more logistically complicated. <UK> There may be more perverse incentives encouraged by eliminating the section of the ships voyage near ice, such as planning more unnecessary trips in ice conditions, which could lead to more CO2 emissions. <ICS> ICS are fully aligned with FSA’s responses and comments to section H of the round 4 TOR 3 questionnaire. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under Correct – China, Spain, UK, WSC Accurately Correct – Belgium, Chile, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Sweden, EC, ICS, INTERTANKO, IPTA Over Correct | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Finland> Our view, referring to our reply to question Q4 in Round 3 of the CG, the proposed voyage adjustment is precise, and the level playing field between ships sailing in ice conditions and ships sailing only in open water can be achieved by adopting the proposed measure. For reporting of the fuel oil consumption, the monitoring of the onboard fuel oil consumption for each consumer (ME+DG and Aux Boilers) could be improved by introducing mass flow meters with data logging capability to accurately capture the fuel oil consumption. The use of mass flow meters in combination with the ships GPS signal and ice charts, it’s possible to accurately measure the distance travelled and the additional fuel oil consumption of voyage when the vessel is sailing in ice condition, starting at the ice edge and ending at port. <Sweden> Sweden supports excluding operations in severe adverse weather conditions from a shipʹs carbon intensity indicator (CII) rating calculation as proposed as “accurately correct”. <UK> To harmonise this CF with the adverse weather condition CF, or to just eliminate a proportion of the emissions during near ice voyages, which should be well justified with real data on emissions during ice passages, but not to completely eliminate these emissions from the formula. <EC> Underpinned by ship-specific data and analysis <ICS> ICS are fully aligned with FSA’s responses and comments to section H of the round 4 TOR 3 questionnaire. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Belgium, Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, Sweden, EC, ICS, INTERTANKO, IPTA, WSC Further development needed – Norway, UK, US No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <Norway> In appendix 2, Guidance on reporting of fuel oil consumption and distance travelled for voyages for ice-classed ships when sailing in ice conditions in the draft guidelines on correction factors and voyage adjustments for CII calculations (G5), we suggest the following addition to the text in section 1.1 and 2.1: “..If the portion of the ship's voyage is excluded from calculations of attained CII index value due to sailing in ice conditions, this should be clearly marked in the SEEMP monitoring plan, ships logbook should include data entries for the portion of the voyage with the date, and time and location when the ship encountered ice conditions and data added to the data reporting format. In the “Standardized data reporting format for collection of data for voyage exclusions”, the word “time” should be added. “Date, time and location of the excluded voyage(s)” There is nothing in the guidelines currently that says ice charts need to be provided for verification. We propose to add the following item that could be reported to the Administration for verification: “Ice charts related to the period for voyage adjustments” |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Belgium, Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Sweden, UK, EC, ICS, IPTA No – Spain, US, WSC | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <Spain> This factor should be dynamic depending on the development in routes, ice and temperatures <UK> Provide further analysis of real data on extra emissions in ice conditions that can statistically justify the correction factor, which should be a fraction of the actual emissions in ice, but still not equivalent, we would rather choose a correction factor that partially eliminates the estimated extra emissions, so to not encourage perversive incentives such as planning unnecessary voyages near ice, especially given that the definition of near ice is still quite vague. <US> Like any other correction factor or voyage exclusion, this proposal would increase the complexity of the calculation and make verification more difficult. It is not clear how the authorities would verify when a portion of a voyage is in ice conditions (or if the severity of those conditions warrants an adjustment) <EC> Under specific conditions as proposed. <WSC> Although scope of applicability is clear, the collection of evidence should be subject to a specific and clear paragraph, exclusion period being at all times related to weather bulletins in the sailed area. |  |

# CFshort Standalone Correction Factor for Short Voyages of 72 hours or less

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, Greece, Liberia, [Malaysia], Norway, INTERCARGO, INTERTANKO, IPTA, WSC No – China, Denmark, France, Italy, Japan, The Netherlands, Spain, Sweden, UK, US, EC, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <China> - There is a very low correlation between AER and Time as the R2 values is very small. - The fuel consumption at the arrival port may be not related to the underway time. - The fuel consumption used for cargo operations may be excluded by using other correction factors. <Denmark> Denmark do not support the CFshort proposal. This would undermine the CII regulation. This would exclude all shortsea shipping <France> This “adjustment” would lead to large parts of fuel oil consumption to be excluded, which would inevitably undermine the effect of the CII as a means to reduce CO2 emissions from ships. <Greece> Bulk carriers, tankers, containerships and other ship types with many short voyages are being unfairly treated by the current CII Guidelines compared with sister/similar vessels with limited or zero number of short voyages in three ways: 1. While underway (voyage leg), due to due to additional number within the calendar year engine load transient operations, maneuvering and reversing. 2. While at berth due to additional number within the calendar year of cargo discharging/loading operations 3. Due to waiting time at port or anchorage. There is no provision in the current CII framework to readjust these vessels which are being penalized due to their increased number of port visits compared to sister/similar ships with normal operational profile. Using the same methodology, other ship types engaged in short voyages could be considered. Analysis for Vehicle Carriers and respective correction factor proposal has been submitted to the CG for consideration enabling the scope to be further extended. <Italy> Broad measure, overlapping with more targeted proposals, scope dependent on the definition of "voyage" and risk of unintended behavioral changes. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. - The proposal claims to introduce a correction factor for bulk carriers, etc. that sail within 72 hours, but it does not clearly state the basis for whether the figure of 72 hours is quantitatively correct. <Liberia> This correction Factor should be applicable to all ships impacted by port dwell time, i.e. not only ships on short voyages. We note and concur with the argumentation provided in the information document “WSC Additional Supporting Information - Port Dwell Time Correction” submitted on IMO Space by WSC. <Malaysia> LNG tankers stay longer for a period more than 24 hrs. for loading or unloading operation in the port. Generally, LNG tankers takes about 18 to 26 hrs. for cargo operation. Some LNG tankers need to wait longer than 24 hrs. in anchorage for loading and unloading operations. <The Netherlands> At this moment we don’t see the need for a compensation of short voyage compensation. Ships operating in these trades all have the same disadvantages. We should first analyze based on real data from Cii calculation whether it is necessary to adjust for short voyages. There is no competition between a deep sea tanker and and short sea tankers. <Norway> We would not support excluding short voyages, or the fuel consumed during discharge operations, as we want to encourage energy efficiency measures in all aspects of ship operation. We could support a correction for short voyages for tankers, bulk carriers and container ships. Other ships types may also be relevant to include, but further analysis is needed. This should be seen in conjunction with adjustments of reference lines for smaller ship segments. <Spain> The fundamental problem is the lack of a definition of transport work here. Creating a factor like this is very broad and therefore this issue should have been part of the discussions in the CII reference lines. We understand the incentives and disincentives of ships staying in ports but most vessels would have the same problem. The way to address this again should have been the reference lines. See our comment at the end of the document <Sweden> Operational factors affecting fuel consumption should be included in the baseline for A-E rating. <UK> Short voyages should be encouraged to use alternative propulsion such as electric or hybrid. <US> The CII standard is rate-based and therefore the CII metric is the same when the ship is underway, regardless of the distance. The CII reference line was developed using all trips and the standard is not based on a single trip, but rather the average over a year. Additionally, there are several approaches that could be used to reduce emissions at port, and these should not be discouraged by excluding this operation. <EC> Broad measure, overlapping with more targeted proposals, scope dependent on the definition of "voyage" and risk of unintended behavioral changes <ICS> ICS agrees the need for this correction factor, and we note the proposal currently extends to container ships, tankers and bulk carriers. However, many other ship types may be operated in a similar manner with short voyages of 72 hours or less. Hence, we suggest that group members should be invited to submit data and justification for the extension of application of this correction factor to other ship types. <INTERTANKO> INTERTANKO agrees there is a merit on looking to this correction factor, but it should not be applicable to DP Shuttle Tankers as all these do have voyages less than 72 hours. INTERTANKO also has a proposed correction factor to apply to tankers engaged in Ship-to-Ship (STS) operations on which the time of the voyage is limited also to 72 hours but has an additional limitation of the distance of the voyage to 600 nautical miles to avoid any perverse incentives or under/over corrections. Data to be collected should not include data from DP Shuttle Tankers. In addition, the data for voyages up to 72 hours should include the distances covered. <IPTA> IPTA fully supports the need for this correction factor which is fundamentally caused by only using distance travelled in the calculation. Port time and associated delays will have a significant negative effect on short voyages. <Pacific Environment> To understand whether or not it is appropriate to exclude or correct for voyages less than 72 hours in duration, one needs to know the proportion of absolute fuel consumption and emissions this would affect. Support for or opposition to this proposal will vary depending on whether voyages less than 72 hours make up a small proportion of total annual fuel consumption for these ships or a large portion. We also need to know if a 72 hour exclusion would incentivize additional short, inefficient trips, which would be a perverse incentive. Pacific Environment and CSC oppose any exclusions or correction factors because the purpose of the CII regulation is to reduce the actual operational carbon intensity of ships, not just the carbon intensity of long voyages or the carbon intensity of ships while they’re underway. <WSC> We consider that the sensitivity of AER to distance travelled and time in port needs careful consideration since if not properly considered it could result in efficient ships being poorly rated. The underlying problem is extended dwell time in port (either at anchor or at berth) and the affect this has on AER values. As such it affects all ships and is not just a short voyage issue. Delays and extended dwell times caused by port congestion and delays at berth affect all ships. It would be unacceptable to penalize ships because of such issues, transferring accountability for port congestion from port authorities onto ship operators. To demonstrate that long port dwell time affects long distance deep-sea ships just as much as ships engaged in short voyages WSC has provided an analysis of a typical North – South service rotation. As such, we consider that the matter needs further consideration but that significant additional work is necessary to analyze and understand the problem. Two issues in particular need further work: - Interaction with other correction factors should be analyzed to understand how a correction for extended port dwell times will affect other factors; - The duration of the residence time needs to be carefully considered; we consider a figure of 72 hours appropriate; Further work should be carefully considered to prioritize effort, for example it may be useful to initially prioritize certain size categories as a matter of urgency. An alternative option might be to address inefficiency caused by port congestion and delays at berth via the SEEMP guidelines and SEEMP verification guidelines. In addition, we note that an earlier proposal to exclude extend periods out of service from rating values has been removed from the list of correction factors. If ships are laid up but need to operate auxiliary engines to maintain crew habitability and essential systems this will result in ships being D or E rated. We suggest that periods of [30] days or more at lay up or otherwise not in service be excluded from rating values. We consider that this part of work to consider the effects of dwell time could be addressed very quickly and that there is general agreement that including periods stationary for more than 30 days within the ships rating would be counterproductive. To do so would result in meaningless ratings for some ships and distort the rating system. As such we suggest that this element be completed as a matter or urgency. Please refer to additional information document submitted by WSC which demonstrates the influence of port dwell time on CII ratings and that it is a matter for all ships, not only ships engaged in short voyages. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Greece, Liberia, Malaysia, Norway, Sweden, ICS, INTERCARGO, INTERTANKO, IPTA, WSC 2026 – Brazil, Canada, The Netherlands <France> In our view this issues should not be considered anymore <WSC> We would like to qualify our answer to this question. We consider the issue of port dwell time to be an urgent matter, however we also recognize that significant further work is needed. Therefore, we would suggest the following: - That the correspondence group consider the matter of dwell time caused by berth non-availability, time at anchor or slow steaming offshore awaiting a berth and delayed departure time. The group should carefully consider whether this can be addressed now or whether work should continue after the first set of guidelines. WSC members are available to gather supporting data on this issue; and - A correction for ships laid up or anchored for more than 30 days be completed now. If the first bullet point cannot be addressed in the time available to the group, then we suggest that work should continue so that such a correction can be agreed before 2026. | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Greece, Malaysia, Norway, INTERCARGO No – China, Denmark, France, Italy, Liberia, The Netherlands, Spain, Sweden, US, EC, ICS, INTERTANKO, IPTA, WSC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <China> In case of ships with more cargo operations having voyages that consist of a few voyages of less than 72 hours and a majority of voyages of more than 72 hours, the poor attained AER cannot benefit from CFshort as cargo operations may consume large fuel consumption. <France> First we would like to recall that what is proposed is an exclusion, not a “correction” or “adjustment”. We are opposed to such exclusion, for the following reasons: - It is a very broad exclusion, with significant overlap with other, more targeted proposals. Corrections or adjustments should be possible only for very specific issues, affecting a limited number of ships or situations easy to identify. - Correcting all short voyages would essentially impact entire regions without underlying specific conditions. - The attained CII will be calculated over the yearly-averaged fuel consumption values, which will enable shipowner to adapt to variability in carbon intensity performances of ships due to the different proportion of short and longer voyages that will operate on during one year - Such variability and operational constraints have by definition already been taken into account when designing the measure, with reference lines based on actual 2019 data – including average operating conditions of this year for all ships, regardless the duration of the voyages. - The rating mechanism has been designed to allow the needed flexibility to shipowners. Also, in the explanation provided by the proponents, we can read “Bulk carriers, tankers and containerships with frequent port calls have increased fuel consumption for transient operations, maneuvering/reversing and for cargo discharging while at berth. These ships which due to their special commercial obligations carry out frequent port visits and thus low distance travelled, will present a significantly increased attained carbon intensity indicator (against a sister ship with different operational profile) and therefore a distorted picture with regards to ships’ real efficiency and rating.” This paragraph assumes sister ships are frequently used with different operational profiles, and this would constitutes the main cause of distortion, but this assumption is not demonstrated. In our view, ships always engaged in shortsea shipping are generally smaller and ships always engaged in deep sea shipping are bigger, with no significant overlap between the two types (in particular not possible for big ships to enter into big ports). And the CII frameworks already enables to take into account the structural differences in carbon intensity due to size throughout the shape of the reference lines. <Sweden> Operational factors affecting fuel consumption should be included in the baseline for A-E rating. <US> The CII standard is rate-based and therefore the CII metric is the same when the ship is underway, regardless of the distance. It appears that the results in ISWG-GHG-8/3/1 showing higher AER for short trips are a result of including the port visit as part of each trip. In fact, this paper calls for an exclusion of port visits in addition to short voyages. Two approaches are presented. The first is to exclude all voyages less than 72 hours. The second is to apply a correction factor to trips less than 72 hours, that ranges from 0 to 1.0 depending on the length of the trip. Both approaches propose to exclude port visits. <EC> Correcting all short voyages would essentially impact entire regions without underlying specific conditions. Subject to overlap with the navigation on ice but also more ship-specific proposals such as FC STS or boilers. <ICS> We suggest that group members should be invited to submit data and justification for the extension of application of this correction factor to other ship types. <INTERTANKO> Need to review the data and the parameters under which this correction factor is used <WSC> Although the proposal provided is a good starting point, we consider that the real issue is port dwell time, not short voyages. As such the correction should address the underlying issue of dwell time. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Greece, Liberia, Malaysia, Norway, ICS, INTERCARGO, INTERTANKO, IPTA, WSC No – Denmark, France, Italy, The Netherlands, Spain, Sweden, US, EC | ☐ Yes ☐ No |
| Comments <France> See comments above: the CII will be a yearly-average value. <Greece> For vessel with high number of frequent voyages, there is a significant negative impact on their attained CII and rating. The higher the number of short voyages, the higher the impact. The total fuel consumption/CO2 emissions will be reported, and the short voyages will not be excluded but a correction factor will apply for a fair treatment ensuring a level playing field. <Italy> Correcting all short voyages would essentially impact entire regions without underlying specific conditions. Subject to overlap with the navigation on ice but also more ship-specific proposals such as FC STS or boilers. <Sweden> Operational factors affecting fuel consumption should be included in the baseline for A-E rating. <US> The CII standard is rate-based and therefore the CII metric is the same when the ship is underway, regardless of the distance. The CII reference line was developed using all trips and the standard is not based on a single trip, but rather the average over a year. Additionally, there are several approaches that could be used to reduce emissions at port, and these should not be discouraged by excluding this operation. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Greece, Malaysia, Norway, Sweden, ICS, INTERCARGO, IPTA No – China, Denmark, France, Italy, The Netherlands, US, EC, INTERTANKO | ☐ Yes ☐ No |
| If not, please explain your answer <China> To get a better CFshort , ships may be speeded up as CFshort will decrease with a decrease of time. <France> This exclusion would create a loophole in the regulation that could be seen as a perverse incentive (cf answer to question 7 below). In contrary, it could generate a risk of unintended behavioral changes (provide motivation to increase short sea activity). <Greece> If the measure is not implemented, ships with high number of short voyages within the calendar year, will need to reduce their speed to compensate for the bad rating or may decline engaging on this type of voyages thus supply problems may arise on specific areas. Shipping is the most energy efficient way of transportation. Unfair treatment on short voyage operations may result in an increase in land transportation. <Spain> We cannot comment at this time as we consider significant further work is needed to understand the correction factor. <US> The perverse incentive would be to remove any requirement for improvements in GHG emissions at port or for voyages up to 3 days long, even though there are technologies available to reduce these emissions. In fact, ships could speed up during short trips with no thought to their GHG impact. This is especially true for voyage exclusions. The perverse incentive is perhaps less for the correction factor approach, but this would still over-correct as described below and adds unnecessary complexity to the CII. <INTERTANKO> Setting a time limit alone for the applicability of a correction factor may incentivise ships speeding up to cover longer distances in the time limit. <WSC> We cannot comment at this time as we consider significant further work is needed to understand the correction factor. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Greece, Malaysia, ICS, INTERCARGO, IPTA Over Correct – Sweden, US, INTERTANKO | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> This question cannot be answered as what is proposed is not a correction but a pure exclusion. <Greece> The proposed correction factor is based on analysis carried out using the IMO DCS data from DNV database for: • 650 containerships with total 59,071 voyages • 1218 bulk carriers with total 52,304 voyages • 1170 tankers with total 44,819 voyages A large number of data have been used to reach a high level of accuracy. <Italy> Insufficient evidence to assess, also given the fact that every short voyage would be corrected (even if only one per year). <Spain> We cannot comment at this time as we consider significant further work is needed to understand the correction factor. <Sweden> Operational factors affecting fuel consumption should be included in the baseline for A-E rating. Therefore we understand the proposal as “over correct” in this case. <US> The correction factor approach is seems to over-correct. The correction factor appears to have been calculated using the port visit as part of the voyage. However, as the proposal is to exclude port operations, then apply the correction factor to the underway voyages less than three days, such a correction factor would be inherently too high. For a 36 hour voyage, the correction would be on the order of 20-30%. <EC> Insufficient evidence to assess, also given the fact that every short voyage would be corrected (even if only one per year) <INTERTANKO> The correction factor should be a function of two parameters defining the voyage: time limit + maximum distance. The correction factor should apply only to the fuel consumption during these voyages. <WSC> We cannot comment at this time as we consider significant further work is needed to understand the correction factor. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Greece, Malaysia, The Netherlands, Norway, ICS Further Development needed – Denmark, Italy, Liberia, EC, INTERCARGO, INTERTANKO, IPTA, WSC No – France, Spain, Sweden, US | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> In practice, this exclusion will require from Administrations, recognized organisation or PSC officers a lot of complex work to check the correspondance between the duration of the voyages undergone during all the year and the fuel consumption excluded from the CII calculation. For this reason, this exclusion could constitute a loophole that could be used by shipowner to adjust the attained CII. This would put in danger the level playing field. This exclusion would generate a lot of complexity and increase the administrative burden for the shipowners (in particular small and medium companies operating in short-sea shipping), Administrations and ROs. <Greece> Verification is a quite simple process as the required data are being submitted in accordance with para.4.1 of Resolution MEPC.292(71). Once the hours underway from the departure to the arrival port are less than 72hours, the correction factor will be applied to the emissions during the voyage and those occurred at the arrival port. • The boundary conditions are the emissions occurred from the beginning of the voyage until the vessel leaves for the next port. This information is available through arrival/noon/departure reports. Also, the consumption at port is measured separately and adequately reported. • No additional measuring devices needed • Logging requirements: fuel consumption and hours underway recorded in ships’ log books, reporting templates, electronic system. • Definition: Short voyage means voyage between two ports with duration less than 72 hours underway. • No additional documentation above the required by MEPC.292(71) <US> This proposal adds a great deal of complexity to the CII. It could be very subjective on what constitutes a trip and it would be nearly impossible to verify. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Greece, Liberia, Malaysia, The Netherlands, Norway, ICS, INTERCARGO, IPTA No – Denmark, France, Italy, Spain, Sweden, US, EC, INTERTANKO | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <France> The proponent propose items to be considered for a good reporting by the ship, which are relevant, but they do not propose any tool or procedure or modification of the DCS guidelines to collect and verify these records. We believe it will be impossible to verify independently and robustly such reportings, and therefore that this voyage exclusion is not applicable. <EC> Further specification of detailed reporting provisions and definitions would be needed. Can further discriminate small operators with less short voyages given the high administrative burden <WSC> We cannot comment at this time as we consider significant further work is needed to understand the correction factor. |  |

# FCelectrical Correction Factor for Reefer Electrical Consumption

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Cyprus, Denmark, Finland, France, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Singapore, Spain, UK, US, EC, ICS, WSC No – Japan, Sweden, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Denmark supports this proposal. There is full evidence that this correction is verifiable. The proposal is targeted, and there is a risk that CII will not be accurate enough to reward/penalize the most efficient/non-efficient vessels. <France> We thank the proponents for the analysis provided, but in our view there is still lacking an estimate of the total carbon intensity effort (and therefore CO2 emissions) that would be deducted from the use of this correction factor. This is necessary to assess whether there would be a need to compensate this deduction in the reduction factors (G3 guidelines) and, if so, how this could be done. This would have to be done before the adoption of any correction factors, during the review of the short-term measure if it is not possible before MEPC 78. This would be particularly necessary for this correction factor, that would probably be the most impacting in terms of amount of emissions related. A basic estimation in the table below shows that CO2 emissions related to reefers powering onboard is a significant share of the total CO2 shipping emissions: scenario A; scenario B. Estimated movement of reefers/year; 65 708 445; 65 708 445. Reefer mean power consumption onboard (kW); 2; 6. onboard mean travel time of a reefer (days); 10; 10. Total electrical energy consumption (GWh); 31540; 94620. Electricity from fuel efficiency; 0.4; 0.4. Fuel LHV (MJ/kg); 42.7; 42.7. Fuel to CO2 ratio (for MDO); 3.21; 3.21. Fuel energy consumption (GWh); 78850; 236550. Fuel mass consumption (Mt); 7; 20. Total CO2 mass (Mt); 21; 64. Total shipping emissions (Mt); 1 056; 1 056. % Total CO2 shipping emissions due to reefers; 2%; 6%. Besides, the analysis provided does not take into account the fact that the reefer trade is expected to grow annually at around +8% TEU/year between 2020 and 2030 (source: https://www.psmarketresearch.com/market-analysis/reefer-container-market-report) <Italy> Targeted measure, substantiated by evidence. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Norway> We recognize that maintenance of some cargoes will require more energy than others, but don’t think subtracting all fuel used for cargo maintenance is appropriate since some will be reflected in the reference line. In general, we recognize that cargo maintenance energy use should be handled to enable a fair comparison of ships, but we believe that this should not be solved by deducting consumption but rather as more simple correction factors. <Sweden> Reefer electrical consumption is a part of the transport capacity, as a general value included in the baseline for containerships. <UK> Unsure as to why power meters would not rapidly become widely adopted as an accurate method of quantifying FC electrical that can simply be deducted from the emissions. This is in keeping with the intent of the base equation (emissions/work done) to capture accurate measure of emissions. WSC has proposed a three-year transition during which use of a default value would be permitted to allow retrofitting of power meters. If this activity is started in Jan 2022 then only E rated ships at the end of 2023 and 2024 will be affected and this can be considered on a case-by-case basis. Access to finance and cargo owners wanting highly rated ships will further encourage the take up and installation of power meters. The only remaining issue is consideration of leniency relative to other container ships that can slow down to meet CII. Reefer vessels may not be able to solw down due to perishable goods. That should be dealt with in 2024 IF rated E or 2026 IF rated D for 3 years. <EC> Targeted measure, substantiated by evidence (same route, different cargo, risk of perverse incentives). <ICS> We fully agree with the justifications provided by WSC for the inclusion of this correction factor, thereby supporting transportation via refrigerated containers. These reasons include: - Farmers in many parts of the world, including within developing countries are dependent upon refrigerated containers to transport their produce to market; - Security of supply of perishable fresh food and medical supplies at affordable cost is heavily dependent on this mode of transport; - Without this correction factor there is a risk of modal shift to air transport. <Pacific Environment> Similar to our other comments, we are of the view that fuel consumption associated with cargo cooling should not be exempt from or corrected for in the CII calculations because this fuel consumption does have climate impacts and does affect the actual carbon intensity of transport work. <WSC> Please refer to additional information submitted by WSC on November 29, as well as the comprehensive information and analysis submitted to round 3. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Cyprus, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, Spain, Sweden, UK, EC, ICS, WSC 2026 – Brazil, Canada, France, Norway, US | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, Spain, EC, ICS, WSC No – France, Japan, Norway, Sweden, UK, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> In general we do not support correction factors for reefer ships as their relatively lower energy performance is due to operational and market choices that can be driven and changed (and not to unavoidable external factors). Therefore, we consider that these ships should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity due to reefers with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. That said, if the inclusion of such correction factor had to be adopted by the Committee, we suggest the following amendment in the formula: to replace 0.75]-[0.03yi by 0.75-0.75×yi8 This would have the consequence to reduce the period in which correction factors are used in practice to 8 years from 2023 (instead of 25 years as proposed initially) and to sharpen the decrease of their effect over time, so that operators be incentivized to improve more rapidly the specific carbon intensity of ships benefitting from this correction factors and potentially other correction factors adopted at MEPC 78. <Japan> Simply allowing GHG emissions deductions for cargo reasons would hinder the achievement of GHG reduction targets. It is not appropriate to make an exception only for electricity consumption related to reefer containers. <Sweden> Reefer electrical consumption is a part of the transport capacity, as a general value included in the baseline for containerships. <UK> A correction factor adds unneeded complexity <US> Such an approach would reduce the incentive to minimize CO2 emissions associated with refrigerated cargo, such as using low or zero carbon fuels. It would add a great deal of complexity to the CII calculations and enforcement, as well as reduce the effectiveness of the CII standards. However, we are open to considering this during the 2026 review in the context of more stringent standards. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, EC, ICS, WSC No – France, Spain, Sweden, UK | ☐ Yes ☐ No |
| Comments <Spain> The mitigation should have been to consider this issue in the reference lines considering an average number of refrigerated containers for different sizes (boxes) and then develop the correction accordingly. This is the typical problem of one ship carrying containers, which will be the same as ships carrying heated cargo. <Sweden> Reefer electrical consumption is a part of the transport capacity, as a general value included in the baseline for containerships. <EC> Provides incentives for actual metering with a conservative default value <WSC> Failure to agree a correction will move ships from superior to inferior ratings because of the cargo they carry, including food stuffs to maintain a balanced food supply in member States reliant on imported food and pharmaceuticals (including vaccines). The only option to avoid such a penalty in the near – midterm would be to carry fewer reefer containers on each ship. This could have significant negative impacts for those member States which either export or import refrigerated food, pharmaceuticals and other essential goods which have to be refrigerated in transit. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, Sweden, ICS, WSC No – France, Norway, UK | ☐ Yes ☐ No |
| If not, please explain your answer <Denmark> It may lead to increased absolute emissions due to the need for more capacity and possible modal shift. Furthermore, it will increase the prices on reefer slots and have a disproportionate negative impact on trade on certain reefer goods. <France> In our view, the absence of this correction factor would not generate any perverse incentive. Regarding perverse incentives related to the implementation of the correction factor (such as preventing from making all efforts to reduce the carbon intensity of the concerned ships elsewhere or preventing an earlier use of onshore power supply), we are of the view that they could be limited only if the 𝑦𝑖 factor increases more rapidly, as proposed in question 3 above. <Italy> Provides incentives for actual metering with a conservative default value. <Norway> In general, we recognize that cargo maintenance energy use should be handled to enable a fair comparison of ships, but we believe that this should not be solved by deducting consumption but rather as more simple correction factors. <UK> While ships could move away from the reefer market in order to raise their rating, they can also have a power meter installed and maintain access to this market. <WSC> There are no perverse incentives associated with the correction factor. The correction will under-correct therefore ships carrying reefer containers will still have a worse AER value than if not carrying reefer containers. The correction will, however, limit this penalty and prevent ships carrying reefer containers being unduly penalized. The only perverse incentives will arise if the correction is not supported. A greater number of ships may be deployed to maintain reefer transport supply and in some cases there may be modal shift. Both of which will increase aggregate emissions. In addition, it should be underlined that the use of a shaft generator does not lead to any misuse of the correction or over correction for the following reasons: - Assuming that reefer consumption is taken from shaft generator would mean that no power is consumed by reefers from the arrival at the pilot station throughout the port stay untill the vessel sails and drops the pilot which is obviously incorrect. Indeed the power which is taken to feed reefers cannot be taken on any shaft generator but on independent AEs. - Any load on a shaft generator is taking power from the shaft, meaning that main engine fuel load needs to be increased to maintain the same speed if the shaft generator is used to supply the ships electrical power. This results in higher main engine fuel consumption. - Not all container vessels have shaft generators, and in most cases even where provided the shaft generator does not supply all electrical power load when carrying reefer containers. This is because shaft generators (where installed) are primarily specified for the ships auxiliary load rather than the cargo load. We raise this point as in discussions it is apparent that some misunderstand both the use of shaft generators in container ships and how many container ships are provided with shaft generators. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct – China, Finland, WSC Accurately Correct – Chile, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, EC, ICS Over Correct – Norway, Sweden | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Finland> As explained by WSC in their reply to question Q4 in the 3rd Round of the CG, the use of the default value option would under correct the power used, but if using power metering, accuracy would be assured. <Italy> Accurate with actual metering, under correct with a default value <Spain> This needs to be assessed and at this time we are not in a position to consider whether it is under correct <Sweden> Reefer electrical consumption is a part of the transport capacity, as a general value included in the baseline for containerships, we understand as “over correct”. <EC> Accurate with actual metering, under correct with a default value <WSC> A correction cap and annual reduction will ensure under-correction. In addition, the proposed default value for a three-year transitionary period has been deliberately set at a low level to prevent ships using it being advantaged relative to those using power metering. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, Spain, EC, ICS, WSC Further development needed - France No – Japan, Sweden | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> The use of this correction factor would require additional specific data collection such as the number of actual in-use reefers carried or the number of reefers onboard while in port. This complex scheme would result in additional administrative burden for the ships, it would require time-consuming verification process for Administration/recognized organisations, with no guarantee on their capacity to really check the declared additional information in the end. Such complex scheme can be perceived as a loophole for some operators potentially using it to adjust their obtained CII if needed. <Japan> Practical problems such as DCS data collection are likely to occur. Since the power consumption of reefer containers is not monitored individually, but is included as part of the total power consumption including other power consumption, it is difficult to isolate this and count it separately as an increase in the fuel consumption of generators. When using the Default value (2.75kW), it is necessary to count the number of Reefer Containers, but the data and documents required for verification need to be clarified. Of the containers on board, which sometimes exceed 10,000, is there anything other than the shipping company's quotation to prove the number of Reefer containers? Even if there is, isn't it something that requires excessive administrative burden (e.g., thousands of Bills of Lading)? Consideration is needed. <Spain> This factor related to FCelectrical may be used for many ships where there will be a need to deal with cargo, e.g. reefer, a ship using cranes or a ship with hotel needs (see below) |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Russian Federation, Singapore, Spain, EC, ICS, WSC No – France, Sweden | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <France> If decided so, we are of the view that such correction factor should be possible only for ships that have the ability to monitor reefer electrical consumption, by using a kWh meter counter enabling to calculate the specific energy consumption of reefers actually on board. We also note that, in section 2 of Part A of Appendix 1 of draft G5 guidelines, there is no rationale for the determination of Cxc and Cxf values to represent the average electrical consumption of reefers. In absence of records of the actual values of reefers energy consumption, it is currently not possible to develop accurate and reliable – and therefore fair – parameters for implementation of these formulas for ships non-equipped with kWh meter counter. For the two reasons above, this section 2 of Part A of Appendix 1 should be deleted. <Sweden> Reefer electrical consumption is a part of the transport capacity, as a general value included in the baseline for containerships. |  |

For correction factors only, it has been proposed to implement a 75% factor (adjustment factor) that reduces by 3% per year (incentive factor) in order to ensure that there is an incentive to improve the energy efficiency of the equipment or activity that is subject to correction. However it seems that the potential rate of improvement of different equipment may vary, and it was also mentioned for example that steam driven discharge pumps in tankers cannot be powered by shore power.

| Other Questions | Answer |
| --- | --- |
| - Please indicate for this specific correction factor, the adjustment factor to be used. 75% - Chile, Denmark, France, The Netherlands, Sweden, US 85% - Liberia, ICS, WSC 100% - China, Russian Federation Other - Norway | ☐ 75% ☐ 85% ☐ 100% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <China> The electrical load of refrigerated containers may conservatively attain by using the default value. Therefore, 100% factor may apply to avoid double reduction while using the default value. <Norway> We do not agree to an adjustment factor relative to actual consumption |  |
| - Please indicate for this specific correction factor, the incentive factor to be used 3% - Chile, Denmark, Liberia, The Netherlands, Sweden 0% - China, Russian Federation Other – France (reduced over 8 years), US 2% - ICS, WSC | ☐ 3% ☐ 0% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <China> Utilizing shore power may be the only way to reduce emissions of fuel for reefer containers as operators have made a concerted effort to improve reefer container efficiency for many years, investing in improved container designs and optimizing operation. However, the popularity of shore power is constrained by the port's infrastructure, which will be difficult for less developed countries. Therefore, it is difficult to annually improve the reefer container efficiency by 3% that is as same as the incentive factor. <US> The United States would support a larger value than 3%. 25 years is much too long to phase out correction factors for this short-term measure. <WSC> As a general comment, we consider that if a cap and annual reduction is agreed for correction factors (such as 85% cap and a 2% per year reduction) then this should apply to all G5 correction factors equally for which a cap and reduction has been proposed. Equally, if no cap and reduction is to be applied to any of the correction factors for which such has been proposed then this should also be applied to all of this group of correction factors. Otherwise, ship types will be advantaged or disadvantaged and it will distort the rating system, making it impossible to assess the achievement of different segments in reducing AER/cgDIST values. |  |

# FCelectrical Correction Factor for Cargo Handling Electrical Consumption

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Spain, Sweden, UK, US, EC, ICS, INTERCARGO No – France, Japan, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <France> Not sufficient data and evidence is provided by the proponent, in particular for ship types other than bulk carriers <Italy> Substantiated for self-unloading bulk carriers in particular. Not sufficient evidence for other cases. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. - In this proposal, the estimation method using the electric power table is mentioned for the fuel consumption required for cargo handling. However, the CII rating system is an actual fuel consumption evaluation index, which evaluates annual actual fuel consumption. Therefore, subtracting the design value based on the electric power table from the actual fuel consumption is wrong in the first place in terms of the handling of numerical values. At the very least, the meaning of the CII and the reliability of the calculated values themselves may be shaken unless they deal with the actual fuel consumption, which is commensurate with the actual power consumption during cargo handling. - The purpose of the electric power table when designing a new ship is to determine the design points of the engine plant, which is very different from the actual operation. Using the electric power table with a different purpose and making assumptions as a method of estimating actual power consumption can greatly distort the actual situation. - Furthermore, since the electric power table is not a standardized calculation method and is not approved, it is problematic to use it as the basis for calculation. <Norway> We recognize that maintenance of some cargoes will require more energy than others, but don’t think subtracting all fuel used for cargo maintenance is appropriate since some will be reflected in the reference line. In general, we recognize that cargo maintenance energy use should be handled to enable a fair comparison of ships, but we believe that this should not be solved by deducting consumption but rather as more simple correction factors. <Spain> If refrigerated cargo is considered before it needs to be considered now. The following arguments may be used: a ship working with its own handling equipment will likely work in developing countries. Not considering this factor will create disadvantages for these developing countries, therefore the factor would deserve merit, however we feel that it should have been done before. Criteria should have been defined <Sweden> Cargo handling gear should as a mean value be included in baseline and also be included in transport capacity. <EC> Substantiated for self-unloading bulk carriers in particular. Not sufficient evidence for other cases. <ICS> We fully agree with the reason that Intercargo provide for inclusion of this correction factor, i.e. geared cargo ships may use significant amounts of fuel to generate power for cargo operations. Although this fuel consumption is taken account of within the CII calculation, there is no resulting transport work. Hence compared to gearless cargo ships, equivalent geared vessels will be rated more severely under the CII rating system, and without the FCelectrical correction factor this will unfairly penalize the geared vessels. Hence, ICS fully supports the inclusion of this correction factor, and we look forward to reviewing the further supporting data that will be provided at the next round. <INTERCARGO> The information provided allows a solution for the increased fuel consumption due to cargo handling. Without the proposed solution there will be geared vessels will be at a regulatory and commercial disadvantage compared to geared vessels. <Pacific Environment> The purpose of the CII is to obtain an accurate estimate of a ship’s actual operational carbon intensity throughout the year, which must include emissions associated with all fuel consumption, including cargo handling. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Greece, Liberia, The Netherlands, Spain, Sweden, UK, ICS, INTERCARGO 2026 – Brazil, Canada, Denmark, France, Italy, Norway, US, EC | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Greece, Liberia, UK, INTERCARGO No – Denmark, France, Italy, The Netherlands, Norway, Sweden, US, EC, ICS | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> In general we do not support such correction factors as we consider that these ships should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity due to cargo handling gears with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. That said, if the inclusion of such correction factor had to be adopted by the Committee, we suggest the following amendement in the formula: to replace 0.75]-[0.03yi by 0.75-0.75×yi8 This would have the consequence to reduce the period in which correction factors are used in practice to 8 years from 2023 (instead of 25 years as proposed initially) and to sharpen the decrease of their effect over time, so that operators be incentivized to improve more rapidly the specific carbon intensity of ships benefitting from this correction factors and potentially other correction factors adopted at MEPC 78. <The Netherlands> Correction factor on cargo handling is needed but based on the remark that more data will be provided at the next round no complete evaluation can be made at this point in time <Sweden> Cargo handling gear should as a mean value be included in baseline and also be included in transport capacity. <US> The purpose of the CII is to drive reductions in CO2 from the ships, including from cargo handling equipment. Any operation that is excluded will never be improved. Additionally, the stringency of the CII is not such that it warrants the complexity associated with such correction factors. However, we are open to considering this during the 2026 review in the context of more stringent standards. <EC> Addresses the situation for self-unloading bulk carriers, to be demonstrated for other categories <ICS> We look forward to reviewing the further supporting data that will be provided at the next round. |  |
| - Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Denmark, Greece, Italy, Liberia, The Netherlands, UK, EC, ICS, INTERCARGO No – France, Sweden | ☐ Yes ☐ No |
| Comments <Sweden> Cargo handling gear should as a mean value be included in baseline and also be included in transport capacity. <ICS> We look forward to reviewing the further supporting data that will be provided at the next round. <INTERCARGO> Without Correction factors for cargo handling, geared vessels will have a higher AER than similar gearless vessels on similar trades thus pacing them at a regulatory and commercial disadvantage |  |
| - Are the perverse incentives addressed? Yes – Chile, China, Greece, Liberia, The Netherlands, Sweden, UK, INTERCARGO No – Denmark, France, Italy, Norway, EC | ☐ Yes ☐ No |
| If not, please explain your answer <France> In our view, the absence of this correction factor would not generate any perverse incentive. Regarding perverse incentives related to the implementation of the correction factor (such as preventing from making all efforts to reduce the carbon intensity of the concerned ships elsewhere or preventing an earlier use of onshore power supply), we are of the view that they could be limited only if the 𝑦𝑖 factor increases more rapidly, as proposed in question 3 above. <Italy> Addresses the situation for self-unloading bulk carriers, to be demonstrated for other categories. <Norway> There should be incentive to improve cargo maintenance fuel consumption, but this is not adequately addressed. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - China Accurately Correct – Chile, Greece, Liberia, The Netherlands, UK, INTERCARGO Over Correct – Norway, Sweden | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> Insufficient evidence to assess. <Italy> Insufficient evidence to assess. <Sweden> Cargo handling gear should as a mean value be included in baseline and also be included in transport capacity. We therefore, understand the proposal as “over correct” <EC> Insufficient evidence to assess. <ICS> We look forward to reviewing the further supporting data that will be provided at the next round. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Greece, Liberia, UK, INTERCARGO Further development needed – Denmark, Italy, Spain, EC No – France, Sweden | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> The use of this correction factor would require additional specific data collection, resulting in additional administrative burden for the ships. It would require time-consuming verification process for Administration/recognized organisations, with no guarantee on their capacity to really check the declared additional information in the end. Such complex scheme can be perceived as a loophole for some operators potentially using it to adjust their obtained CII if needed. <Sweden> Cargo handling gear should as a mean value be included in baseline and also be included in transport capacity. <EC> Subject to further development of the proposal and clarification on the availability of fuel consumption data. <ICS> We look forward to reviewing the further supporting data that will be provided at the next round. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Greece, Liberia, UK, INTERCARGO No – Denmark, France, Italy, Spain, Sweden, EC | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <France> Such correction factor should be possible only for ships that have the ability to monitor cargo handling systems electrical consumption, by using a kWh meter counter enabling to calculate the actual specific energy consumption of cargo handling systems. We also note that, in paragraph 2.2 of Part B of Appendix 1 of draft G5 guidelines, there is no rationale for the determination of the “Load factor”. In absence of records of the actual values of cargo handling systems energy consumption, it is currently not possible to develop accurate and reliable – and therefore fair – parameters for implementation of these formulas for ships non-equipped with kWh meter counter. For the two reasons above, we would like paragraph 2.2 of Part B of Appendix 1 of draft G5 guidelines to be deleted. <Italy> Subject to further development of the proposal and clarification on the availability of fuel consumption data. <ICS> We look forward to reviewing the further supporting data that will be provided at the next round. |  |

For correction factors only, it has been proposed to implement a 75% factor (adjustment factor) that reduces by 3% per year (incentive factor) in order to ensure that there is an incentive to improve the energy efficiency of the equipment or activity that is subject to correction. However it seems that the potential rate of improvement of different equipment may vary, and it was also mentioned for example that steam driven discharge pumps in tankers cannot be powered by shore power.

| Other Questions | Answer |
| --- | --- |
| - Please indicate for this specific correction factor, the adjustment factor to be used. 75% - Chile, Denmark, France, The Netherlands, Sweden, UK, US 85% - Liberia, ICS 100% - China, INTERCARGO Other - Norway | ☐ 75% ☐ 85% ☐ 100% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <China> While excluding 100% fuel oil consumption for cargo discharge operations, some tankers still presented higher CII and appear to be stuck in CII rating E as some tankers consume large amounts of fuel in the unloading period without corresponding travelled distance. <Norway> We do not agree to an adjustment factor relative to actual consumption |  |
| - Please indicate for this specific correction factor, the incentive factor to be used 3% - Chile, Denmark, Liberia, The Netherlands, Sweden 0% - China, INTERCARGO Other – France (spread over 8 years), US 2% - ICS | ☐ 3% ☐ 0% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <China> Utilizing shore power may be the only way to reduce emissions of fuel for cargo discharge as operators have made a concerted effort to improve the electrical cargo pump efficiency for many years. However, the popularity of shore power is constrained by the port's infrastructure, which will be difficult for less developed countries. Therefore, it is difficult to annually improve the electrical cargo pump efficiency by 3% that is as same as the incentive factor. <US> The United States would support a larger value than 3%. 25 years is much too long to phase out correction factors for this short-term measure. <WSC> As a general comment, we consider that if a cap and annual reduction is agreed for correction factors (such as 85% cap and a 2% per year reduction) then this should apply to all G5 correction factors equally for which a cap and reduction has been proposed. Equally, if no cap and reduction is to be applied to any of the correction factors for which such has been proposed then this should also be applied to all of this group of correction factors. Otherwise, ship types will be advantaged or disadvantaged and it will distort the rating system, making it impossible to assess the achievement of different segments in reducing AER/cgDIST values. |  |

# FCelectrical Correction Factor for Cargo Maintenance Electrical Consumption

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, US, EC, ICS, INTERTANKO No – France, Japan, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Denmark supports the proposal. Different types of cargo requires different amount of energy in order to maintain cooling. <France> We thank the proponents for the analysis provided, but in our view there is still lacking an estimate of the total carbon intensity effort (and therefore CO2 emissions) that would be deducted from the use of this correction factor. This is necessary to assess whether there would be a need to compensate this deduction in the reduction factors (G3 guidelines) and, if so, how this could be done. This would have to be done before the adoption of any correction factors, during the review of the short-term measure if it is not possible before MEPC 78. <Italy> Very specific case of different cooling requirments for different cargos for ships in the same category (ex. Ethylene vs LPG). <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Norway> We recognize that maintenance of some cargoes will require more energy than others, but don’t think subtracting all fuel used for cargo maintenance is appropriate since some will be reflected in the reference line. In general, we recognize that cargo maintenance energy use should be handled to enable a fair comparison of ships, but we believe that this should not be solved by deducting consumption but rather as more simple correction factors. <Sweden> Since Gas carriers can carry different loads like butan, propan with different requirements for cooling, it seems reasonable with correction factors for different loads. <EC> Very specific case of different cooling requirements for different cargoes for ships in the same category (ex. Ethylene vs LPG) <Pacific Environment> The purpose of the CII is to obtain an accurate estimate of a ship’s actual operational carbon intensity throughout the year, which must include emissions associated with all fuel consumption, including cargo cooling. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, UK, EC, ICS, INTERTANKO 2026 – Brazil, Canada, France, Norway, Sweden, US | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Cyprus, Denmark, Greece, Italy, Liberia, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERTANKO No – France, Japan, The Netherlands, Norway, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> In general we do not support such correction factors as we consider that the concerned ships should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity due to cargo maintenance energy consumption with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. That said, if the inclusion of such correction factor had to be adopted by the Committee, we suggest the following amendement in the formula: to replace 0.75]-[0.03yi by 0.75-0.75×yi8 This would have the consequence to reduce the period in which correction factors are used in practice to 8 years from 2023 (instead of 25 years as proposed initially) and to sharpen the decrease of their effect over time, so that operators be incentivized to improve more rapidly the specific carbon intensity of ships benefitting from this correction factors and potentially other correction factors adopted at MEPC 78. <Japan> Simply allowing GHG emissions deductions for cargo reasons would hinder the achievement of GHG reduction targets. <The Netherlands> With regard to the proposed changes from 75% to 85% load factor and from 3% to 2 % annual reduction, suggest that more detailed information is provided by Intertanko to support these proposed changes. <US> The purpose of the CII is to drive reductions in CO2 from the ships, including from cargo cooling equipment. Any operation that is excluded will never be improved. Additionally, the stringency of the CII is not such that it warrants the complexity associated with such correction factors. However, we are open to considering this during the 2026 review in the context of more stringent standards. <EC> Avoids unfair comparison between LPG and ethylene carriers |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERTANKO No - France | ☐ Yes ☐ No |
| Comments |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Cyprus, Denmark, Greece, Italy, Liberia, The Netherlands, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERTANKO No – France, Norway | ☐ Yes ☐ No |
| If not, please explain your answer <France> In our view, the absence of this correction factor would not generate any perverse incentive. Regarding perverse incentives related to the implementation of the correction factor (such as preventing from making all efforts to reduce the carbon intensity of the concerned ships elsewhere or preventing an earlier use of onshore power supply), we are of the view that they could be limited only if the 𝑦𝑖 factor increases more rapidly, as proposed in question 3 above. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct – China, INTERTANKO Accurately Correct – Chile, Denmark, Greece, Italy, Liberia, The Netherlands, Russian Federation, Sweden, UK, EC, ICS Over Correct - Norway | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> Insufficient evidence to assess. <Japan> - Fuel consumption of the re-liquefaction system: When fully loaded, the value can be calculated by using the power consumption PAE calculated by the formula in EEDI Guideline 2.2.5.6.3 and the fuel consumption rate of the generator. To calculate the value for ballast voyage, it is necessary to evaluate the value relative to the fully loaded value based on the electric power table prepared by the shipyard, or to introduce a new calculation formula. - It is difficult to accurately calculate the amount of excess BOG during tank cooldown or gas free operation, but the fuel consumption used by the IGG or GCU, which are operated incidentally, can be measured. <Sweden> What loads should be considered for correction? <EC> Based on actual electricity consumption <INTERTANKO> It is possible to accurately correct by monitoring and recording more parameters, for example sea water pump for general service if used to supply cooling plant water the proportion of flow should represent the proportion of kWh consumed for this pump, as it is easily understood this electrical consumption is difficult to capture and record, as it needs dedicated kWh meter for the pump as well as different water flows measurements, this monitoring complexity may be difficult to verify unless a customized, integrated, sealed and automated system is installed on board which should be approved by RO and Flag state. Another correction methodology would be also acceptable to implement for ships that do not have ability to monitor cargo maintenance electrical consumption as per the current “DRAFT GUIDELINES ON CORRECTION FACTORS AND VOYAGE ADJUSTMENTS FOR CII CALCULATIONS (G5)” PART B. Paragraph 2.2. FCelectrical_cargo handling,j=(Load factor) x Pinstalled x (running hours) x (SFOCAE) Regarding INTERTANKO submission MEPC 76/7/46, hotel load intended in the paper is the total load on generators while the vessel is in idle/anchor situation with no cargo on board which necessitate maintenance/cooling. This load/consumption can be either estimated using vessel load balance as proposed in the paper or can be estimated based on an average of “in service records” over different periods which satisfy idle situation described above. For plants which are provided which shaft generators (SG). Power metering of SG should translate to % of Main Engine power and then to fuel consumption. Methodology for specific plants not addressed in the guidelines should follow similar principles contained in the guidelines, and approved by RO. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Denmark, Greece, Italy, Liberia, Russian Federation, Sweden, UK, EC, ICS, INTERTANKO Further Development Needed – The Netherlands No – France | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> The use of this correction factor would require additional specific data collection, resulting in additional administrative burden for the ships. It would require time-consuming verification process for Administration/recognized organisations, with no guarantee on their capacity to really check the declared additional information in the end. Such complex scheme can be perceived as a loophole for some operators potentially using it to adjust their obtained CII if needed. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Denmark, Greece, Italy, Liberia, The Netherlands, Russian Federation, Sweden, UK, EC, ICS, INTERTANKO No - France | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <France> Such correction factor should be possible only for ships that have the ability to monitor reefer electrical consumption, by using a kWh meter counter enabling to calculate the actual specific energy consumption of cargo maintenance systems. We also note that, in paragraph 2.2 of Part B of Appendix 1 of draft G5 guidelines, there is no rationale for the determination of the “Load factor”. In absence of records of the actual values of cargo maintenance systems energy consumption, it is currently not possible to develop accurate and reliable – and therefore fair – parameters for implementation of these formulas for ships non-equipped with kWh meter counter. For the two reasons above, we would like paragraph 2.2 of Part B of Appendix 1 of draft G5 guidelines to be deleted. |  |

For correction factors only, it has been proposed to implement a 75% factor (adjustment factor) that reduces by 3% per year (incentive factor) in order to ensure that there is an incentive to improve the energy efficiency of the equipment or activity that is subject to correction. However it seems that the potential rate of improvement of different equipment may vary, and it was also mentioned for example that steam driven discharge pumps in tankers cannot be powered by shore power.

| Other Questions | Answer |
| --- | --- |
| - Please indicate for this specific correction factor, the adjustment factor to be used. 75% - Chile, Denmark, France, The Netherlands, Sweden, UK, US 85% - Liberia, ICS, INTERTANKO 100% - China, Russian Federation Other - Norway | ☐ 75% ☐ 85% ☐ 100% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <Norway> We do not agree to an adjustment factor relative to actual consumption <INTERTANKO> As a compromise solution, we suggest that 85% of fuel consumption dedicated to Gas cargo cooling is considered. Since this can be directly measured, a correct deduction would have been for 100% as there is nothing that an existing plant can do to reduce this consumption. |  |
| - Please indicate for this specific correction factor, the incentive factor to be used 3% - Chile, Denmark, Liberia, The Netherlands, Sweden 0% - China, Russian Federation Other – France (Over 8 years), US 2% - ICS, INTERTANKO | ☐ 3% ☐ 0% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <US> The United States would support a larger value than 3%. 25 years is much too long to phase out correction factors for this short-term measure. <INTERTANKO> We suggest an annual 2% improvement. There are no means of improvement for an existing plant to reduce this fuel consumption, so an annual improvement may be difficult to realise with consequences of keeping the cargo at the required temperature and safe pressure. <WSC> As a general comment, we consider that if a cap and annual reduction is agreed for correction factors (such as 85% cap and a 2% per year reduction) then this should apply to all G5 correction factors equally for which a cap and reduction has been proposed. Equally, if no cap and reduction is to be applied to any of the correction factors for which such has been proposed then this should also be applied to all of this group of correction factors. Otherwise, ship types will be advantaged or disadvantaged and it will distort the rating system, making it impossible to assess the achievement of different segments in reducing AER/cgDIST values. |  |

# FCBoiler Correction Factor for Boiler Consumption

This correction factor covers cargo heating and cargo discharge pumps, and the evidence provided also covers FCothers – that is pumps equipped with their own generator.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Belgium, Brazil, Canada, Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Norway, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA No – France, Japan, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Denmark supports this proposal <Finland> Correction factors for boiler consumption have been proposed in MEPC 76/7/26 submitted by INTERTANKO and in MEPC 76/7/36 submitted by IPTA. In Round 3 of the CG China and IPTA have provided further information on this issue. <France> We thank the proponents for the analysis provided, but in our view there is still lacking an estimate of the total carbon intensity effort (and therefore CO2 emissions) that would be deducted from the use of this correction factor. This is necessary to assess whether there would be a need to compensate this deduction in the reduction factors (G3 guidelines) and, if so, how this could be done. This would have to be done before the adoption of any correction factors, during the review of the short-term measure if it is not possible before MEPC 78. <Italy> Demonstrated disproportionate impact for cargo heating and discharging. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. - The energy used to hold cargo during transportation is part of the maritime transportation business, and separating only this and exempting it is incompatible with the direction in which maritime transportation should aim to reduce GHG emissions. <Norway> We recognize that maintenance of some cargoes will require more energy than others, but don’t think subtracting all fuel used for cargo maintenance is appropriate since some will be reflected in the reference line. In general, we recognize that cargo maintenance energy use should be handled to enable a fair comparison of ships, but we believe that this should not be solved by deducting consumption but rather as more simple correction factors. <Spain> This correction should have been considered in the reference lines for oil (product tankers) and chemical tankers. It would have been better to consider it there based on averages or historic data <Sweden> Supported, since different quality of oils require different fuel consumption related to handling, heating etc. <EC> Demonstrated disproportionate impact for cargo heating and discharging. <ICS> We fully support the inclusion of this correction factor, and strongly recommend that it should also be applied to fuel storage units. <IPTA> IPTA has supplied Additional information to further confirm the need for this correction to maintain a level playing field and to avoid penalizing ships carrying heated cargoes. <Pacific Environment> The purpose of the CII is to obtain an accurate estimate of a ship’s actual operational carbon intensity throughout the year, which must include emissions associated with all fuel consumption. Emissions from cargo heating and tank washing still have an impact on the climate and count towards the actual carbon intensity of the ship. While we remain opposed to any exclusion or correction factor for any ship, support from the broader group may rest on understanding what proportion of total fuel consumption is proposed to be excluded? |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Belgium, Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Spain, Sweden, UK, EC, ICS, INTERTANKO, IPTA 2026 – Brazil, Canada, France, Norway, US | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Sweden, UK, EC, ICS, INTERTANKO, IPTA No – Belgium (see comments below), France, Norway, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Belgium> We believe a slightly smaller reduction of FcBoiler would be more in line with reducing overall emission numbers Especially in the case of LNG carriers, removing the boiler completely is clearly not incentivising the most optimal use of LNG transportation which should be done with limited waiting times and avoidance of loading in a warm tank or loading warm cargo. If this is accepted for Boilers then there is no incentive to optimise this or use reliquification plants. We therefore would propose a limit where the Fboiler can be taken into account for 75% and require an annual reduction similar to the Felectric used for other types of vessels. This would encourage LNG carriers to optimise their planning and give priority to using reliquification as this will emit less CO2 compared to pure burning. At the same time, it would ensure that new vessels delivered are equipped with reliquification units. Having a linear regression over the years will incentivize fleet renewal in a similar way as is being promoted for other vessel types. <China> This correction factor may also cover water heating for tank washing <Denmark> Will reduce the unfairness between cooled/heated cargo <France> In general we do not support such correction factors as we consider that the concerned ships should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity due to cargo maintenance energy consumption with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. Besides, we do not agree with the following assumption made by the proponents in the justification of the CF: “Shipowners expect that a relatively large number of vessels will go from B/C-rating to D/E-rating solely due to the cargo consumption, which will of course have significant impact on the trade of this cargo”. We don’t really see what such “significant impact” would be, as this situation would concern all tankers operating for similar trades, for which the demand would not be changed. There would not be market distorsion as not other types of ships could replace ships that would have a relatively worse CII due to this specific operational profile. That said, if the inclusion of such correction factor had to be adopted by the Committee, we suggest the following amendement in the formula: to replace 0.75]-[0.03yi by 0.75-0.75×yi8 This would have the consequence to reduce the period in which correction factors are used in practice to 8 years from 2023 (instead of 25 years as proposed initially) and to sharpen the decrease of their effect over time, so that operators be incentivized to improve more rapidly the specific carbon intensity of ships benefitting from this correction factors and potentially other correction factors adopted at MEPC 78. <US> The purpose of the CII is to drive reductions in CO2 from the ships, including from boilers. Any operation that is excluded will never be improved (e.g. better efficiency, alternative fuels, shore power, better operating procedures, etc). Also, a D rating must occur for 3 consecutive years to trigger corrective actions. This helps address the issue of year-to-year variability. However, we are open to considering this during the 2026 review in the context of more stringent standards. <IPTA> Further evidence has been provided. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Belgium, Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Sweden, UK, EC, ICS, INTERTANKO, IPTA No - France | ☐ Yes ☐ No |
| Comments <Finland> Based on the information provided by China, INTERTANKO and IPTA in their answers to question Q3.b in Round 3 of the CG, boiler fuel oil consumption has a significant negative impact on ships. <Sweden> Supported, since different quality of oils require different fuel consumption related to handling, heating etc. <EC> Reduces unfairness between heated/non-heated cargoes, and number of discharge operations. <IPTA> Yes in particular on ships carrying and tank cleaning from heated cargo |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Sweden, UK, EC, ICS, INTERTANKO, IPTA No – Belgium, France, Norway | ☐ Yes ☐ No |
| If not, please explain your answer <Belgium> See our answer for question 3 <France> In our view, the absence of this correction factor would not generate any perverse incentive. Regarding perverse incentives related to the implementation of the correction factor (such as preventing from making all efforts to reduce the carbon intensity of the concerned ships elsewhere or preventing an earlier use of onshore power supply), we are of the view that they could be limited only if the 𝑦𝑖 factor increases more rapidly, as proposed in question 3 above. <IPTA> There are no perverse incentives as we are suggesting only to mitigate 85% of the boiler consumption. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - China Accurately correct – Chile, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Sweden, UK, EC, ICS, INTERTANKO, IPTA Over correct – Belgium (addressing GCU), Norway | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> Insufficient evidence to assess <EC> Actual fuel consumption <IPTA> The correction factors are as accurate as they can be with highly variable voyages |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Belgium, Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Sweden, UK, EC, ICS, INTERTANKO, IPTA Further development needed No - France | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> The use of this correction factor would require additional specific data collection, resulting in additional administrative burden for the ships. It would require time-consuming verification process for Administration/recognized organisations, with no guarantee on their capacity to really check the declared additional information in the end. Such complex scheme can be perceived as a loophole for some operators potentially using it to adjust their obtained CII if needed. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Belgium, Chile, China, Denmark, Finland, Greece, Italy, Liberia, The Netherlands, Sweden, UK, EC, ICS, INTERTANKO, IPTA No - France | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <France> We note that, in paragraph 1 of Part B of Appendix 1 of draft G5 guidelines, there is no rationale for the use of 75% of the boiler fuel consumption (FCBoiler,j) to calculate the part of the energy required by the boiler for cargo heating needs. |  |

For correction factors only, it has been proposed to implement a 75% factor (adjustment factor) that reduces by 3% per year (incentive factor) in order to ensure that there is an incentive to improve the energy efficiency of the equipment or activity that is subject to correction. However it seems that the potential rate of improvement of different equipment may vary, and it was also mentioned for example that steam driven discharge pumps in tankers cannot be powered by shore power.

| Other Questions | Answer |
| --- | --- |
| - Please indicate for this specific correction factor, the adjustment factor to be used. 75% - Belgium, Chile, Denmark, France, The Netherlands, Sweden, UK, US 85% - Liberia, ICS, INTERTANKO, IPTA 100% - China Other - Norway | ☐ 75% ☐ 85% ☐ 100% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <China> While excluding 100% fuel oil consumption for cargo heating and cargo discharge operations, some tankers still presented higher CII and appear to be stuck in CII rating E. <Norway> We do not agree to an adjustment factor relative to actual consumption |  |
| - Please indicate for this specific correction factor, the incentive factor to be used 3% - Chile, Denmark, Liberia, The Netherlands, Sweden, IPTA 0% Other – France (Over 8 years), US 2% - ICS, INTERTANKO | ☐ 3% ☐ 0% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <China> The boiler or diesel efficiency in technology and management has improved for many years. There is no option left to annually improve their efficiency by 3% that is as same as the incentive factor. <US> The United States would support a larger value than 3%. 25 years is much too long to phase out correction factors for this short-term measure. <INTERTANKO> We suggest a 2% annual improvement. Cargo heating installations are supported by software minimising the fuel consumption during voyage. The efficiency of the system is based on continuous monitoring of the difference between the cargo and the ambient temperatures and the allowance of temperature variation for which the cargo remains safe. Therefor potential for improvement is limited. <WSC> As a general comment, we consider that if a cap and annual reduction is agreed for correction factors (such as 85% cap and a 2% per year reduction) then this should apply to all G5 correction factors equally for which a cap and reduction has been proposed. Equally, if no cap and reduction is to be applied to any of the correction factors for which such has been proposed then this should also be applied to all of this group of correction factors. Otherwise, ship types will be advantaged or disadvantaged and it will distort the rating system, making it impossible to assess the achievement of different segments in reducing AER/cgDIST values. |  |

# FCBOG Correction Factor for Boil Off Gas

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Denmark, Greece, Italy, Liberia, Malaysia, The Netherlands, Norway, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERTANKO No – France, US, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Denmark supports the proposal as similar to cargo maintenance proposal for other types of gas carries. Correction factor only for reliquefaction of BOG. Not for GCU. <Italy> Case demonstrated but deduction of GCU boil off must be matched by reporting, which is not consistently done today. <Malaysia> Correction factor for handling LNG - cooling and boil off is important for LNG tanker in managing the Boil off Gas (BOG) during voyage and heavy weather. For managing the BOG during the laden passage, the LNG tanker need to go to full speed to consume BOG to control tank pressure. EPL or SHApoli cannot be used during this period. Prior arrival, tank terminal has requirement to control the tank pressure and temperature. <Norway> In general, we are not in favor of specific correction for different technologies. LNG carriers are part of the same segment and subject to the same cargo handling requirements. The intention of the CII is to incentivize ships with low carbon intensity and the ships with best solutions should get the best CII. We do recognize the safety aspect for handling boil off gas, and propose that this matter is further considered towards 2026. <Spain> The current gas contention systems have a very reduced boil off. This boil off comes with the choice to have decided for a propulsion system using gas. We don’t favour this issue right now but in the future. Furthermore the issue of Nitrogen content in boil off gas is similar to the issue of sludge in HFO. This aspect should have been considered beforehand. <Sweden> Seems reasonable since there are different ways to solve the problem resulting in different fuel consumption. <UK> We support. However, we should put in place measures to ensure that energy recovery (via reliquefaction or another means) is required for all new carriers (if not already), and that all measures are taken seriously to prevent wasted energy. Increase in speed during passage should be assessed and balanced against how that plays against the losses incurred (efficiency of reliquefaction versus journey time decrease) and the number of ships built for a given route. Overall efficiency (system wide) should be pushed for. Additionally, in looking at this in a system way, we should also look at how to drive improved commercial operation of the carriers, as this affects waste too. There is the need to reduce absolute emissions rather than just drive a good CII score. <EC> Case demonstrated but deduction of GCU boil off must be matched by reporting, which is not consistently done today <Pacific Environment> The purpose of the CII is to obtain an accurate estimate of a ship’s actual operational carbon intensity throughout the year, which must include emissions associated with all fuel consumption. Emissions from fuel used to maintain cargo pressure and temperature still have an impact on the climate and count towards the actual carbon intensity of the ship. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Greece, Liberia, Malaysia, Russian Federation, UK, ICS, INTERTANKO 2026 – Brazil, Canada, Denmark, France, Italy, The Netherlands, Norway, Spain, Sweden, EC | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Greece, Liberia, Malaysia, Russian Federation, Spain, UK, ICS, INTERTANKO No – Denmark, France, Italy, The Netherlands, Norway, Sweden, US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <Belgium> We believe a slightly smaller reduction of FcBoiler would be more in line with reducing overall emission numbers Especially in the case of LNG carriers, removing the boiler completely is clearly not incentivising the most optimal use of LNG transportation which should be done with limited waiting times and avoidance of loading in a warm tank or loading warm cargo. If this is accepted for Boilers then there is no incentive to optimise this or use reliquification plants. We therefore would propose a limit where the Fboiler can be taken into account for 75% and require an annual reduction similar to the Felectric used for other types of vessels. This would encourage LNG carriers to optimise their planning and give priority to using reliquification as this will emit less CO2 compared to pure burning. At the same time, it would ensure that new vessels delivered are equipped with reliquification units. Having a linear regression over the years will incentivize fleet renewal in a similar way as is being promoted for other vessel types. <France> We do not support such correction factor as LNG carriers have their own reference line, already taking into count specificities of their operations and constraints compared to other ship types. The need for any correction factor should should be justified by heterogeneity of design features, cargo needs or operating profile that are not sufficiently addressed, which is not the case here as LNG carriers are a specific ship type. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. <Italy> While the management of excess boil off gas is variable and unavoidable, it is difficult to assess at this stage whether the proposal would address the issue, especially given the ambiguity of current reporting requirements on GCU consumption. <Sweden> More details must be defined how correction per method and connected fuel consumption applies <UK> Yes, though comments above apply in terms of new builds using reliquefaction or another means of recovering energy. <US> There is no reason that this should be treated differently than any other fuel consumed aboard the vessel. <EC> While the management of excess boil off gas is variable and unavoidable, it is difficult to assess at this stage whether the proposal would address the issue, especially given the ambiguity of current reporting requirements on GCU consumption. <INTERTANKO> We agree the issue is adequately addressed however we have the following minor comment: a couple of factors proposed related to the use of reliquefication plants would fit better under FCelectrical,j in accordance with paragraph 4.2 of the G5 draft instead of being included under FCBOG,j., this should not affect the methodology of the solution. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Denmark, Greece, Italy, Liberia, Malaysia, The Netherlands, Russian Federation, Spain, Sweden, UK, EC, ICS, INTERTANKO No – France, Norway, US | ☐ Yes ☐ No |
| Comments <France> While the management of excess boil off gas is variable and unavoidable, it is difficult to assess at this stage whether the proposal would address the issue, especially given the ambiguity of current reporting requirements on GCU consumption. <Greece> It is debatable how a single CII reference line can cover the design diversity of the LNG carriers that are currently in operation. It is, therefore, important to apply voyage adjustments for each type of LNG carrier in order not to penalize the vessels which are involved frequently or constantly in less efficient voyages and operations. <Malaysia> For steam turbine LNG tankers, the handling LNG - cooling and boil off is important in managing the Boil off Gas (BOG) during voyage and heavy weather. For managing the BOG during the laden passage, tanker need to go to full speed to consume BOG due to control tank pressure, EPL or SHApoli cannot be used during this period. Prior arrival, tank terminal has requirement to control the tank pressure and temperature. This excess steam dumped in MT and this fuel consumption should be deducted. Using the KYMA System excess steam dump is accounted as much as 10% to 15% of total FOC as excess BOG and steam generated is dumped for the purpose of controlling cargo tank pressure and temperature for steam turbine LNG Tankers. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Greece, Liberia, Malaysia, Russian Federation, Spain, Sweden, UK, ICS, INTERTANKO No – France, Norway, US | ☐ Yes ☐ No |
| If not, please explain your answer <France> In our view, the absence of this correction factor would not generate any perverse incentive. Regarding perverse incentives related to the implementation of the correction factor (such as preventing from making all efforts to reduce the carbon intensity of the concerned ships elsewhere), we are of the view that they could be limited only if the 𝑦𝑖 factor increases more rapidly, as proposed in question 3 above. <Norway> We are concerned that the correction will incentivize dumping steam or burning the BOG in the GCU instead of using it for propulsion purposes. <US> The proposal would create a disincentive to minimizing BOG. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct – China, Denmark Accurately Correct – Chile, Greece, Liberia, Malaysia, Russian Federation, Sweden, UK, ICS, INTERTANKO Over Correct – US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> Insufficient evidence to assess <Malaysia> As a general comment, we believe that a note should be added to the G5 guidelines: Note: “When LNG liquid volume measurement is converted to calculate Boil-off Gas mass, the Liquid Methane density of 422 kg/m³ should be used” The rational for additional note: LNG Boil-off Gas is mainly composed of Methane because LNG is transported as Methane at boiling point, while other heavier hydrocarbons are contained as minority in the LNG have higher boiling point than Methane which means it does not evaporate and remain at liquid state <UK> Any meters and systems performing measurements should be assessed by the administrations to ensure they are installed, remain suitably calibrated, are suitably reliable, and suitably available. This will ensure that there are not over or under estimates. In particular where it states “Electric load of combined reliq system and fuel gas supply: Conversion of reliquefied quantity to electric load using maker’s data and subject to approval from the verifier.”, care must be taken here to agree suitable figures based on the reality of operations, not simply a potentially optimistic set of data from the maker in lab/test conditions. <US> The correction factor is not necessary. <EC> Insufficient evidence to assess <INTERTANKO> As a general comment, we believe that a note should be added to the G5 guidelines: Note: “When LNG liquid volume measurement is converted to calculate Boil-off Gas mass, the Liquid Methane density of 422 kg/m³ should be used” The rational for additional note: LNG Boil-off Gas is mainly composed of Methane because LNG is transported at Methane boiling point, while other heavier hydrocarbons contained as minority in the LNG have higher boiling point than Methane which means do not evaporate and remain at liquid state. Nitrogen is also present in the Boil-off Gas as it has lower boiling point than Methane, however, does not participate in CO2 generation. For practicality of calculation, the Nitrogen content shouldn’t be considered, even though the calculated CO2 emissions are higher than the actual. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Greece, Liberia, Malaysia, Russian Federation, Sweden, ICS, INTERTANKO Further development needed – Denmark, Italy, The Netherlands, EC No – France, UK, US | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> The use of this correction factor would require additional specific data collection, resulting in additional administrative burden for the ships. It would require time-consuming verification process for Administration/recognized organisations, with no guarantee on their capacity to really check the declared additional information in the end. Such complex scheme can be perceived as a loophole for some operators potentially using it to adjust their obtained CII if needed. <UK> There is no data to use for verification <US> This proposal would add a great deal of complexity to the CII, for little value. <EC> Subject to further development of reporting requirements |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Greece, Liberia, Malaysia, Russian Federation, ICS, INTERTANKO No – Denmark, France, Italy, Sweden, UK, US, EC | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <France> This would have to be specified in the G5 guidelines (there is nothing for the moment) <Sweden> More details about different techniques must be developed. <UK> There is no data to use for verification <US> Requires additional procedures and equipment, but no real way to verify the proposed correction. |  |

For correction factors only, it has been proposed to implement a 75% factor (adjustment factor) that reduces by 3% per year (incentive factor) in order to ensure that there is an incentive to improve the energy efficiency of the equipment or activity that is subject to correction. However it seems that the potential rate of improvement of different equipment may vary, and it was also mentioned for example that steam driven discharge pumps in tankers cannot be powered by shore power.

| Other Questions | Answer |
| --- | --- |
| - Please indicate for this specific correction factor, the adjustment factor to be used. 75% - Chile, Denmark, France, Malaysia, The Netherlands, Sweden, UK, US 85% - Liberia, ICS, INTERTANKO 100% - China, Russian Federation | ☐ 75% ☐ 85% ☐ 100% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <Malaysia> As a compromise solution, consumption dedicated to BOG handling is not entirely accounted for, only 75% of this consumption is considered to support adoption of this factor. A fair solution would have be 100%, meaning no factor adjustment, as there is nothing that an existing LNG plant can do to reduce this consumption. <INTERTANKO> We suggest discounting 85% if the fuel consumption dedicated to BOG handling. The BOG amount beyond of what is used for onboard combustion can be precisely measured and thus can be totally deducted. However, we agree with the simplicity of having one single correction factor formula accounting the fuel consumption used for other similar cargo handling operations. Therefore, 85% is n our view a more fair approach. |  |
| - Please indicate for this specific correction factor, the incentive factor to be used 3% - Chile, Denmark, Liberia, Malaysia, The Netherlands, Sweden, UK 0% - China Other – France, US 2% - ICS, INTERTANKO | ☐ 3% ☐ 0% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. <Malaysia> As a compromise solution, 3% is proposed to support this specific factor adoption, a fair solution would have been 0%, as there is nothing that an existing plant can do to reduce this consumption. <US> The United States would support a larger value than 3%. 25 years is much too long to phase out correction factors for this short-term measure. <INTERTANKO> As a compromise solution, 2% is proposed to support this specific factor adoption. A fair solution would have been 0%, as all BOG in excess of what is needed onboard is a function of operational conditions with no options to have an annual constant improvement/reduction. <WSC> As a general comment, we consider that if a cap and annual reduction is agreed for correction factors (such as 85% cap and a 2% per year reduction) then this should apply to all G5 correction factors equally for which a cap and reduction has been proposed. Equally, if no cap and reduction is to be applied to any of the correction factors for which such has been proposed then this should also be applied to all of this group of correction factors. Otherwise, ship types will be advantaged or disadvantaged and it will distort the rating system, making it impossible to assess the achievement of different segments in reducing AER/cgDIST values. |  |

# FCBOG,Nitrogen Correction Factor for Boil Off Gas

Correction factor to deal with varying amount of nitrogen in LNG cargoes which is normally included in total fuel consumption.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO No – Norway, US, EC | ☐ Yes ☐ No |
| Comments <Malaysia> With regards to Nitrogen in boil-off gas, all LNG carriers which load cargoes containing nitrogen are affected and should be deducted from the total fuel consumption. <Norway> Nitrogen content is an issue, but we do not think it should be solved through a correction on the CII, but rather through the fuel consumption reporting. The DCS data and the uncorrected CII should accurately reflect the CO2 emissions, and as such it needs to be corrected at reporting. The LNG conversion factor of 2.75 assumes pure methane, and as such the combusted amount should not include nitrogen (ignoring that there are other minor non-methane components to LNG). <EC> Specific case not demonstrated |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO 2026 | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO No – US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <EC> Not sufficient evidence to assess the rationale and impact of this proposal, which could lead to large deductions from fuel consumption. It is also not clear how it relates to the current fuel reporting practice. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO No – US, EC | ☐ Yes ☐ No |
| Comments <Malaysia> Malaysian LNG carriers which frequently loads cargoes from Bintulu (Nitrogen content about 0.2%mol in LNG cargo) will consume more nitrogen in the BOG while an LNG carrier which loads cargoes from Sabine Pass, USA (Nitrogen content less than 0.1%mol) will consume almost pure methane. <US> LNG carriers can use the BOG as fuel, making it like any other fuel on the ship. This should be a very small portion of fuel used, making the nitrogen issue too small to warrant the complexity of including this correction factor in the CII. |  |
| 5. Are the perverse incentives addressed? Yes – Greece, Liberia, Malaysia, UK, ICS, INTERTANKO No - EC | ☐ Yes ☐ No |
| If not, please explain your answer |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO Over Correct | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <UK> Administrations must ensure that measures are taken to ensure accuracy of data. <EC> Insufficient evidence to assess |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO Further development needed No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <EC> Insufficient evidence to assess |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes - Greece, Liberia, Malaysia, UK, ICS, INTERTANKO No | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <EC> Insufficient evidence to assess |  |

For correction factors only, it has been proposed to implement a 75% factor (adjustment factor) that reduces by 3% per year (incentive factor) in order to ensure that there is an incentive to improve the energy efficiency of the equipment or activity that is subject to correction. However, it seems that the potential rate of improvement of different equipment may vary, and it was also mentioned for example that steam driven discharge pumps in tankers cannot be powered by shore power.

| Other Questions | Answer |
| --- | --- |
| - Please indicate for this specific correction factor, the adjustment factor to be used. 75% 85% 100% - INTERTANKO | ☐ 75% ☐ 85% ☐ 100% ☐ Other (please indicate) |
| <INTERTANKO> The formula proposed for FCBOG,Nitrogen Should be considered in the context of the CII formulas, as the 0.75 will be present in both numerator and denominator which will simplify Nitrogen is an inert gas and does not create C02 emissions when mixed with gas flow being burnt in engines, boilers, and Gas Combustion Units (GCU). The above explains why 100% of Nitrogen should be excluded from Total gas consumption. |  |
| - Please indicate for this specific correction factor, the incentive factor to be used 3% 0% - INTERTANKO Other | ☐ 3% ☐ 0% ☐ Other (please indicate) |
| Please provide further explanation in this box if needed. |  |

# CFPT Port Time Correction Factor for Cruise Passenger Ships

Note that this port time correction factor is for all cruise passenger ships, and not only those with non-conventional propulsion.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, Cyprus, France, Italy, Liberia, The Netherlands, Norway, Singapore, Spain, Sweden, UK, CLIA, EC, ICS No – Denmark, Japan, US, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> This proposal needs further consideration <France> Without prejudice to the necessity to assess and take into account the potential effects of such CF, we can support in principle the development of a port time correction factor for cruise passenger ships in order to avoid unduly penalizing cruise ships that spend a lot of time at berth, and even to avoid incentivizing these ships to extend hours underways to improve their CII and thus increasing their absolute GHG emissions (assuming that a ship at berth consumes much less fuel than a ship underway). <Italy> Italy is of the opinion that the issue should be discussion bearing in mind the need to consider all green technologies that can be used. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. - In particular, this proposal calls for exempting port berthing periods, which is unacceptable because it will reduce the motivation such as expanding the use of shore power, and would pour cold water on the overall shipping effort to reduce GHG emissions. - In addition, there have been cases in the past where the length of berthing periods has increased or decreased or been prolonged, such as prolonged waiting offshore due to novel coronavirus infections, port congestion, and long-term mooring during economic downturns, which are unique to maritime transport, and there is little basis for specifically exempting passenger cruise ships. <Spain> This issue needs to be addressed, however the proposal from CLIA seems to be very conservative. Even if the EEDI was developed for diesel electric propulsion cruise ships and the CII includes all cruise ship this aspect deserve merits. More information is needed <Sweden> Alternative 10.2 with minimum 20% not travelling can be supported and the time exceeding this number can be corrected for. All ships have time in harbor, at anchor. <UK> The UK supports a change to the current equation for cruise ships. However, the port correction factor is an interim fix that does not address the fundamental misinterpretation of the base equation. The base equation is essentially (CII=emissions/work_done). Work done is distance x capacity for most ships that take cargo from A to B. For cruise ships work_done is better represented by passenger hours with no inclusion of distance travelled. It is appreciated this will not allow comparison with other ships but the cruise industry is sufficiently different for this not to be possible anyway. The proposed amendment more cleanly represents the intent of the original underlying equation. It should be noted that further work and analysis is still required. In the meantime, the port correction factor can be accepted until a better definition of work done is agreed. Any correction factor applied in this case should be considered interim. <US> Excluding port operation would remove the incentive from the CII to control a significant share of the emissions from cruise ships. This correction would only apply for ships with above average time in port, leading to a weakening of the CII standards. <CLIA> Cruise passenger ships have high port time and hotel load compared to many other ship types. These vessels are disproportionately impacted by CII as distance is in the denominator and the calculation doesn’t take into account operational efficiencies in hotel operations. This issue deserves further consideration. If implemented in its current form, CII would actually result in increased absolute emissions as ships increase distance and thereby fuel emissions in an effort to meet CII requirements. Apart from emissions considerations, other impacts could include an economic impact on ports based on their location and other repercussions which are decoupled from the goals of the IMO. <EC> Perverse incentives leading to potential increase in total GHG emissions demonstrated <ICS> We fully agree with the reason that CLIA provide for inclusion of this correction factor, i.e. cruise ships spend significant periods of time in port with high hotel load. Due to these periods not counting towards transport work, the CII rating of cruise vessels will be unfairly impacted, unless the CFPT correction factor may be applied. <Pacific Environment> We are concerned that excluding or “correcting” fuel consumption and emissions when no distance is travelled, including in port, is counterproductive to decarbonizing the sector and perpetuates the negative human health impacts of burning bunker fuels near port communities, many of which are disproportionately negatively affected by in-port and near-port pollution. If fuel consumption and emissions at berth, at anchor, or when the ship is otherwise stationary are excluded or “corrected”, what incentive is there for cruise passenger ships to invest in zero-emission technologies or shore power connections for use in port? Fuel consumption and emissions that occur when the ship is not moving are important to reduce and eliminate because they are wasteful and harmful: no transport work is being done, and yet fuel is still being consumed, GHGs and black carbon are emitted, air pollution is emitted, and water pollution is emitted for ships with scrubbers. Worse, this pollution is occurring near human populations and in shallow waters, where their negative consequences are magnified. For cruise passenger ships that wish to avoid being penalized for fuel consumed in port, they need only to invest in shore power connections on the ship and, if not already available, at the ports at which they call. Keeping 100% of in-port fuel consumption in the CII calculation promotes these kinds of investments in emissions reduction technologies, excluding or correcting them does not. We therefore strongly oppose this proposal. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Cyprus, France, Italy, Liberia, The Netherlands, Norway, Singapore, Spain, UK, CLIA, EC, ICS 2026 – Brazil, Canada, Sweden | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes - Chile, Liberia, Norway, Singapore, Sweden, CLIA, ICS No – Cyprus, Denmark, France, Italy, The Netherlands, Spain, UK, US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> Assuming that almost all cruise passenger ships would benefit from it (i.e. that almost all of them spend more than 20% of their time at berth), we think that the CF as proposed would result in an improvement of the attained CII for the whole ship category of cruise ships with non-conventional propulsion, as the reference line of this category has not been designed taking into account this CF. As an example, we can imagine that if cruise ships spend on average 30% of their time in port, this would result in a CFPT of 1.10 on average, resulting roughly on average in a superior rating score for the category taken as a whole. This would be unfair for the other ship categories, which would not benefit such “automatic” improvement of their CII, and most of all it would delay or lower the efforts needed by cruise shipowners to comply, finally undermining the effectiveness of the measure. However, we recognize that the CF proposed by CLIA and ICS constitute a good basis and can be improved to remedy the defects mentioned. <Italy> Further discussion is needed as regards the solution proposed, both in terms of the 20% threshold and the correction to distance approach. <The Netherlands> We agree that the issue should be addressed. But the current proposal over compensate for the time spend at berth. More data and calculations are needed to assess a fair adjustment factor for time spend in port. It should incorporate an incentive to also optimize the energy consumption or the pick up of cold ironing in port. <US> This paper estimates that cruise ships spend 22-30% of their time in port, representing about 15% of their CO2 emissions. As ships reduce emissions underway to meet declining CII standards, this percentage could increase substantially. This is an important part of the operation of cruise ships, is included in the CII baseline, and should be targeted for GHG reduction similar to operation underway. This proposal ignores the availability of technology such as, improved engine efficiency shore power, and low/zero carbon fuels for reducing carbon emissions at port. It also ignores practices available to minimize power use during hotelling. <CLIA> The proposed correction factor adequately addresses the port time penalty for passenger ships based on the currently available parameters within IMO’s Data Collection System. Although the correction factor is adequate in the short-term, additional metrics will have to be considered in the future to fully align with IMO’s long-term goals. <EC> Further discussion is needed as regards the solution proposed, both in terms of the 20% threshold and the correction to distance approach |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Cyprus, Denmark, France, Italy, Liberia, The Netherlands, Norway, Singapore, Sweden, UK, CLIA, EC, ICS No - US | ☐ Yes ☐ No |
| Comments <The Netherlands> There is a significant impact. However once ships can connect to shore power facilities the effect of fuel consumption in port is minimised. This should also be addressed. <US> While, on the margin, the AER metric could create an incentive to operate at cruise more often, the more likely incentive will be for cruise ships to minimize their emissions at port. <CLIA> Under the current proposal a cruise ship would be incentivised to reduce port time and increase distance travelled to optimise for CII leading to an increase in absolute CO2 emissions. This is shown in the chart below where green areas indicate the positions optimized for best CII performance as currently calculated without inclusion of the port time correction factor. |  |
| 5. Are the perverse incentives addressed? Yes - Chile, Liberia, Norway, Singapore, Sweden, UK, CLIA, ICS No – Cyprus, Denmark, France, Italy, The Netherlands, US, EC | ☐ Yes ☐ No |
| If not, please explain your answer <France> Not enough in the way the CF is proposed – see comment above <The Netherlands> The current proposal over compensate for time spend in port. This issue should firstly be addressed before applying this factor. As other factors the factor should incentives the efficient use of energy in port and the move towards cleaner zero emission solutions. <US> No. If anything, this proposal provides the perverse incentives of 1) spending more time at port and 2) removing incentive for minimizing fuel consumption at port (due to an over-correction) <CLIA> The perverse incentive arises from the impact of time spent in port. The correction factor aims to decouple port time from CII rating, a cruise ship should not be penalized for longer or more frequent port stays and reduced absolute emissions. From the example ship provided below the corrected CII (orange line) is relatively independent of port time showing the perverse incentive has been minimized using available DCS data. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - UK Accurately Correct - Chile, Liberia, Norway, Singapore, Sweden, CLIA, ICS Over Correct – France, The Netherlands, US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> Option 1: Adjust the reference lines (and subsequently the rating bands) for the cruise ships having non conventional propulsion. - This could be done quite simply and rapidly in applying the correction factor (with the average time spent at berth on year 2019, using the “hours underway” DCS data) to the 2019 fuel consumption data used to calculate the reference line. Option 2: Recalibrate the Correction Factor for all cruise ships. - We propose the following adjustment: instead of assuming a minimum of 20% of time spent at berth, we propose to determine the average time spent at berth on year 2019 (or the in reverse the average time underway) of the whole category, using 2019 DCS data, and to delete any threshold to its application. The CF would look like the following: CFPT= 1+Haverageatsea8760-Hoursunderway8760 with Haverageatsea being the average time underway by cruise ships with non conventional propulsion in 2019. For example, if it is determined that cruise ships spend on average 30% of their time at berth, hence 70% of their time at sea, the correction factor will be the following: CFPT=1.7-Hoursunderway8760 - It is important to note that this CF would apply as is to all cruise ships, even those which spent less than 30% of their time at berth during the year. Therefore, cruise ships spending more than the average time at berth will have a CFPT of more than 1 and cruise ships spending less than the average time at berth will have a CFPT of less than 1. In this manner, there is no more any incentive to spend more time at sea to decrease the attained CII. - Considering that the average of all the CFPT actually applied to cruise ships will be around 1, this option has significant advantages: there is no need to change the reference line, and will not undermine the overall environmental effectiveness of the measure. <Sweden> Details must be defined how attained index should be calculated <US> Cruise ships are unique in that they have high auxiliary loads, both underway and in port. These ships often have several main engines that power generators, which in turn supply power for both auxiliary loads and propulsion. A cruise ship may only operate one engine in port, but several engines while underway. Two options are put forward in this proposal: 1) a correction factor based on time in port and 2) a proposed correction of 15 nm/hr for port operation The first option is intended to zero out GHG emissions from cruise ships at port, where the port operations exceed 20% of total time of ship operations. By applying this factor, the amount of GHG emissions excluded is based on average emission rates rather than emission rates only at port. Because cruise ships would be expected to consume significantly more fuel underway than at port, this is a significant over correction for ships that have more than average time at port. By using % of hours underway in the equation, the proposed correction factor would inappropriately treat time that a ship is laid up for repair, or other reasons, as port operation. The second option of applying 15nm/hr as a vessel speed for port operation implies that a cruise ship at port should consume the same amount of fuel as a cruise ship that is underway at 15 knots. This does not seem reasonable as the cruise ship underway could have two or three engines operating. This would be also be significant overcorrection. Also, since there is no proposed correction for less than average time at port, the net impact is a weaking of the CII standards. <CLIA> From the examples provided below, it can be seen that the correction does not overly compensate for port time, as this would lead to a reduction in CII value with increasing port time. The correction provides a relatively consistent compensation with a high level of accuracy. Example ship CLIA dataset analysis <EC> Insufficient evidence to assess |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes - Chile, Liberia, The Netherlands, Norway, Singapore, Sweden, CLIA, ICS Further development needed – Denmark, France, UK, US No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> Need for more robust guarantees for a good verification of data declared <CLIA> The correction factor only uses the ‘hours underway’ data that is already provided in the DCS reporting making verification of the value relatively simplistic. Also it should be noted for ships that were scrapped or delivered in the year being analyzed that the total amount of hours the ship was operations may be less than 8760, and this should be included in the analysis. <EC> Insufficient evidence to assess |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, France, Liberia, The Netherlands, Norway, Singapore, CLIA, ICS No – Sweden, UK, US | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <Sweden> Details must be defined how attained index should be calculated <US> This correction factor adds complexity to the calculation and to verification without providing corresponding value to the CII standards. <EC> Insufficient evidence to assess |  |

# CFSTS Correction Factor for Tankers with STS voyages

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, Cyprus, Greece, Italy, Liberia, Malaysia, The Netherlands, Norway, Singapore, Spain, UK, US, EC, ICS, INTERTANKO, IPTA No – Denmark, France, Japan, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Denmark> Do not see the different between a STS operation and a normal port operation <France> No sufficient data is provided by the proponent <Greece> Tankers with STS voyages fall under the scope of the CFshort correction factor proposal. The operation of tankers with STS voyages has been taken into consideration when developing the CFshort correction factors. There is value in applying the CFshort correction factor to this specific limited sample to confirm the applicability of the proposal. <Italy> Focus on tankers with majority of STS operations as per the revised proposal. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Liberia> This correction factor should be applicable to both tankers and bulk carriers undertaking STS operations <Malaysia> Applying a correction factor for the STS operations, so that the annual attained CII value is calculate as follows: CII for data from regular operations + CfSTS x CII for data reported for STS operations The lower the number of STS operation, the lower the data to be corrected by CfSTS. For tankers involved in one or two STS operations, the correction will have a little impact on the aggregate annual attained AER. <EC> Focus on tankers with majority of STS operations as per the revised proposal <ICS> We fully agree with Intertanko that the CFSTS correction factor is necessary. This vessel type incurs significant fuel consumption during long waiting time at zero distance travelled. This unfairly distorts the AER value, irrespective of the achieved operational efficiency. From the Intertanko submission we note that a typical STS tanker consumes up to 59 tonnes of fuel during a typical discharge, all of which is included within the CII calculation, but with no alleviation for transport work. Hence without the CFTS correction factor this ship type will be unfairly penalized. STS ship operations enable utilization of large, deep draft vessels with high fuel efficiency to serve shallow port locations. If the CFTS correction factor were not approved by IMO we fear the outcome would be a reduction in STS operations. This would then require a greater number of smaller shallow draft vessels, with relatively low fuel efficiency, to substitute for the larger vessels which currently serve these routes. Hence in the absence of the CFSTS correction factor, the outcome is likely to be an overall increase in emissions. We recommend this correction factor is also applied to bulk carriers that undertake STS operations. <IPTA> IPTA has no comments on this proposal but fully supports the need for a correction factor. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Italy, Liberia, Malaysia, Norway, Singapore, UK, US, EC, ICS, INTERTANKO, IPTA 2026 – Brazil, Canada, Cyprus, France, The Netherlands, Spain | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Cyprus, Italy, Malaysia, Russian Federation, Singapore, Spain, UK, EC, ICS, INTERTANKO, IPTA No – France, Japan, Liberia, [The Netherlands], Norway, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> We recognize that “shuttle tankers” or tankers engaged in STS operations may have a higher CII than other tankers. However, we consider that these ships should contribute equally to the overall level of ambition, compensating their higher carbon intensity with additional efforts elsewhere if necessary. Also, as all the shuttle tankers and all the ships engaged in STS operations would have a higher CII, due to their operational profile, we don’t consider this constituting a perverse incentive or a market distorsion among the shuttle tankers or STS tankers market. That said, if the inclusion of such correction factor had to be adopted by the Committee, we suggest the following amendement in the formula: to replace 0.75]-[0.03yi by 0.75-0.75×yi8 This would have the consequence to reduce the period in which correction factors are used in practice to 8 years from 2023 (instead of 25 years as proposed initially) and to sharpen the decrease of their effect over time, so that operators be incentivized to improve more rapidly the specific carbon intensity of ships benefitting from this correction factors and potentially other correction factors adopted at MEPC 78. <Japan> Practical problems such as DCS data collection are likely to occur. <Liberia> This correction factor should be applicable to both tankers and bulk carriers undertaking STS operations <The Netherlands> INTERTANKO has indicated to develop correction factors for shuttle tankers and for tanker engaged in STS operation, respectively and present the results at the next Round of CG exchanges”. it appears that Intertanko needs more time to fully define the CF. <Norway> The proposed text with explanation of the CII for STS operation needs editing – it is difficult to understand. It is also not in line with the actual calculation shown where it is the CO2 emissions from STS operations and not the CII (i.e divided by the distance and dwt) which is used. There seems to be a disconnect between the STS definition in the OCIMF guide which includes only from approach to departure maneuver, while the proposed factor seems to include sailing to and from the STS area and port operations as well. This sailing is not different from other operations and should not need to be corrected. Short voyages will also be additionally corrected. <Spain> We rather prefer to consider this item in the next revision in 2026 and see how the current CII works <US> We support further work on this issue, but are concerned that the proposed approach ignores fundamental operating modes for these vessels. A better approach than voyage exclusions would be to use a CO2/DWT-hour metric for vessels that engage in frequent STS operations. Alternatively, an appropriate value for DIST/hr could be assigned for STS operation to reflect the work done by the engines in normal operation. In either case, more work would be needed to develop a mature proposal. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Cyprus, Italy, Liberia, Malaysia, Norway, Russian Federation, Singapore, UK, US, EC, ICS, INTERTANKO, IPTA No - France | ☐ Yes ☐ No |
| Comments <Italy> Based on the revised proposal, normalising the significant higher fuel consumption for STS tankers at zero distance, including boundaries in terms of distance and time for each leg. <Malaysia> Operational aspect of STS tankers results in a higher attained AER values than the AER values attained by tankers engaged in regular trade only. Operationally, STS tankers have a higher overall fuel consumption while travelling shorter distances which disadvantages them from an AER perspective. This is shown in the expanded AER formula on which one can see that while the total distance will be reduced, the fuel consumption will increase by larger number of cargo discharges and fuel consumption during longer waiting time than tankers engaged in regular trade <EC> Based on the revised proposal, normalising the significant higher fuel consumption for STS tankers at zero distance, including boundaries in terms of distance and time for each leg. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Italy, Liberia, Malaysia, Russian Federation, Singapore, EC, ICS, INTERTANKO, IPTA No – France, UK, US | ☐ Yes ☐ No |
| If not, please explain your answer <France> The likely perverse incentives if a correction factor is not implemented are not clear in our view. In particular considering the following graph: It is assumed the market will prefer the option in red (3 suezmax) rather than the option in blue (VLCC+3 shuttle tankers) because the CII grade of the 3 suezmax is better than the CII grade of the 3 shuttle tankers, the red option generating more absolute emissions. This “transfer” hypothesis is not obvious in our view, for the following reasons: - We do not have the source and details of the calculations. - The VLCC having a better CII than the 3 suezmax, why wouldn’t the market, after a whole analysis, prefer the option in blue ? The option in red could be much more costly, while option blue allows for significant economies of scale. - A potential reorientation of the market may occur in a different way than the red scenario, e.g. a VLCC serving a big port, and then feedering from this port with smaller tankers but not necessarily shuttle tankers. <UK> We should be careful with how the STS ops are identified and verified in tankers that may also be engaged in normal operations, so that no perverse incentives are introduced such as declaring an STS journey leg when it is a normal operation one. <US> The perverse incentive would be to remove any requirement for improvements in GHG emissions during fundamental modes of operation, even though there are technologies available to reduce these emissions. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Cyprus, Italy, Liberia, Malaysia, Norway, Singapore, UK, EC, ICS, INTERTANKO, IPTA Over Correct - US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> Insufficient evidence to assess <Italy> Based on revised proposal as submitted <Norway> See comments above related to scope. The actual correction is likely correct, but the scope may need to be adjusted and then correction needs to be aligned. <EC> Based on revised proposal as submitted |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Italy, Liberia, Malaysia, The Netherlands, Norway, Singapore, UK, EC, ICS, INTERTANKO, IPTA Further development needed – Denmark, US No - Japan | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <Japan> Japan presumes that it would be difficult to cut out the power consumed solely for STS operations as an increase in fuel consumption of the generator. The possibility of over-reporting the FOC used during the period in which STS was implemented cannot be ruled out. <Norway> There needs to be precise definitions of when the STS operation starts and ends. See comments above. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Italy, Liberia, Malaysia, The Netherlands, Norway, Singapore, UK, EC, ICS, INTERTANKO, IPTA No | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <EC> Based on STS checklist |  |

# CFShuttle Tankers Correction Factor for Shuttle Tankers

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, Denmark, Greece, Italy, Liberia, Malaysia, The Netherlands, Norway, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA, OCIMF No – France, Japan, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <France> We consider this issue to be similar to STS operational profiles – see above <Greece> We consider that shuttle tankers may fall under the scope of the CFshort correction factor proposal. There is value in applying the CFshort correction factor to confirm the applicability of the proposal in cases of shuttle tankers engaged in short voyages less than 72hours. <Italy> Around half of fuel consumption of shuttle tankers is at zero distance. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Malaysia> The calculation of the annual attained CII/AER value for DP STs should be corrected by a statistically generate correction factor. Propose to add CII DP-STS Correction for Shuttle Tankers with Dynamic Positioning installations (DP STs). <Spain> Large amount of energy spent in DYNPOS, hence it needs consideration <Sweden> It is not supported that there is no transport work done. However, we can support the idea that special operation of these ships affecting fuel consumption could be considered. <EC> Around half of fuel consumption of shuttle tankers is at zero distance. <ICS> We fully agree with Intertanko that DP shuttle tankers are distinct from tankers in the following ways: - Shuttle taker DP equipment consumes fuel at zero distance travelled when loading at an offshore location. - This vessel type undertakes very short voyages resulting in a smaller total distance than the average total distance of tankers engaged in regular trade - Shuttle tankers undertake a much higher number of cargo discharges, 3, 4 and up to 10 times than the average number of discharges of same size tankers engaged in regular trade - Shuttle tankers have very long waiting periods when significant fuel is consumed, but no transport work is undertaken. Without correction, the above differences would result in shuttle tankers suffering more adverse CII ratings than equivalent sized tankers. Therefore ICS fully agrees with the necessity of this correction factor. <IPTA> IPTA has no comments on this proposal but fully supports the need for a correction factor <OCIMF> When it comes to DP shuttle tankers, they are different from (ordinary) tankers in their hull shape (some of them are designed for shallow water operations), propulsion systems (due to redundancy requirements) and operations (distance travelled, waiting time). The fact they do not require tugs in their operations should also be considered (because they are saving fuel that would have been sued by tugs). Out of two proposals at that time (different grouping or correction factor), MEPC 76 decided to take the latter. If we keep the situation without a correction factor, we will have to compare totally different ship groups under the same measurement. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Denmark, Italy, Liberia, Malaysia, Norway, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA, OCIMF 2026 – Canada, France, The Netherlands | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Denmark, Italy, Liberia, Malaysia, Norway, Spain, Sweden, EC, ICS, INTERTANKO, IPTA, OCIMF No – France, Japan, The Netherlands, UK, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> We consider this issue to be similar to STS operational profiles – see above <Japan> Practical problems such as DCS data collection are likely to occur. <Malaysia> Shuttle Tankers load cargo at offshore locations, exposed to harsh weather conditions. The reason DP STs are having a higher CII/AER line is due to the additional fuel consumption by their DP machinery equipment with due to the significant difference in operations with long waiting times and many more cargo operations than tankers engaged in regular trade. <The Netherlands> INTERTANKO has indicated to develop correction factors for shuttle tankers and for tanker engaged in STS operation, respectively and present the results at the next Round of CG exchanges”. it appears that Intertanko needs more time to fully define the CF. <UK> The proposal completely eliminates the fuel consumed during no distance operations, which we don’t agree with. We need to encourage efficiency in these operations too. <US> We do not support a correction factor approach as a way of changing the standards for all shuttle tankers. It would be more appropriate to revisit the reference line (and potentially metric) for these vessels. We support further work on this issue, but are concerned that the proposal ignores fundamental operating modes for these vessels. We would support revisiting the reference line (and potentially metric) for these vessels. A better approach than voyage exclusions would be to use a CO2/DWT-hour metric for vessels that engage in dynamic positioning. Alternatively, an appropriate value for DIST/hr could be assigned for dynamic positioning to reflect the work done by the engines in normal operation. In either case, more work would need to be done to develop a mature proposal <EC> Alternative solution of creating a separate ship category for shuttle tankers would be disproportionate <OCIMF> The group should benefit from examining the proposal by INTERTANKO and RINA (Round 4) carefully, but their data looks very exhaustive. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Denmark, Italy, Liberia, Malaysia, Norway, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA, OCIMF No - France | ☐ Yes ☐ No |
| Comments <Italy> Alternative solution of creating a separate ship category for shuttle tankers would be disproportionate. <OCIMF> Every ship must strive to improve efficiency. But the current regime is to "compare" with other ships within the group of ships to measure relative performance within the group of ships. When it comes to DP shuttle tankers, they are different from (ordinary) tankers in their hull shape (some are designed for shallow water operations), propulsion systems (due to redundancy requirements and dynamic positioning systems) and operations (distance travelled, waiting time). It is not technically sound to compare different ship types without compensation. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Denmark, Italy, Liberia, Malaysia, Norway, Spain, Sweden, EC, ICS, INTERTANKO, IPTA, OCIMF No – France, UK, US | ☐ Yes ☐ No |
| If not, please explain your answer <UK> The proposal completely eliminates the fuel consumed during no distance operations, which we don’t agree with. We need to encourage efficiency in these operations too. <US> The perverse incentive would be to remove any requirement for improvements in GHG emissions during fundamental modes of operation, even though there are technologies available to reduce these emissions. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct – Chile, UK Accurately Correct – Liberia, Malaysia, Norway, Sweden, ICS, INTERTANKO, IPTA Over Correct - US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> We consider this issue to be similar to STS operational profiles – see above <Malaysia> The calculation of the annual attained CII of the Shuttle Tankers with Dynamic Positioning installations (DP-STs) should be calculated as follows: CIIDP-ST= CfDP-ST x CO2DWTmax x Dtotal where CfDP-ST= 5.6805 x Capacity-0.208 The CfDP-ST correction factor is applicable to shuttle tankers equipped with Dynamic Positioning installation which are engaged in cargo oil distribution from offshore production sites to shore terminals, as approved by the verifier. <EC> Based on indicative proposal submitted, subject to confirmation by means of statistical analysis |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Denmark, Italy, Liberia, Malaysia, The Netherlands, Norway, Sweden, EC, ICS, INTERTANKO, IPTA Further development needed - US No – Japan, UK | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> We consider this issue to be similar to STS operational profiles – see above <Japan> Japan presumes that it would be difficult to cut out the power consumed solely for Dynamic position operation as an increase in fuel consumption of the generator. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Liberia, Malaysia, The Netherlands, Norway, Sweden, ICS, INTERTANKO, IPTA No - Denmark, Italy, UK, EC | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <EC> More detailed data needed. |  |

# fc EEDI Capacity Correction Factor for Chemical Tankers

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Denmark, Finland, Greece, Italy, Liberia, Norway, Spain, Sweden, UK, US, EC, ICS, INTERTANKO, IPTA No – France, Japan, The Netherlands, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <France> Not sufficient data is provided by the proponent on the general effect on the achievement of the carbon intensity and absolute levels of ambition of the Strategy. <Italy> As per the EEDI framework <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <The Netherlands> We cannot support the inclusion of this factor. Already around MEPC 62 the Netherlands showed in their study that the cubic capacity factor is not suitable for distinguishment between chemical tankers and other tankers. According to the study submitted in EE WG 2/INF.2 It was shown that the R for chemical tankers are the same as for VLCC’s and other tankers. Secondly based on the R values of chemical tankers, this factor will give a compensation to the Cii value of about 9 to 13%. (Average R=0,83) Chemical tankers already have other proposals which are more suitable for correction of their extra CO2 emissions based on their specific cargo requirements such as compensation factor for fuel consumption in relation to tank heating. By allowing this factor in combination with factors on cargo related consumption we create an overcompensation for these ship types in the Cii calculation and this will have an adverse effect on the effectiveness of the Cii. <Spain> If using the EEDI framework we would need a justification how/when factors are going to be taken from EEDI <Sweden> Sweden agrees in principle with the discussion part of this document, however we think that the specific proposal is not entirely clear. <EC> As per the EEDI framework <IPTA> IPTA fully supports the need for this correction factor. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Denmark, Finland, Greece, Italy, Liberia, Norway, Spain, UK, EC, ICS, INTERTANKO, IPTA 2026 – Canada, France, The Netherlands, US <The Netherlands> The consideration whether we should apply EEDI correction factors to the Cii calculation should be done in 2026 during the review when we have extra data and can analyses whether we put some ships in disproportional disadvantage compared to the reference line and competitors. | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Finland, Greece, Liberia, Norway, UK, ICS, INTERTANKO, IPTA No – France, The Netherlands, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> We do not support such correction factors as we consider that chemical tanker ships should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity due to heavier lightship and cargo maintenance requirements with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. <Spain> Not enough information provided at this time <US> This correction factor appears to increase the complexity of the standards with little benefit. However, we are open to considering this during the 2026 review in the context of more stringent standards. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Finland, Greece, Liberia, Norway, Spain, UK, ICS, INTERTANKO, IPTA No – France, The Netherlands, US | ☐ Yes ☐ No |
| Comments <Finland> Based on the information provided by INTERTANKO in their answers to question Q3.b in Round 3 of the CG, data is provided to justify the need to retain the cubic correction factor 𝑓𝑐 (paragraph 2.2.12 of MEPC.308(73)) in the formula of the attained CII/AER for chemical tankers. <Spain> Similar to other ship types, but this ship may do many stops in one port with short distances. <US> A case has not been made that chemical tankers would be substantially affected given the leniency of the CII standards agreed to date. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Finland, Greece, Liberia, Norway, UK, ICS, INTERTANKO, IPTA No – France, The Netherlands | ☐ Yes ☐ No |
| If not, please explain your answer <France> In our view, the absence of this correction factor would not generate any perverse incentive. Regarding perverse incentives related to the implementation of the correction factor (such as preventing from making all efforts to reduce the carbon intensity of the concerned ships elsewhere or preventing an earlier use of onshore power supply), we are of the view that they could be limited only if the 𝑦𝑖 factor increases more rapidly, as proposed in question 3 above. <The Netherlands> The Fc factor and the R ratio do not address the issue correctly as expained by question 1. In combination with cargo related compensation will this factor result in an over compensation and therefore create a perverse incentive. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - China Accurately Correct – Chile, Finland, Greece, Liberia, Norway, UK, ICS, INTERTANKO, IPTA Over Correct – The Netherlands | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Finland, Greece, Liberia, The Netherlands, Norway, UK, ICS, INTERTANKO, IPTA Further development needed No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Finland, Greece, Liberia, The Netherlands, Norway, UK, ICS, INTERTANKO, IPTA No | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# fiVSE EEDI Capacity Correction Factor for Voluntary Structural Enhancement

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Canada, Chile, Denmark, Finland, Greece, Liberia, The Netherlands, Norway, Spain, UK, US, EC, ICS, INTERTANKO No – France, Italy, Sweden, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <France> Not sufficient data is provided by the proponent on the general effect on the achievement of the carbon intensity and absolute levels of ambition of the Strategy <Italy> As per the EEDI framework. <The Netherlands> In general we do not support the use of EEDI correction factors in the Cii framework. The EEDI is a fixed indicator which you cannot optimize after it is set. It also has no rating boundaries. In case of the EEDI you fulfil the requirement or not. In case of the Cii you always have the ability to optimize your sailing, fuel consumption, application of new fuels etc to reduce the Cii. Secondly correcting your ship for design criteria which your competitor might not have has its advantages and disadvantages. By correcting for a specific design element in case of the Cii, we might incentives certain design elements which increase fuel consumption. They might be applied to ships which normally would not have any benefit of this design element, but can now allow themselves to apply it to its ship. <Norway> This would concern a small number of ships and we support this proposal in principle, although given that it came in very late, we have not managed to evaluate it. <US> The title of this correction factor seems to be broader than for self-unloading bulk carriers, which we would not support. Our comments are related specifically to self-unloading bulk carriers using gravity fed conveyor systems. <EC> As per the EEDI framework |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Denmark, Finland, Greece, Italy, Liberia, Spain, US, EC, ICS, INTERTANKO 2026 – Canada, France, The Netherlands, UK <The Netherlands> The consideration whether we should apply EEDI correction factors to the Cii calculation should be done in 2026 during the review when we have extra data and can analyses whether we put some ships in disproportional disadvantage compared to the reference line and competitors. <Spain> Same criteria as ice classed ships, but we don’t expect very high impact <Sweden> Sweden agrees in principle with the discussion part of this document, however we think that the specific proposal is not entirely clear. | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Finland, Greece, Liberia, ICS, INTERTANKO No – France, The Netherlands, Spain, US | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> We do not support such correction factors as we consider that ships having Voluntary Structural Enhancement should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. The CII calculation cannot take into account all specific situations, otherwise the measure itself would risk becoming meaningless. <US> We believe a better approach would be to develop a reference line specific to self-unloading bulk carriers using gravity fed conveyor systems. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Finland, Greece, Liberia, Spain, US, ICS, INTERTANKO No – France, The Netherlands | ☐ Yes ☐ No |
| Comments <Finland> Based on the information provided by ICS in their answers to questions Q2.c and Q3.b in Round 3 of the CG, this correction factor is needed in order to mitigate the negative impact of self unloaders using gravity-fed conveyor systems on the attained CII of this specific class of bulk carriers. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Finland, Greece, Liberia, ICS, INTERTANKO No – France, US | ☐ Yes ☐ No |
| If not, please explain your answer <France> In our view, the absence of this correction factor would not generate any perverse incentive. <US> Verification of these design factors would be very challenging. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Finland, Greece, Liberia, ICS, INTERTANKO Over Correct – The Netherlands, US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <US> Verification of these design factors would be very challenging. |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Finland, Greece, Liberia, The Netherlands, ICS Further development needed – Norway, US, INTERTANKO No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <Norway> We would need to understand how the calculation and verification of voluntary structural enhancements can be carried out for pre-EED vessels as a reference design is needed. <US> It is not clear on how the reduced cargo capacity would be determined or verified. Guidance may be necessary. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Finland, Greece, Liberia, The Netherlands, ICS, INTERTANKO No - US | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# FCextra weight Additional Fuel Consumption for increased displacement

Standalone correction factor to adjust fuel consumption of self unloading bulk carriers.

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, Greece, Liberia, The Netherlands, Norway, Spain, UK, US, ICS No – Denmark, France, Italy, Japan, Sweden, EC, CSC, Pacific Environment | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <France> Not sufficient data is provided by the proponent on the general effect on the achievement of the carbon intensity and absolute levels of ambition of the Strategy <Greece> As long as it is not related to the EEDI fiVSE and this results to a double correction. <Italy> Double-correction (see EEDI capacity correction above) <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Norway> This would concern a small number of ships and we support this proposal in principle, although given that it came in very late, we have not managed to evaluate it. <Spain> Same criteria to be applied for other ships handling cargo or excess weight, however duplication needs to be avoided. At this point in time some factors are overlapping <US> The title of this correction factor seems to be broader than for self-unloading bulk carriers, which we would not support. Our comments are related specifically to self-unloading bulk carriers using gravity fed conveyor systems. <EC> Double correction (see EEDI capacity correction above) |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, Greece, Liberia, Spain, UK, US, ICS 2026 – Brazil, Canada, France, The Netherlands <The Netherlands> There is not sufficient data provided by the submitters. The proposal is not complete at the moment. | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, Greece, Liberia, Spain, UK, ICS No – Denmark, France, Italy, The Netherlands, US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> We do not support such correction factors as we consider that self unloading bulk carriers should contribute equally to the overall level of ambition, compensating their structural higher carbon intensity with additional efforts elsewhere if necessary. We also prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. The CII calculation cannot take into account all specific situations, otherwise the measure itself would risk becoming meaningless. <US> We believe a better approach would be to develop a reference line specific to self-unloading bulk carriers using gravity fed conveyor systems. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, Greece, Liberia, Spain, UK, US, ICS No – Denmark, France, Italy, The Netherlands, EC | ☐ Yes ☐ No |
| Comments |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Greece, Liberia, Spain, UK, ICS No – Denmark, France, Italy, The Netherlands, US, EC | ☐ Yes ☐ No |
| If not, please explain your answer <France> In our view, the absence of this correction factor would not generate any perverse incentive. <US> Verification of these design factors would be very challenging. |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct Accurately Correct – Chile, Greece, Liberia, UK, ICS Over Correct – Japan, The Netherlands, US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <Italy> Double correction <Japan> The correction of extra fuel consumption due to extra weight and the DWT correction due to reduced cargo capacity from the mass and volume of the conveyor belt system are supposed to be duplicate counts. <US> Verification of these design factors would be very challenging. <EC> Double correction |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Greece, Liberia, The Netherlands, UK, ICS Further development needed - US No | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <US> It is not clear on how the increased weight would be determined or verified. Guidance may be necessary. |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, Greece, Liberia, The Netherlands, UK, ICS No - US | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# CFsludge Correction Factor for Sludge

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, China, Liberia, Sweden No – Cyprus, Denmark, France, Greece, Italy, Japan, The Netherlands, Norway, Spain, UK, US, EC, ICS, INTERTANKO, CSC, Pacific Environment, WSC | ☐ Yes ☐ No |
| Comments <Brazil> We consider that it is early to adopt correction factor without fully understanding the behaviour of the indicators on the global fleet, considering various trades. Until 2026, the member states will be able to gain experience and propose correction factors that don’t cause perverse incentives nor exclude the accounting of emissions generated by operational factors, under control of the ship operator. There will also be the need for a specific field to be created in the DCS for consumption in situations affected by each correction factor adopted. <Canada> As expressed in previous round Canada's preference is to wait for a discussion on correction factors / exclusions as part of the review of the short-term measure by 2026, when more data and analysis of the information is available <Italy> Broad measure, duplicating existing provisions for emission factors (taking into account the effect of sludge). <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <The Netherlands> Sludge waste production on board should not be compensated in the Cii calculation. Fuel consumption can be measured based on direct fuel consumption measuring at the engines itself. The consumption measure is based on the fuel flow towards the engine, minus the retour flow. The fuel flowing to the engine is fuel after purification by purifiers, separators and filters. Therefor the fuel consumption measured is always without the sludge. Even when fuel consumption is based on the content of the daily service tank instead of flow meters, this is without sludge. In most cases the daily service tank is supplied by a fuel stream after purification and filtering. This is already recognizes in MEPC 282(70), the SEEMP guideline chapter 7 Guidance on methodology for collecting data on fuel oil consumption etc par 7.1.2.3 where it is stated that for direct flow meter method, it should be noted that it should not be necessary to correct this fuel oil measurement method for sludge if the flow meter is installed after the daily tank as sludge will be removed from the fuel oil prior to the daily tank Secondly, it is strange to allow a waste stream such as sludge to be used to compensate the CO2 emission by a ship. This can lead to a wrong incentive to keep on using dirty fuels with high sludge production. <Norway> We do not support this correction. This issue should be solved on the reporting side. E.g. if using fuel flow meters, the sludge would not generally be included in the reporting. <Spain> We appreciate the information provided, and agree that a percentage of fuel oil loaded is lost during fuel treatment (water removal, purifier and filter blowdown/cleaning etc). However, these losses are reasonably stable and as such are already reflected in the CII reference lines. CII corrections should be used to correct for highly variable parameters which are not a function of ship efficiency but which will degrade CII values if not addressed (for example, adverse weather, cargo conditioning). <UK> This does not, in our view, present a credible enough case for the inclusion of a correction factor for sludge. <US> This proposal would increase the complexity of the CII calculations to address a problem that seems really small (within the error of measuring fuel consumption). Directionally, it would promote lower quality fuels. <EC> Broad measure, duplicating existing provisions for emission factors (taking into account the effect of sludge) <ICS> For vessels equipped with fuel flow meters, the fuel consumption will be measured after any sludge or water has been removed by the on board settling tanks. Hence the sludge will have no effect on the measured consumption. For vessels not equipped with flow meters we believe the effect on the CII rating is likely to be small, and hence we do not believe the CFSludge factor merits further investigation. <INTERTANKO> The difference in fuel consumption reported between ships with mass flow meters and those without is small. In addition, in response to the CII regulations, ships should be more precise in measurement of the actual fuel consumption, thus use of mass flow meters is recommended. <WSC> We appreciate the information provided, and agree that a percentage of fuel oil loaded is lost during fuel treatment (water removal, purifier and filter blowdown/cleaning etc). However, these losses are reasonably stable and as such are already reflected in the CII reference lines. CII corrections should be used to correct for highly variable parameters which are not a function of ship efficiency but which will degrade CII values if not addressed (for example, adverse weather, cargo conditioning). |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now – Chile, China, Liberia, Sweden 2026 – Brazil, Canada Should not be considered – The Netherlands | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, China, Liberia No – Denmark, France, Italy, The Netherlands, Sweden, US, EC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> It is a broad proposal, duplicating existing provisions for emission factors (taking into account the effect of sludge), and with significant overlap with other, more targeted proposals. Corrections or adjustments should be possible only for very specific issues, affecting a limited number of ships or situations easy to identify. We prefer not multiplying correction factors before the review of the measure in order to have a more transparent vision of its implementation and effect in the first years – so that the need for potential correction factors may be better assessed at that time. <Sweden> Sweden can support the deduction of a 50% of total sludge delivered ashore by ships during the reporting year. However, the details on how this would influence an attained index must be developed. <US> We do not believe this correction factor is necessary. The proposal is based on a limited data set and shows a very small impact. <EC> No evidence of disproportionate impact or perverse incentives |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, China, Liberia, Sweden No – Denmark, France, Italy, The Netherlands, US, EC | ☐ Yes ☐ No |
| Comments <France> No evidence of disproportionate impact or perverse incentives has been provided. <US> Given the small impact of sludge on the CII calculations, we do not believe this correction factor is necessary. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, China, Liberia, Sweden No – Denmark, France, Italy, The Netherlands, US, EC | ☐ Yes ☐ No |
| If not, please explain your answer |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - China Accurately Correct - Chile, Liberia, Sweden Over Correct - US | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <China> - fixed correction factor of 1.0% of total quantity of residual fuel consumed during the reporting year for ships other than container ships. - fixed correction factor of 2.0% of total quantity of residual fuel consumed during the reporting year for container ships. <The Netherlands> Sludge should not be used as a basis for a correction factor. <Sweden> Correction factor and calculations must be developed to correct establishment of attained index. <US> This correction is not necessary and creates additional complexity for the CII calculation and for enforcement. <EC> Insufficient evidence to assess |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, China, Liberia, Sweden Further development needed No - US | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Chile, China, Liberia, Sweden No - US | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” |  |

# fladen Correction factor for sailing in laden conditions

While Capacity to assess effects will not be scored, given the data provided in this section, it will be useful to gauge whether CG members thinks the issue merits further consideration (without pre-judging a solution) either at this stage or later. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Capacity to Assess their effects (for info) (Q2) | Answer |
| --- | --- |
| 1. Given the information provided, does this issue merit further consideration? Yes – Brazil, Canada, Chile, France, Liberia, The Netherlands, Norway, Sweden, UK, ICS, INTERTANKO, IPTA No – China, Cyprus, Denmark, Italy, Japan, Spain, US, EC, CSC, Pacific Environment, WSC | ☐ Yes ☐ No |
| Comments <Brazil> We do not agree with the proposal for a Correction Factor for sailing in load conditions, since it does not constitute a specific operation different from the normal activities of ships consisting of transporting cargo between ports. <Denmark> Denmark would prefer to include the actual cargo carried in the CII requirements and calculation <Italy> Broad measure, effectively reversing the choice of AER as the primary metrics. <Japan> Japan is basically opposed to the introduction of correction. The reasons are as follows. - Allowing corrections would discourage the substitution of ships with poor fuel efficiency, which would distort the essence of short-term measures aimed at reducing GHG emissions. - Once a correction is introduced for a specific type of ship and/or type of operation, it will lead to endless arguments that other correction should be introduced for the other types of ships and/or types of operation as well. - Regardless of types of ships and/or types of operation, all ships have to be required to reduce GHG emissions in the same way. The introduction of corrections would result in different degrees of GHG emissions reduction according to specific types of ships and/or types of operation, which would be unfair. <Spain> This correction deserves to consider a wider picture and the CII philosophy <Sweden> Sweden agrees with the proposal. Full loaded ships should be preferred to a number of partly loaded ships to reduce total emissions per total load. <US> This proposal seems to attempt to use a correction factor to make the AER metric behave more like the EEOI metric for certain vessels. The United States does not support this approach to account for cargo carried. <EC> Broad measure, effectively reversing the choice of AER as the primary metrics <IPTA> IPTA has no comments but supports this proposal <Pacific Environment> The CII reference line already takes into account the proportion of fuel consumption under ballast/laden conditions by capacity. <WSC> The proposal would provide a preferential rating to a very small sub-group of ships because they are said to achieve a high degree of utilization. We are concerned with the proposal for two reasons: - Utilization affects all ship types, allowing a small sub-group of ships to claim a credit for high utilization is discriminatory and will unfairly penalize all other ships operated with high utilization, distorting the rating system; and - Shipowners are already encouraged to report additional data and to calculate alternative CIIs on a voluntary basis, to facilitate analysis of alternative calculation methods which include utilization. This voluntary encouragement is applicable to all ships within the scope of the regulation and will facilitate an informed review in 2026. This is a more appropriate way to address the underlying concern raised in the laden voyage correction factor proposal. Given these concerns, we cannot recommend support for the proposal. |  |
| 2. Should this issue be considered now, or in line with the 2026 review? Now - Chile, Liberia, Norway, Sweden, UK, ICS, INTERTANKO, IPTA 2026 – Brazil, Canada, Denmark, The Netherlands | ☐ Now ☐ 2026 |

While Policy Justification will not be scored, we would still like to gather the views of CG members. The answers to this question may however be relevant for evaluation of proposals to amend G2 and G4.

| Policy Justification (for info) (Q3) | Answer |
| --- | --- |
| 3. Do you agree that the issue is adequately solved by the solution suggested? Yes – Chile, France, Liberia, Norway, Sweden, UK, ICS, INTERTANKO, IPTA No – Cyprus, Denmark, Italy, The Netherlands, Spain, US, EC, WSC | ☐ Yes ☐ No |
| If not, please provide specific comments in this box with a similar level of evidence and justification as provided in the proposals. <France> This correction factor aims at fixing the main defect of the AER/cgDIST as an operational carbon intensity indicator, identified since a long time. Such CF, if possible, could make the CII more legitimate and credible, as the mandatory use of the EEOI is not possible. However, we would be reluctant to adopt it as proposed, as it would improve the attained CII of all ships concerns but without reviewing the whole reference lines. In that sense, it would be very “generous” and could weaken the overall environmental ambition of the short-term measure (given the number of ships favoured). Therefore, we suggest to explore the following alternative: The advantage of this formula is that it rebalances what is “given” to some ship by “removing” to others, creating an incentive to better load without weakening the overall ambition of the measure. Here is an illustration of the calculation of the AER corrected and uncorrected for 2 fictive ships having different proportion of laden and ballast voyages: With: <Italy> Introduces imbalance between combination carriers, bulk carriers and tankers, which is not justified by evidence. <The Netherlands> This CF would make the CII more legitimate and credible, as the mandatory use of the EEOI is not possible. However the measure might need some modification to avoid over compensation or weakening of the reduction factors for this group of ships. <Spain> This should have been discussed previously. See our comment at the end of this document <US> The policy decision was made to base the CII on an AER metric, which uses DWT as a surrogate for cargo carried. If this approach to representing cargo carried is to be revisited, it should be through the metric and associated standards and not through a correction factor. <EC> Introduces imbalance between combination carriers, bulk carriers and tankers, which is not justified by evidence <WSC> As already stated, in addition to concerns that the proposal discriminates against most ships, the underlying issue is already addressed by the encouragement to submit additional data on a voluntary basis to facilitate an informed review in 2026. We do not consider a correction factor to be an appropriate means to address vessel utilization. |  |
| 4. Do you agree that without mitigations (in the form of correction factors, voyage adjustments, changes to the reference line), there is a significant negative impact on ships? Yes – Chile, France, Liberia, The Netherlands, Norway, Sweden, UK, ICS, INTERTANKO, IPTA No – Denmark, Italy, Spain, US, EC | ☐ Yes ☐ No |
| Comments <France> We should try to avoid penalising relatively operators making efforts to optimize their cargo, as long as it is possible to apply and control in practice (see Q5 below). <The Netherlands> Cargo utilization optimisation should not be penalised because of the CII <US> No. The AER recognizes the majority of carbon reduction strategies. As proposed, the correction factor would only apply to ships with higher cargo utilization than average (and not to ships with lower cargo utilization than average). Thus, the correction factor would serve to weaken the stringency of the CII standards. |  |
| 5. Are the perverse incentives addressed? Yes – Chile, Liberia, The Netherlands, Norway, Sweden, UK, ICS, INTERTANKO, IPTA No – Denmark, France, Italy, Spain, US, EC | ☐ Yes ☐ No |
| If not, please explain your answer |  |

| Accuracy (Q4) | Answer |
| --- | --- |
| 6. The correction factors and voyage adjustments proposed Under correct - US Accurately Correct - Chile, Liberia, Norway, Sweden, UK, ICS, INTERTANKO, IPTA Over Correct – France, The Netherlands | ☐ under correct ☐ accurately correct ☐ over correct |
| Do you have any suggestions to improve accuracy or general comments? <France> We are afraid a constant of 0.33 to adjust for increased consumption on a laden voyage compared to a ballast voyage may be too high. First we don’t find the linear regression in Figure 1 very meaningful. Second it is not considered in the proposal that ships in ballast sail generally a bit faster than laden (in particular bulk carriers), canceling a part of the gain in fuel consumption due to the absence of cargo. From empirical data from actual bulk carriers and tankers, we find that Cl is probably closer to 0.2 than 0.33, and even more probably to 0.1 for bulk carriers specifically, because of their average higher speed in ballast than in laden condition. If applied to the alternative formula proposed above, it would “smooth” the application of this correction factor, which is probably safer in the end, and still better than no correction factor at all. Furthermore, regarding the average part of voyage under laden condition, we note from Table 1 in document MEPC 77/7/14 that 53% is very low for small bulk carriers and tankers. Therefore, it might be relevant to use a higher, more conservative value, at east for the smallest bins. In conclusion, for both the Cl and the 53% factor, we think that more analysis is needed to ensure that this CF does not overcorrect. This is an important point as it would concern two of the three biggest emitting ship categories. <US> This factor would need to be applied to all ships, even in cases where it would worsen the CII score, to not weaken the CII standards. <EC> Insufficient evidence to assess |  |

Note that proposals calling for changes to G2 and G4 do not need to be evaluated in this section.

| Applicability (Q5) | Answer |
| --- | --- |
| 7. Is verification of the correction factors or voyage adjustments feasible? Yes – Chile, Liberia, Norway, Sweden, UK, ICS, INTERTANKO, IPTA Further development needed – France, The Netherlands, US No – Cyprus, Italy, EC | ☐ Yes ☐ Further development needed ☐ No |
| Please provide comments in this box if the answer is “Further development needed” or “No” <France> We are concerned by the controllability of the cargo data reporting by the Administration/RO. In absence of robust guarantees to check the actual cargo carried during the whole previous year, this correction factor could constitute a dangerous loophole enabling ships to “play” with it to improve their attained CII. Further, there may be a need to define more precisely “laden voyages”. If the Committees finds that the current guarantees are not strong enough, we would prefer not having such correction factor. <US> To the extent that data are collected on cargo utilization and an EEOI approach is taken, more work would need to be done on guidance for collecting, reporting, and verifying cargo utilization. <EC> Condition is the accurate cargo reporting that is currently not available at global level and is subject to trial metrics on voluntary basis |  |
| 8. Are the suggestions for evidence to be provided sufficient for verification to be carried out? Yes – Liberia, The Netherlands, Norway, Sweden, UK, ICS, INTERTANKO, IPTA No – Cyprus, Denmark, Italy, US, EC | ☐ Yes ☐ No |
| Please provide comments in this box if the answer is “No” <Italy> Condition is the accurate cargo reporting that is currently not available at global level and is subject to trial metrics on voluntary basis. <US> As discussed above, this would need to be further developed. |  |

General comment by Spain:

Spain is now confused with the current approach. We don’t understand very well the correction factors to be considered. The large amount of documents submitted refer either to basic principles and aspects that should have been previously discussed or factors that would need deeper consideration.

Spain, knowing that the fact that transport work (undefined) is in the denominator, considers that most of the proposal have valid points and some of them should have been included in the reference lines. We don´t see which criteria needs to be followed to accept one or the other factor and we also fear that the CII, if using many factors, customizing each and every group of ships, no matter large or small will dilute the proposal and will make it of low use when progressing to a midterm measure. What is being done risks being an “EEDI Part II”

We consider that the correspondence group, MEPC or ISWG GHG need to define criteria before accepting them.