IMO	CG on Carbon Intensity Indicators

ToR 3 Round 4 Summary and Outcome

10 January 2022

Contents

1	Introduction	3

2	Proposals to change G2 and G4	5

2.1	cgDIST for Ro-Ro Cargo Ships	5

2.2	Reference line split at 25,000 dwt for Ro-Ro Cargo Ships	5

2.3	Split reference lines for Vehicle Carriers	6

2.4	HSC	6

2.5	Reference Line adjustment for Combination Carriers	7

2.6	Rating Boundary Normalisation	7

3	Proposals for G5	8

3.1	Voyage adjustment for Adverse Weather	8

3.2	Voyage Adjustment for sailing in Ice	8

3.3	Short Voyage Correction	8

3.4	FCelectrical Reefer	9

3.5	FCelectrical Cargo Handling	10

3.6	FCelectrical Cargo Maintenance	10

3.7	FCBoiler and FCOthers	11

3.8	FCBOG for LNG Carriers	11

3.9	Nitrogen in BOG	12

3.10	Cruise Port Time Correction	13

3.11	Tankers with STS Voyages	14

3.12	Shuttle Tankers with DP	14

3.13	Capacity Correction Factor for Chemical Tankers	15

3.14	Capacity Correction Factor for Voluntary Structural Enhancement	15

3.15	FCextra weight for Self Unloading Bulk Carriers	16

3.16	Sludge Correction	16

3.17	FCLaden	17

# Introduction

The answers provided in round 4 have been summarised in an excel table, and a percentage in favour has been calculated for each question for each of the correction factor and voyage adjustment proposals. For the question on accuracy, accurately correct and under correct are both treated as being in favour; and for the question verification, yes and further development needed are both treated as yes.

As explained, the questions regarding accuracy and applicability are the primary ones to be scored and assessed. However where the answers for these questions are unclear, the coordinators will consider the comments provided and use capacity to assess effects and policy justification to help come to a position.

The exception to this are the proposals to change G2 and G4 where capacity to assess effects and policy justification questions will need to be considered because the questions for accuracy and applicability may not be relevant.

Overall summary is as follows:

Proposals to change G2 and G4

| cgDIST for Ro-Ro Cargo ships | Recommendation to amend G2 and G4 |
| --- | --- |
| Ref line split for Ro-Ro Cargo ships | No action pending further analysis |
| Split reference line for vehicle carriers | Recommendation to amend G2 |
| HSC | Recommendation to amend G2 and G4, form of amendment to G2 to be clarified |
| Reference line adjustment for combination carriers | Recommendation to amend G2 |
| Rating Boundary Normalisation | Rejected |

Proposals for G5

| Adverse weather | Rejected |
| --- | --- |
| Voyage adjustment for ice | Recommended for inclusion in G5 |
| Short voyage correction | Rejected |
| FCelectrical Reefer | Recommended for inclusion in G5 with 75%/3%, require further work on verification |
| FCelectrical Cargo Handling | Rejected (for bulk carriers and self-unloading bulk carriers), recommended for inclusion for tankers with electric discharge pumps. |
| FCelectrical Cargo Maintenance | Recommended for inclusion in G5 with 75%/3%, limited to gas cooling on gas carriers and for implementation and verification to be further developed in CG Round 5. |
| FCBoiler & FCOthers | Recommended for inclusion in G5 with 75%/3% for cargo heating and discharge, and also including FCelectrical for discharge and for implementation and verification to be further developed in CG Round 5 |
| FCBOG | Rejected |
| Nitrogen in BOG | To be dealt with at fuel reporting stage |
| Cruise Port Time Correction CFPT | Recommended for inclusion in G5 |
| Tankers with STS Voyages | Recommended for inclusion in G5, clarification of scope and designation of STS voyages to be developed in Round 5 |
| Shuttle tankers with DP | Recommended for inclusion in G5, IMO Secretariat to update correction factor using DCS data |
| Capacity correction factor for Chemical tankers fc | Recommended for inclusion in G5 |
| Capacity correction factor for voluntary structural enhancement fiVSE | Recommended for inclusion in G5 for self-unloading bulk carriers only |
| FCextra weight for self unloading bulk carriers | Rejected |
| Sludge Correction | Rejected |
| FCLaden | Rejected |

The updated CII formula is now as follows:

jCFj⋅FCj-FCvoyage,j+TFj+0.75-0.03yi∙ FCelectrical,j+FCboiler,j+ FCothers,jfi⋅fm⋅fc⋅fiVSE⋅Capacity⋅Dt-Dx⋅AFPT

The following are the new terms introduced:

- TFj = 1-AFTanker⋅FCS,j  represents the quantity of fuel j removed for STS or shuttle tanker operation, where FCS,j= FCj for shuttle tankers and FCS,j is the total quantity of fuel j used on STS voyages for STS vessels. If TFj>0 then FCelectrical,j=FCboiler,j= FCothers,j=0;

- AFTanker represents the correction factor to be applied to Shuttle tankers or STS voyages according to paragraph 4.2 of G5;

- AFPT is the Port Time Correction factor for cruise passenger ships only

Note that FCvoyage may also be used for situations that fall under MARPOL Annex VI 3.1.

# Proposals to change G2 and G4

Where the overall view of the group is positive, this will be reflected in the CG report to MEPC 78 and a recommendation will be made to amend guideline G2 accordingly. The coordinators will make the initial calculations with the anonymised and rounded data and these will be checked and implemented by the IMO Secretariat using the non-anonymised and non-rounded dataset.

Note percentage under “Considered now or later” indicates percentage in favour of “Now”.

## cgDIST for Ro-Ro Cargo Ships

21 CG members provided an evaluation of this proposal. Based on the answers provided, the overview for this proposal is as follows:

As can be seen, the CG was almost unanimously in favour of cgDIST being used for ro-ro cargo ships and as such the CG recommends that G2 will be amended accordingly.

Result: Recommendation to amend G2 and G4

## Reference line split at 25,000 dwt for Ro-Ro Cargo Ships

20 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The CG was somewhat supportive of this proposal. However the answers for policy justification and capacity to assess effects indicated some uncertainty. Some comments also indicated disagreement with having a non-continuous reference line. The originator of this proposal, INTERFERRY, has also requested to keep this issue open for now while a better solution is found. As such we suggest that this proposal is not taken up.

Result: No action pending further analysis.

## Split reference lines for Vehicle Carriers

21 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The CG was supportive of this proposal. Some comments suggested that there appears to be an over-correction or errors with the derivation of the lines, and also that this needs to be reconfirmed with DCS data.

The Co-ordinators have carried out an initial investigation into the effect on the split with DCS data and concluded that there are some differences with the MRV data that will result in a discontinuity in the reference line at 30,000 GT, but this is to be confirmed with the IMO Secretariat.

Result: Recommendation to amend G2.

## HSC

18 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The CG was supportive of this proposal. The only choice concerns how the offset reference line will be reflected in G2, whether in the table itself or as a footnote, while avoiding the need (for now) to create a new definition of HSC in MARPOL Annex VI.

The proposal will require a revision of reference line parameters for ro-ro passenger ships (since HSC are taken out of the population) in G2 and an update to the rating boundaries in G5.

Result: Recommendation to amend G2 and G4; form of amendment to G2 to be clarified.

## Reference Line adjustment for Combination Carriers

16 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The CG was supportive of this proposal.

Result: Recommendation to amend G2.

## Rating Boundary Normalisation

23 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

There were significant concerns and objections to this proposal as well as comments suggesting further analysis is needed. As such it will not be put forward for amendments to G4.

Result: Rejected

# Proposals for G5

## Voyage adjustment for Adverse Weather

27 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The CG had significant concerns about the proposal for voyage adjustments due to adverse weather and this will not be incorporated into G5.

Result: Rejected

## Voyage Adjustment for sailing in Ice

26 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The proposal may be considered to be well supported by the CG, however some adjustments have been suggested.

Result: Recommended for inclusion into G5

## Short Voyage Correction

24 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

There was significant uncertainty and objection to this proposal. Some of the reasons given included overlaps with other correction factors, overly broad and not targeted correction, perverse incentive to speed up to meet the 72 hour threshold, and that significant further work is needed. As such this will not be incorporated into G5.

Result: Rejected

## FCelectrical Reefer

25 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The CG strongly supported this proposal, and the evaluation was consistent across all the answers to the questions.

Only 11 members evaluated the requirements for adjustment and incentive factors, however there was a majority supporting both 75% adjustment and 3% annual incentive factor.

Some concerns were raised around the concept of removing all reefer related fuel consumption and the relationship with the baseline, as well as increasing the complexity of the measure however no concrete alternative proposals were provided. There were also concerns raised regarding the administrative burden that might be introduced in practice.

On this basis, the Reefer correction will be included in G5, however further work on verification requirements will be needed in Round 5 of the CG.

Result: Recommended for inclusion in G5 with 75%/3%, require further work on verification.

## FCelectrical Cargo Handling

21 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

Two proposals were received under this heading, the first for geared bulk carriers, and the second for specialised self-unloading bulk carriers (although this was proposed under FCothers).

From the overall evaluation of the CG, there is significant opposition and uncertainty, and further data was proposed to be submitted to Round 5 of the CG. However it will not be possible for CG members to receive data in the course of Round 5 and carry out an evaluation simultaneously.

It is noted that there was more support for correction factors for self-unloading bulk carriers compared to geared bulk carriers, and that two more correction factors have been proposed for this category of ship.

It may be more effective to designate self-unloading bulk carriers as a different ship type.

Result: Rejected

## FCelectrical Cargo Maintenance

23 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

Evidence was provided for gas carriers loaded with different types of gaseous cargo and the correction factor was widely supported by the CG on this basis.

Only 12 members evaluated the requirements for adjustment and incentive factors, however there was a majority supporting both 75% adjustment and 3% annual incentive factor.

Further implementation and verification details will need to be developed during Round 5.

Result: Recommended for inclusion in G5 with 75%/3%, limited to gas cooling on gas carriers and for implementation and verification to be further developed in CG Round 5.

## FCBoiler and FCOthers

24 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

Evidence was provided for boiler consumption relating to cargo discharge and cargo heating. Note that FCothers is implemented to deal with cargo discharge pumps which have their own attached generator.

The correction factor was widely supported by the CG. It should be noted that some tankers also have electrically driven pumps instead of steam driven pumps and consequentially FCelectrical cargo handling is interpreted to be supported for electrically driven discharge pumps otherwise there is an unfair advantage given to ships equipped with steam driven pumps.

Only 12 members evaluated the requirements for adjustment and incentive factors, however there was a majority of 2 supporting the 75% adjustment factor and larger majority supporting a 3% annual incentive factor.

Further implementation and verification details will need to be developed during Round 5.

Result: Recommended for inclusion in G5 with 75%/3% for cargo heating and discharge, and also including FCelectrical for discharge and for implementation and verification to be further developed in CG Round 5.

## FCBOG for LNG Carriers

23 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The response from the CG was mixed. While the proposals were considered to be accurate and that verification is feasible (albeit requiring further work), many felt that there was insufficient evidence available for verification.

There was some uncertainty over whether boil off gas sent to the GCU is reported to the IMO DCS since it is not explicitly included in paragraph 7.1 of the SEEMP – if it is not reported it clearly should not be deducted/corrected.

On the subject of perverse incentives there were a number of concerns raised, notably unequal treatment of different boil off gas management methods. There are 4 boil off gas management methods at the moment:

- Use in propulsion

- Steam dumping

- Burnt in gas combustion unit

- Reliquefaction

Concern was raised that the correction factor would incentivise steam dumping or use of GCU but use in propulsion cannot be deducted or corrected. Another point of view suggested that reliquefaction should be incentivised over propulsion, steam dumping or GCU.

Just under half of respondents indicated that the issue was not solved and half considered that this should be dealt with under the review to be completed prior to 2026. A majority also indicated that it needed further consideration and had a significant negative impact.

The differences of opinion highlights that this is a complex issue that cannot be resolved within the timescale of this correspondence group.

14 members evaluated the requirements for adjustment and incentive factors, however there was a majority supporting the 75% adjustment factor and larger majority supporting a 3% annual incentive factor.

Result: Rejected

## Nitrogen in BOG

9 CG members provided an evaluation of this proposal, and only 7 answered the accuracy and applicability questions. Based on the answers provided,  the overview for this proposal is as follows:

The small number of CG members who evaluated this correction factor were mostly supportive, except for one member who did not think there was sufficient information to implement or carry out an evaluation.

One CG member pointed out that this should be solved at the fuel reporting stage rather than as a correction factor, since LNG fuel reported should correspond to the Cf of 2.75. This seems a preferable way forward.

Result: To be dealt with at fuel reporting stage

## Cruise Port Time Correction

21 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

From reading some of the comments received, CG members did not seem  to have taken into account the updated proposal from CLIA, but instead evaluated a previous proposal which had a number of options. The current proposal from round 3 of the CG is formulated as follows:

AFPT=1.8-Hours Under Way8760

Where Hours Under Way is as reported to the IMO Fuel consumption database and CFPT is applied in the denominator of the CII.

The proposal was broadly supported in spite of this.

One CG member suggested that this correction factor is a temporary fix until a better metric can be found.

Some comments mentioned cruise ships with non-conventional propulsion. It should be clarified that cruise ships with non-conventional propulsion are a category only for the purposes of EEDI and EEXI since the reference lines and calculation methodologies are specific to diesel electric systems, however for CII this distinction does not exist, all cruise ships above 5,000 GT regardless of propulsion type are in scope.

Result: Recommended for inclusion in G5

## Tankers with STS Voyages

24 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

The proposal was supported by the CG.

Some queries were raised regarding scope and designation of voyages as STS which will need to be clarified in Round 5 of the CG.

Result: Recommended for inclusion in G5, clarification of scope and designation of STS voyages to be developed in Round 5

## Shuttle Tankers with DP

23 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

This proposal was somewhat supported by the CG, with only a slightly lower evaluation of whether the evidence is sufficient for verification. The coordinators note that this is a fixed correction factor rather than a variable deduction based on fuel consumption, so it is assumed that the answers relate to the derivation of the correction factor itself.

The proposal was developed with 65 shuttle tankers and the approach will need to be replicated by the IMO Secretariat with DCS data for all DP shuttle tankers which will change the parameters of the correction factor.

Result: Recommended for inclusion in G5; IMO Secretariat to update correction factor using DCS data

## Capacity Correction Factor for Chemical Tankers

23 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

This correction factor was supported by the CG. However there was also concerns raised that this would result in over correction.

Result: Recommended for inclusion in G5

## Capacity Correction Factor for Voluntary Structural Enhancement

19 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

Evidence was provided for self unloading bulk carriers which was evaluated by the CG. The correction factor was supported by CG. Concerns were expressed over challenges in deriving the correction factor.

As discussed above in FCelectrical cargo handling, there seems to be general support to deal with issues specific to self unloading bulk carriers, and while multiple correction factors are proposed during the course of this Correspondence group, the proponents should consider definition of a separate ship category.

Result: Recommended for inclusion in G5 for self-unloading bulk carriers only.

## FCextra weight for Self Unloading Bulk Carriers

19 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

While this proposal was supported from the applicability point of view, it did not score well on accuracy. Looking at the answers to the other questions, there seems to be some uncertainty over this correction factor, and this is reflected in the comments suggesting this is a double correction.

As discussed above in FCelectrical cargo handling, there seems to be general support to deal with issues specific to self unloading bulk carriers, and while multiple correction factors are proposed during the course of this Correspondence group, the proponents should consider definition of a separate ship category.

Result: Rejected

## Sludge Correction

22 CG members provided an evaluation of this proposal, however only 5 answered the questions on accuracy and applicability. Based on the answers provided,  the overview for this proposal is as follows:

While the scores for accuracy and applicability would indicate that this is supported, too few answers were given to be definitive about this. When considering the provided comments, a number mentioned that this may be alleviated via use of flow meters, while the scoring for the other questions indicate that there is significant opposition to this correction factor. As such, this will not be taken further.

Result: Rejected

## FCLaden

23 CG members provided an evaluation of this proposal. Based on the answers provided,  the overview for this proposal is as follows:

While the scores for accuracy and applicability seem to indicate support for this proposal, the comments overall indicate significant concern over such a broad correction that targets bulk carriers and tankers above a certain size, and the choice of constants used. There were also comments that it would be preferable to implement EEOI directly rather than try to approximate EEOI using a correction factor for some ships only.

This is also reflected in the percentage scores.

Result: Rejected.