ANNEX 1

DRAFT GUIDELINES ON CORRECTION FACTORS AND VOYAGE ADJUSTMENTS FOR CII CALCULATIONS (G5)

1	Introduction

1.1	These Guidelines address the corrections factors and voyage adjustments which may be applied to the operational carbon intensity indicator (CIIship) as defined by the Guidelines on Operational Carbon Intensity indicators and the Calculation Methods (CII Guidelines, G1).

2	Definitions

For the purpose of these Guidelines, the definitions in Regulation 2 of MARPOL Annex VI, as amended, apply. In addition and for the scope of these guidelines, the following definitions should be considered.

2.1	MARPOL means the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocols of 1978 and 1997 relating thereto, as amended.

2.2	IMO DCS means the data collection system for fuel oil consumption of ships referred to in regulation 22A 27 and related provisions of MARPOL Annex VI.

2.3	For the purpose of these Guidelines, the definitions in Regulation 2 of MARPOL Annex VI, as amended, apply.

2.3	A voyage is [a period of time where the vessel meets the criteria to apply any adjustment]

2.4	A voyage exclusion [adjustment][correction] deducts all main engine, auxiliary engine, boiler and boil-off gas (if reported) related fuel consumption, as well as the associated distance travelled from the calculation of attained CII for a defined period subject to certain threshold conditions being met.

2.5	A correction factor means a factor which [partially] adjusts [parameters in] the calculation of CII.

2.6	A refrigerated container or "reefer container" is an intermodal shipping container that is refrigerated (including chilled and frozen containers) for the transportation of temperature-sensitive cargo.

2.7	Ice edge is defined by paragraph 4.4. of the WMO Sea-Ice Nomenclature, March 2014 as the demarcation at any given time between the open sea and sea ice of any kind, whether fast or drifting.

2.8	A tanker should be considered in Ship-to-Ship (STS) operation when operating in accordance to Regulation 41.2 of MARPOL Annex I and applying the best practices in accordance with the OCIMF Ship to Ship Transfer Guide for Petroleum, Chemical and Liquefied Gases. For the purpose of these guidelines, a tanker is engaged in STS operations if the voyage leg between cargo loading and cargo discharging locations is no longer than 600 nm and the time for each voyage leg is limited to 72 hours.

2.9	A Shuttle Tanker with Dynamic Positioning is a tanker which is equipped with Dynamic Positioning and specialized cargo handling equipment making them capable of loading cargo oil at offshore installations for further crude oil distribution to receiving terminals and refineries. Shuttle tankers are purpose built for such operations and may not be replaced by conventional tankers for this type of trade.

3	Application

3.1	For all ships to which regulation 28 of MARPOL Annex VI applies, the operational carbon intensity formula defined in section 4 should be applied when using voyage adjustments or correction factors.

3.2	Rating of ships according to Resolution MEPC.339(76) 2021 Guidelines on the Operational Carbon Intensity Rating of Ships (CII Rating Guidelines G4) should be carried out using the corrected CII value.

3.3	Corrections factors for electrical related fuel consumption, boiler consumption, boil off gas management and other related fuel consumption should not be used for periods where voyage exclusions adjustments apply.

4	Attained CIIShip Formula for Voyage ExclusionsAdjustments and Correction Factors

Use of voyage exclusionsadjustments and correction factors require changes to be made to the overall attained CIIShip formula as follows:

jCFj⋅FCj-FCvoyage,j+TFj+0.75-0.03yi∙ FCelectrical,j+FCboiler,j+ FCothers,jfi⋅fm⋅fc⋅fiVSE⋅Capacity⋅Dt-Dx⋅AFPT

Where:

- j is the fuel type;

- CFjrepresents the fuel mass to CO2 mass conversion factor for fuel type j, in line with those specified in the 2018 Guidelines on the method of calculation of the attained EEDI for new ships (resolution MEPC.308(73)), as may be further amended;

- FCj is the total mass (in grams) of consumed fuel of type j in the calendar year, as reported under IMO DCS and;

- FCvoyage,j is the total mass (in grams) of fuel of type j, consumed during voyages in periods during the calendar year which may be deducted according to paragraph 4.1 of this Resolution;

- STS means ship to ship transfer operations for tankers as defined in Regulation 41.2 of MARPOL Annex I;

- TFj = 1-AFTanker⋅FCS,j  represents the quantity of fuel j removed for STS or shuttle tanker operation, where FCS,j= FCj for shuttle tankers and FCS,j is the total quantity of fuel j used on STS voyages for STS vessels. If TFj>0 then FCelectrical,j=FCboiler,j= FCothers,j=0;

- AFTanker represents the correction factor to be applied to Shuttle tankers or STS voyages according to paragraph 4.2 of this Resolution;

- yi is a consecutive numbering system starting at y2023=0, y2024=1, y2025=2, etc;

- FCelectrical,j is the total mass (in grams) of fuel type j, consumed for production of electrical power which is allowed to be deducted according to paragraph 4.2 of this Resolution;

- FCboiler,j is the total mass (in grams) of fuel type j, consumed by the boiler which may be deducted according to paragraph 4.3 of this Resolution;

- FCBOG,j is the total mass (in grams) of fuel type j, consumed for the management of boil off gas which may be deducted according to paragraph 4.4 of this Resolution;

- FCothers,j is the total mass (in grams) of fuel type j, consumed by other related fuel consumption devices according to paragraph 4.5 of this Resolution ;

- fi is the capacity correction factor for ice-classed ships as specified in the 2018 Guidelines on the method of calculation of the attained EEDI for new ships Resolution MEPC.308(73);

- fm is the factor for ice-classed ships having 1A Super and 1A as specified in the 2018 Guidelines on the method of calculation of the attained EEDI for new ships Resolution MEPC.308(73) as amended at MEPC 74;

- fc represents the cubic capacity correction factors for chemical tankers as specified in paragraph 2.2.12 of the 2018 Guidelines on the method of calculation of the attained EEDI for new ships Resolution MEPC.308(73);

- fi,VSE represents the correction factor for ship specific voluntary structural enhancement as specified in paragraph 2.2.11.2 of the 2018 Guidelines on the method of calculation of the attained EEDI for new ships Resolution MEPC.308(73), to be applied to self unloading bulk carriers;

- Capacity is deadweight or gross tonnes as defined for each specific ship type in Resolution MEPC.337(76) 2021 Guidelines on the Reference Lines for Use with Operational Carbon Intensity Indicators (CII Reference Lines Guidelines, G2) as amended;

- Dt represents the total distance travelled (in nautical miles), as reported under IMO DCS; and

- Dx represents distance travelled (in nautical miles) for voyage periods which may be deducted from CII calculation according to Paragraph 4.5 of this Resolution

- AFPT is the Port Time Correction factor for cruise passenger ships only

In case the above voyage exclusion or correction factors are applied, the ship should still report total fuel oil consumption (t) of each type of fuel, total hours underway (h) and total distance travelled (nm) to the Administration pursuant to regulation 27 of MARPOL Annex VI.

All relevant data should be recorded in the ship’s logbook. Each parameter, if used, should also be reported to the Administration.

4.1	FCvoyage,j for Voyage Adjustment Exclusion

The parameter FCvoyage,j is the total mass (in grams) of fuel of type j, consumed during voyages in periods during the calendar year which may be deducted from the calculation of the attained CII in case the ship encounters one of the following situations:

.1	scenarios specified in regulation 3.1 of MARPOL Annex VI, which may endanger safe navigation of a ship;

.2 	sailing in ice conditions, which means sailing of an ice-classed ship in a sea area within the ice edge

[.3	sailing in sea states at or greater than a mean significant wave height of 4.0m and a mean wind speed of 28 knots for a time period greater than 8 hoursor more severe than Bf[7];]

[.4	For ships which remain stationary for a period exceeding [a cumulative number] [30 days] such as, but not limited to warm lay-up, drydock, operation as fuel storage unit, stop for technical repairs, the total fuel consumed during the period ]

[.5	For ships where waiting time at port or anchorage exceeds [24] [48] [72] hours, the total fuel consumption for the period in excess of [24] [48] [72] hours;]

[Or combining 4 and 5, For ships which remain stationary for a period exceeding [24] [48] [72] hours, the total fuel consumption for the period in excess of [24] [48] [72] hours]

[.6	For [cruise passenger] ships which spend greater than [20%] of a calendar year in port, at anchor or drifting, the total fuel consumption when in port, at anchor or drifting.]

[.7	Fuel consumed during short voyages of less than 72 hours and associated emissions at the arrival port for tankers, bulk carriers and containerships with deadweight up to 125,000 tonnes.]

[.8	fuel consumption during ship-to-ship transfer (STS) operations, including dynamic positioning operations of shuttle tankers for cargo loading at offshore locations]

In cases where FCvoyage,j is used:

- any associated distance travelled must also be deducted using Dx otherwise ships will benefit from distance travelled without any associated CO2 emission.

- The ship should report data for the excluded voyages to the Administration in accordance with Appendix [2] of these guidelines.

4.2	AFTanker for Corrections to Shuttle Tankers or STS voyages on tankers

Tankers engaged in STS operations as defined above in paragraph 2.9 may apply the correction factor AFTanker,STS to all fuel consumption relating to STS operations, including cargo transfer at offshore location, voyage, cargo discharge and waiting periods at anchor or drifting during which the ship reports being part of an STS operation and voyage related.

The correction factor may only be applied if the voyage leg between cargo loading and cargo discharging locations does not exceed 600 nautical miles and the time for each voyage leg (which does not include port or discharge time) is limited to 72 hours.

The correction is calculated as:

AFTanker,STS=6.1742×DWT-0.246

Where AFTanker,STS is applied, FCelectrical, FCboiler and FCothers should not be used.

Shuttle tankers equipped with Dynamic Positioning may apply the correction factor AFTanker,Shuttle to total fuel consumption:

The correction factor is calculated as:

AFTanker,Shuttle=5.6805 x Capacity-0.208

Where AFTanker,Shuttle is applied, FCelectrical, FCboiler, FCothers and AFTanker,STS should not be used.

4.3	FCelectrical,j for Corrections relating to electrical power

The parameter FCelectrical,j is the total mass (in grams) of fuel of type j, consumed for production of electrical power during the calendar year which may be deducted from the calculation of the attained CII for the following purposes:

.1	Electrical consumption of refrigerated containers (reefers on all ships where they are carried) using the calculation methodology specified in part A of appendix 1.

.2	Electrical consumption of cargo cooling systems on gas carriers maintenance and cargo handling systems, such as, but not limited to cargo heating/cooling systems, nitrogen/inert gas generators, tank washing system, reliquefaction plants, cranes, excavators, side loaders, self unloading systems, using the calculation methodology specified in part B of appendix 1 to this document.

.3	Electrical consumption of electric discharge pumps on tankers

[.3	fuel consumption during dynamic positioning operations of shuttle tankers for cargo loading at offshore locations and ship-to-ship transfer (STS) operations;]

4.4	FCBoiler,j for Corrections relating to Boiler Fuel Consumption

The parameter FCBoiler,j is the total mass (in grams) of fuel of type j, consumed by the oil fired boiler during the calendar year which may be deducted from the calculation of the attained CII,  for the purposes of cargo heating and cargo discharge on tankers.   such as, but not limited to cargo heating, heating of tank washing water, cargo handling and production of inert gas. The calculation methodology for FCBoiler,j is specified in Part B of Appendix 1.

4.4	FCBOG,j for Corrections relating to Boil-off Gas

The parameter FCBOG,j is the total mass (in grams) of fuel of type j, consumed to manage boil-off gases (including cooling of cargo or bunker tanks) during the calendar year which may be deducted from the calculation of the attained CII including:

[.1	Steam dumping]

[.2	Gas combustion unit including boilers used for this purpose]

Boil off gas management via operation of the reliquefaction plant and cooling of cargo tanks is covered by FCElectrical,j.

Note that boil-off gas used for propulsion and electrical load is excluded from FCBOG,j.

For LNG carriers and LNG fuelled ships which need to handle boil-off gas, the calculation methodology is specified in Part B of Appendix 1.

4.5	FCothers,j for Corrections relating to other related fuel consumption devices

The parameter FCothers,j is the total mass (in grams) of fuel of type j, consumed by standalone cargo pump generators during discharge operations on tankers other  fuel consumption devices which may be deducted from the calculation of the attained CII. for the purposes of cargo maintenance and cargo handling systems, such as, but not limited to standalone nitrogen/inert gas generator, standalone diesel engine cargo pump.

4.6	DX Distance Adjustment

Dx represents distance travelled (in nautical miles) for voyages which may be deducted from CII calculation for the scenarios included in paragraph 4.1.

4.7	EEDI Correction factors

The following correction factors as defined in the 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships Resolution MEPC.308(73) and as amended by resolution MEPC.322(74) may be applied:

.1	capacity correction factor for ice-classed ships (), as given in paragraph 2.2.11.1 in the 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships (see resolution MEPC.308(73)); and

.2	correction factor (), for ice-classed ships having IA Super and IA or equivalent, as given in paragraph 2.2.19 of the 2018 Guidelines on the method of calculation of the attained Energy Efficiency Design Index (EEDI) for new ships as amended by resolution MEPC.322(74).

.3	Cubic capacity correction factor (fc) for chemical tankers as per paragraph 2.2.12.1 of Resolution MEPC.308(73)

.4	Correction factor (fi VSE) for ship specific voluntary structural enhancement as per paragraph 2.2.11.2 of Resolution MEPC.308(73)

4.8	Port Time correction factor for Cruise Passenger Ships

The Port Time Correction Factor for cruise passenger ships only is calculated as:

AFPT=(1.8-Hours Under WayHours In Operation)

Where:

- Hours Under Way is as reported to the IMO data collection system according to Appendix IX of MARPOL Annex VI

- Hours In Operation is the number of hours that the ship is in service. This is normally the number of hours in a year (8760), however for ships delivered part way through a year, hours in operation should be calculated from the date of delivery.

For cruise passenger ships where  Hours Under WayHours In Operation≥0.8, CFPT=1.

Coordinator’s note:  The formula has been adjusted to account for situations where newbuild ships are delivered part way through the year and would have less than 8760 hours in operation.

Appendix 1

Correction factors for use in CII calculation

Part A. FCElectrical for Corrections relating to electrical power

1	Reefer Containers

For ships carrying refrigerated containers (reefers), the correction factor FCElectrical may be applied as follows:

.1	For ships that have the ability to monitor reefer electrical consumption, the ship may calculate reefer container kWh consumption as follows:

FCelectrical_reefer,j=Reefer kWh × SFOC

where:

- FCelectrical_reefer,j (Reefer fuel oil consumption) represents the estimated fuel consumption attributed to in-use refrigerated containers carried.

- Reefer kWh is measured on the vessel by the kWh meter counter on the vessel

- SFOC represents the specific fuel oil consumption in g/kWh associated with the relevant source of electrical power [as per EEXI technical file].

Coordinator’s note: Note that the EEXI capacity threshold differs from the CII threshold of 5,000 GT therefore there will be some ships without aux engine SFOC information. Note also that SFOC data from NOx testing is from the parent engine in ideal conditions, and actual SFOC is likely to be worse, so this constitutes an additional unknown margin over the 75%-3%.

If an EEDI or EEXI technical file is available, then the weighted average SFOC of the generators should be used as per the EEDI calculation guidelines. If the ship does not have an EEDI or EEXI technical file, then a default value of [200] g/kWh should be used. The default value should not be set too high otherwise there is a perverse incentive to use the default value.

In the event that shaft generators are used a SFOC of xxx g/kWh should be assumed.

In the event that a waste heat recovery system is used a SFOC of xxx should be assumed to ensure fair treatment.

In order to minimize the reporting burden, the SFOC used should be from the main source of electricity (genset, shaft generator or waste heat recovery system) for each voyage leg reported for verification.

Note that ship reefer kWh consumption should not include consumption during voyage adjustment periods.

.2	For ships that do not have the ability to monitor reefer electrical consumption, the ship may calculate reefer kWh consumption as follows:

FCelectrical_ reefer, j=Cx⋅24⋅SFOCsea⋅Reefer_dayssea+SFOCport⋅Reefer_days port

where:

- Cx represents the in-use average reefer consumption of 2.75 kW.

- Reefer_days sea represents the number of in-use reefer-days over the declared period and may be derived using the number of reefer containers as recorded in the Baplie file multiplied by the number of days at sea.

- SFOCsea represents the specific fuel oil consumption in g/kWh associated with the relevant source of electrical power at sea

- SFOCport represents the specific fuel oil consumption in g/kWh associated with the relevant source of electrical power in port

In ports where shore-power is not used, the number of in-use reefers at port should be calculated as:

Reefer_days port = Noc Arrival+ Noc Departure2×Daysport

where:

- Daysport represents number of days in port

- Reefer_days port represents the number of in-use reefer days while at port

- Noc Arrival represents number of reefer containers on arrival

- Noc Departure represents number of reefer containers at departure.

In all cases, the actual number of in-use reefers carried is documented in the Baplie file.

Note that ship reefer kWh consumption should not include consumption during voyage adjustment periods.

2	Cargo Cooling Systems on Gas Carriers

For gas carriers with electrical cargo cooling systems, the correction factor FCelectrical may be applied as follows:

.1	Gas Carriers may calculate cargo cooling kWh consumption as follows:

FCelectrical_cooling,j=Cooling kWh × SFOC

where:

- FCelectrical_cooling,j (cargo cooling fuel oil consumption) represents the estimated fuel consumption attributed to cooling of gas cargoes.

- Cooling kWh is measured on the vessel by the kWh meter counter on the vessel

- SFOC represents the specific fuel oil consumption in g/kWh associated with the relevant source of electrical power [as per EEXI technical file].

Note that cargo cooling kWh consumption should not include consumption during voyage adjustment periods.

3	Electric Cargo discharge pumps on tankers

For tankers with electrically powered discharge pumps, the correction factor FCelectrical may be applied as follows:

.1	Tankers may calculate cargo discharge kWh consumption as follows:

FCelectrical_discharge,j=discharge kWh × SFOC

where:

- FCelectrical_discharge,j (cargo discharge fuel oil consumption) represents the estimated fuel consumption attributed to use of cargo discharge pumps.

- Discharge kWh is measured on the vessel by the kWh meter counter on the vessel

- SFOC represents the specific fuel oil consumption in g/kWh associated with the relevant source of electrical power [as per EEXI technical file].

Part B. FCBoiler and FCOthers for Corrections relating to cargo heating and discharge on tankers

1	FCBoiler for cargo heating and discharge pumps on tankers

For tankers with fuel fired boilers used for cargo heating or steam driven cargo pumps, the following correction factor may be applied for the period that the cargo heating or discharge pumps are in operation:

.1	In the case of boilers used for cargo heating, the amount of fuel used by the boiler (FCBoiler) should be measured by accepted means, e.g. tank soundings, flow meters

.2	For tankers which use steam driven cargo pumps, the amount of fuel used by the boiler (FCBoiler) should be measured by accepted means, e.g. tank soundings, flow meters

2	FCOthers for discharge pumps on tankers

For tankers with discharge pumps powered by their own generator, the amount of fuel used for the period that the discharge pumps are in operation (FCOthers) should be measured by accepted means, e.g. tank soundings, flow meters.

Appendix 2

Guidance on reporting of fuel oil consumption and distance travelled for voyages for ice-classed ships when sailing in ice conditions

In this appendix guidance is given for reporting and verification of fuel oil consumption and distance travelled concerning voyage adjustments exclusions when sailing in ice conditions.

Reporting of fuel oil consumption for periods voyages when sailing in ice conditions

1	Fuel oil consumption for periods voyages when sailing in ice conditions should include all the fuel oil consumed on board including but not limited to the fuel oil consumed by the main engines, auxiliary engines, gas turbines, boilers and inert gas generator, for each type of fuel oil consumed, regardless of whether a ship is underway or not. Methods for collecting data on fuel oil consumption in metric tonnes when sailing in ice conditions include the method using flow meters or method using bunker fuel oil tank monitoring on board as described in paragraphs [7.1.2] and [7.1.3] of the 2016 2022 Guidelines for the development of a Ship Energy Efficiency Management Plan (SEEMP Guidelines) correspondingly.

.1 	Before the ship commences its voyage on its planned route, it is mandatory for the ship to prepare the voyage plan. In preparation of the ship's voyage plan, the master should consider ambient ice conditions and ice conditions along the ship's entire planned route using official ice charts. At the end of the voyage, if the ship has encountered ice conditions during its voyage, when the ship was underway sailing between the ice edges or between the ice edge and the port, the fuel oil consumed measured in accordance with 7.1.2 or 7.1.3 of the SEEMP Guidelines for the portion of the period in ice voyage when sailing in ice conditions should not be included in the calculations for the annual average attained CII index value. If the portion of the ship's voyage is excluded from calculations of the attained CII index value due to sailing in ice conditions, this should be clearly marked in the SEEMP monitoring plan, the ship's logbook should include data entries for the portion of the voyage with date and time when the ship encountered ice conditions and data added to the data reporting format.

.2	The summary of monitoring data containing records of measured fuel oil consumption when sailing in ice conditions should be available on board.

Reporting of distance travelled for periods voyages when sailing in ice conditions

2	The distance travelled over ground in nautical miles for periods voyages when sailing in ice conditions should be recorded in the log-book in accordance with SOLAS regulation V/28.1 and submitted to the Administration.

.1 	Before the ship commences its voyage on its planned route, it is mandatory for the ship to prepare its voyage plan. In preparation of the ship's voyage plan, the master should consider ambient ice conditions and ice conditions along the ship's entire planned route using official ice charts. At the end of the voyage, if the ship has encountered ice conditions at any part of a single voyage, when the ship was underway sailing between the ice edges or between the ice edge and the port, the distance travelled when sailing in ice conditions measured for the affected portion of the voyage should not be included in the calculations for the annual average attained CII index value. If the portion of the ship's voyage is excluded from calculations of attained CII index value due to sailing in ice conditions, this should be clearly marked in the SEEMP monitoring plan, ships logbook should include data entries for the portion of the voyage with date and time when the ship encountered ice conditions and data added to the data reporting format.

***

An example of an ice chart of the Baltic Sea area

GT should be used for Ro-Ro Cargo ships when this change is made to Resolution MEPC.337(76)

For further information on approximate correspondence between ice classes, see HELCOM Recommendation 25/7, which can be found at http://www.helcom.fi

The number of reefers onboard while in port, should be calculated to equal the number of reefers at arrival and at departure as calculated above. Same calculation applies for Reefer_dayssea in port.