TOR 5: Incorporate the in-service measurement method to determine a reference speed (Vref) into the EEXI calculation guidelines and the EEXI survey and certification guidelines

Comment sheet for Round 5

Country / Organization:_________________________________

Base documents: Annex 1, 2 and 3 of MEPC 77/7/2, as well as the 2021 Guidelines on the method of calculation of the attained energy efficiency existing ship index (EEXI) (resolution MEPC.333(76)) and the 2021 Guidelines on survey and certification of the attained energy efficiency existing ship index (EEXI) (resolution MEPC.334(76))

# Introduction

The CG has been instructed to:

Consider proposed amendments to the EEXI calculation guidelines and the EEXI survey and certification guidelines to incorporate the in-service measurement method to determine a reference speed taking into account documents MEPC 77/7/2 (Japan et al.), MEPC 77/7/24 (India), MEPC 77/7/25 (Republic of Korea) and MEPC 77/7/26 (IACS), and advise the Committee accordingly.

The CG members are invited to consider the following issues concerning:

.1	the in-service measurement method to determine a reference speed (Vref), the track changes in the base documents as outlined in the cohesive Q1-1, Q1-2 and Q1-3, in conjunction with the editorials outlined in paragraph 4.1 and 4.2 of document MEPC 77/7/24, as well as the proposal contained in documents MEPC 77/7/25 and MEPC 77/7/26; and

.2	the maximum allowable deduction for the calculation of Σ𝑃𝑀𝐸(𝑖) for ships fitted with shaft generators and EPL, the track changes as outlined in Q2 based on the proposal in paragraph 11 of document MEPC 77/7/24.

Q1-1.	Amendments to the 2021 Guidelines on the method of calculation of the attained energy efficiency existing ship index (EEXI) (resolution MEPC.333(76)), concerning the in-service measurement method to determine a reference speed (Vref)

The base document contains track-changes from annex 1 of document MEPC 77/7/2.

The CG members are invited to indicate their support to the suggested changes, or propose alternative texts otherwise together with a rationale and a clear reference to the paragraph concerned.

| Document part | Comment |
| --- | --- |
| 1. Complete base document based on annex 1 of document MEPC 77/7/2 | Please indicate your support or provide alternative texts with a rationale and a clear reference to the underlined text in the base document. 2.2.3.4bis: 2.2.3.6.1bis: 2.2.3 Vref ; Ship speed 2.2.3.4 For containerships, bulk carriers or tankers not falling into the scope of the EEDI requirement but whose sea trial results, which may have been calibrated by the tank test, under the design load draught and sea condition as specified in paragraph 2.2.2 of the EEDI Calculation Guideline are included in the sea trial report, the ship speed Vref may be obtained from the sea trial report: … 2.2.3.4bis In cases where the speed-power curve is not available or the sea trial report does not contain the EEDI or design load draught condition, the ship speed Vref can be obtained from the in-service performance measurement method conducted and verified in accordance with the methods and procedures as specified in MEPC.1/Circ.[xxx] (as set out in annex 3 to MEPC 77/7/2). 2.2.3.5 In cases where the speed-power curve is not available or the sea trial report does not contain the EEDI or design load draught condition, the ship speed Vref can be approximated by Vref to be obtained from statistical mean of distribution of ship speed and engine power, as defined below: … 2.2.3.6 Notwithstanding the above, in cases where the energy saving device* is installed, the effect of the device may be reflected in the ship speed Vref with the approval of the verifier, based on the following methods in accordance with defined quality and technical standards: 2.2.3.6.1 sea trials after installation of the device; and/or 2.2.3.6.1bis in-service performance measurement method; and/or 2.2.3.6.2 dedicated model tests; and/or 2.2.3.6.3 numerical calculations. |
| 2. Any other comments |  |

Q1-2.	Amendments to the 2021 Guidelines on survey and certification of the attained energy efficiency existing ship index (EEXI) (resolution MEPC.334(76)), concerning the in-service measurement method to determine a reference speed (Vref)

The base document contains track-changes from annex 2 of document MEPC 77/7/2.

The CG members are invited to indicate their support to the suggested changes, or propose alternative texts otherwise together with a rationale and a clear reference to the paragraph concerned.

| Document part | Comment |
| --- | --- |
| 1. Complete base document based on annex 2 of document MEPC 77/7/2 | Please indicate your support or provide alternative texts with a rationale and a clear reference to the underlined text in the base document. 4.2.2.9bis: 4.2.7bis: 4.2 Verification of the attained EEXI 4.2.1 For verification of the attained EEXI, an application for a survey and an EEXI Technical File containing the necessary information for the verification and other relevant background documents should be submitted to a verifier, unless the attained EEDI of the ship satisfies the required EEXI. 4.2.2 The EEXI Technical File should be written at least in English. The EEXI Technical File should include, but not be limited to: 4.2.2.9 a sea trial report including sea trial results, which may have been calibrated by the tank test, under the sea condition as specified in paragraph 2.2.2 of the EEDI Calculation Guidelines, if available; 4.2.2.9bis an in-service performance measurement report, where applicable, as specified in paragraph 2.2.3.4bis and 2.2.3.6.1bis of the EEXI Calculation Guidelines; 4.2.2.10 calculation process of Vref,app for pre-EEDI ships in cases where the speed-power curve is not available, as specified in paragraph 2.2.3.56 of the EEXI Calculation Guidelines; 4.2.2.11 type of fuel; … … 4.2.7 In cases where the sea trial report as specified in paragraph 4.2.2.9 is submitted, the verifier should request further information from the submitter to confirm that: … 4.2.7bis In cases where the in-service performance measurement report as specified in paragraph 4.2.2.9bis is submitted, the verifier should confirm that the in-service performance measurement was conducted and verified in accordance with the methods and procedures as specified in MEPC.1/Circ.[xxx] (as set out in annex 3 to MEPC 77/7/2). 4.2.8 The estimated speed-power curve obtained from the tank test and/or numerical calculations and/or the sea trail results calibrated by the tank test should be reviewed on the basis of the relevant documents in accordance with the EEDI Survey and Certification Guidelines, the defined quality standards (e.g. ITTC 7.5-03-01-02 and ITC 7.5-03-01-04 in their latest revisions) and the verification of the numerical set-up with parent hull or the reference set of comparable ships. |
| 2. Any other comments |  |

Q1-3.	Proposed new MEPC circular on Guidance on methods, procedures, and verification of the in-service performance measurements, concerning the in-service measurement method to determine a reference speed (Vref)

The base document contains track-changes from annex 3 of document MEPC 77/7/2, as well as paragraph 4.1 and 4.2 of document MEPC 77/7/24, along with paragraph 9 of document MEPC 77/7/26.

The CG members are invited to indicate their support to the suggested changes, or propose alternative texts otherwise together with a rationale and a clear reference to the paragraph concerned.

| Document part | Comment |
| --- | --- |
| 1. Complete base document based on annex 3 of document MEPC 77/7/2, as well as paragraph 4.1 and 4.2 of document MEPC 77/7/24 | Please indicate your support or provide alternative texts with a rationale and a clear reference to the strike through and underlined text in the base document. 4.7: 4.9: 4 During the in-service performance measurements … 4.6 During the in-service performance measurements, accurate recordings of the required parameters are of great importance. Recording of parameters for each run should start when steady state ship conditions are met. 4.7 The following data should be collected at the beginning and end of each performance measurement run: Main engine supply flowmeter reading; [deg] [ltr/h] or [kg/h]. Main engine supply flowmeter temperature; [deg]. Main engine return line flowmeter reading*; [deg] [ltr/h] or [kg/h]. Main engine return line flowmeter temperature*; [deg]. (*For ships fitted with flowmeter on return line) 4.8 The following data should be collected with a sampling rate of at least 1 Hz during the in-service performance measurement: … 4.9 Apart from power, rpm and consumption, average prevailing values for the following main engine parameters should be provided for each run for the following: Scavenge air temperature [deg] Scavenge air pressure [deg] [kg/cm2] Blower air inlet temperature [deg] 4.10 These, as well as any other main engine data should be collected at local sensors' display and not their repeaters inside the Engine Control Room (ECR). … |
| 2. Complete base document based on annex 3 of document MEPC 77/7/2, as well as document MEPC 77/7/26 | Please indicate your support or provide alternative texts with a rationale and a clear reference to the underlined text in the base document. 4.11: … 4.10 These, as well as any other main engine data should be collected at local sensors' display and not their repeaters inside the Engine Control Room (ECR). 4.11 As far as practicable, the in-service performance measurement should be witnessed by the verifier. The verifier should be able to confirm that the in-service performance measurement was conducted in accordance with the agreed procedures. 5 After the in-service performance measurements … |
| 3. Any other comments |  |

Q2.	Amendments to the 2021 Guidelines on the method of calculation of the attained energy efficiency existing ship index (EEXI) (resolution MEPC.333(76)), concerning the maximum allowable deduction for the calculation of Σ𝑃𝑀𝐸(𝑖) for ships fitted with shaft generators and EPL

The base document contains track-changes from paragraph 11 of document MEPC 77/7/24.

The CG members are invited to indicate their support to the suggested changes, or propose alternative texts otherwise together with a rationale and a clear reference to the paragraph concerned.

| Document part | Comment |
| --- | --- |
| 1. Complete base document based on paragraph 11 of document MEPC 77/7/24 | Please indicate your support or provide alternative texts with a rationale and a clear reference to the underlined text in the proposal. 2.2.1: 2.2.1 PME(i) ; Power of maine engines In cases where overridable Shaft/Engine Power Limitation is installed in accordance with the 2021 Guidelines on the shaft/engine power limit to comply with the EEXI requirements and use of a power reserve (resolution MEPC.335(76)), PME(i) is 83% of the limited installed power (MCRlim) or 75% of the original installed power (MCR), whichever is lower, for each main engine (i). In cases where the overridable Shaft/Engine Power Limitation and shaft generator(s) are installed, in referring to paragraph 2.2.5.2 (option 1) of the EEDI Calculation Guidelines, "MCRME" should be read as "MCRlim”. The maximum allowable deduction for the calculation of Σ𝑃𝑀𝐸(𝑖) is to be no more than 𝑃𝐴𝐸 as defined in paragraph 2.2.5.6. For this case, Σ𝑃𝑀𝐸(𝑖) is calculated as the lower of: i=1nMEPME(i)=k × MCRME(i) -0.75 × PPTO(i) with 0.75 × PPTO(i) ≤ PAE Where 𝑘 depends on whether the MCRME(i) is the original installed power (MCRME) or limited installed power (MCRlim): 𝑘 = 0.75 in case of original installed power (MCRME). 𝑘 = 0.83 in case of limited installed power (MCRlim). For LNG carriers having steam turbine or diesel electric propulsion, PME(i) is 83% of the limited installed power (MCRlim, MPPlim), divided by the electrical efficiency in case of diesel electric propulsion system, for each main engine (i). For LNG carriers, the power from combustion of the excessive natural boil-off gas in the engines or boilers to avoid releasing to the atmosphere or unnecessary thermal oxidation should be deducted from PME(i) with the approval of the verifier. |
| 2. Any other comments |  |